3GPP TSG-RAN WG2 #127					      		 R2-2407205

Maastricht, Netherlands – August 19 - 23, 2024

Agenda item:	8.13.2

Source: 	Kyocera

Title: 	Discovery and (re)selection under multihop relay

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues to address for relay discovery and (re)selection under U2N multihop relay.

# Discussion

Based on the work item [1], one of the issues to be addressed is highlighted below:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

This contribution discusses some of the issues that should be addressed under the multihop relay condition for both relay discovery and (re)selection.

## 2.1.	Discovery

For multihop relay discovery, SA2 has already provided conclusion of their study in [1] and for Model A discovery it was described in the Figure 1 below how the upper layer signaling for discovery is intended to work.

Figure 1: Example Model A Discovery operation supporting multi-hop UE-to-Network Relay (from Figure 6.1.2.1.1-1 of [2])

For Model A discovery, the 5G ProSe UE-to-Network Relay reuses the 5G ProSe UE-to-Network Relay Discovery Announcement message as defined in clause 5.8.3 of TS 23.304 [4], with the following additional IE:

-	Hop-Count: This value should be set to 1. It serves as an indication that the 5G ProSe UE-to-Network Relay supports multi-hop operation. In order to be legacy compliant, i.e. serving prior release 5G ProSe Remote Ues, the encoding of this Hop-Count should allow the 5G ProSe Remote UE not support multi-hop operation to ignore it and follow the existing procedures defined in TS 23.304 [4].

For Model A discovery, the Intermediate UE-to-Network Relay(s) forwards the Announcement message with the following modifications:

-	Source Layer-2 ID: this is set to the source Layer-2 ID of the Intermediate UE-to-Network Relay.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration. It should be the same Destination Layer-2 ID of the Announcement message received by the Intermediate UE-to-Network Relay.

-	Announcer Info: identify information (i.e. User Info ID) of the announcing Intermediate UE-to-Network Relay.

-	Relay Service Code: no modification to the RSC of the received Announcement message.

-  Hop-Count: The Intermediate UE-to-Network Relay increment the value of the received Announcement message by 1 before forwarding it. If this value is equal or bigger than the Hop-Limit value of the associated RSC, the message is not forwarded.

Based on the TR [2], for Model A discovery the U2N relay UE transmits a discovery message containing the User Info ID of itself, maximum number of hops and the associated RSC. Subsequently, the intermediate relay UEs are expected to propagate the discovery message from the U2N relay UE while updating the multi-hop counter until the maximum number of hops is reached. It is also expected that the remote UE, when receiving multiple discovery messages from the intermediate relay UEs will choose the intermediate relay UE with the smallest multihop counter.

As in the case for the existing U2N relay UEs, the remote UE or relay UEs (U2N relay UEs or intermediate relay UEs) are allowed to perform discovery transmissions or DCR message with integrated discovery transmissions while in coverage or out-of-coverage regardless of the RRC state.

- Proposal 1	Remote UEs, U2N/intermediate relay UEs are allowed to perform discovery transmissions while in coverage or out-of-coverage regardless of the RRC state.

For the legacy U2N relay case, the relay UE is allowed to transmit discovery messages when both a maximum Uu RSRP threshold and/or a minimum Uu RSRP threshold are met, if configured; however, it still needs to be decided whether any restriction(s) are needed for the intermediate relay UEs. One possibility is to consider that relay UEs that falls below the minimum Uu RSRP threshold are allowed to transmit discovery as an intermediate relay UE, similar to the restriction that was applied to the legacy remote UE.

- Proposal 2	RAN2 should discuss if Uu RSRP threshold(s) should apply to intermediate relay UE’s discovery transmissions.

Another aspect for discovery transmission that should be considered is whether the intermediate relay UE needs to be PC5-RRC connected to the U2N relay UE before it transmits discovery message.  In case the remote UE selects the intermediate relay UE, it should be assumed the PC5 connection between the two relay UEs are already completed. If the remote UE selects the intermediate relay UE and the intermediate UE fails to connect to the U2N relay UE, it would result in failure in the remote UE’s relay (re)selection which should be prevented.

- Proposal 3	RAN2 should consider whether the intermediate relay UE needs to be PC5-RRC connected prior to discovery transmissions.

With regards to the resource pool(s) used for relay discovery, previously for the single-hop relay case, both resource pool(s) dedicated for relay discovery and resource pool(s) for sidelink communication can be used, which can be configured via SIB, dedicated signaling or pre-configuration.  It was also agreed previously that if resource pool(s) for dedicated relay discovery are configured, only those resource pool(s) shall be used for relay discovery.  Since there are multiple relay UEs involved in the transmission of discovery messages, RAN2 should further discuss if the same rule would apply to all relay UEs, i.e., if all relay UEs must use the same resource pool type for discovery transmission and if so, how the relay UEs should be coordinated to use the same resource pool type. Also, depending on the AS layer restriction placed on the intermediate relay UEs for discovery transmission in Proposal 3, it’s possible that the intermediate relay UEs is in-coverage while it is PC5 Connected to the U2N relay UE.  Below are a few potential options:

- Follow the existing rule that if dedicated resource pool for relay discovery is (pre)configured, the relay UE will always use the dedicated resource pool, regardless if the resource pool for sidelink communication is also (pre)configured.

- It is up to relay UE implementation for which resource pool to use for relay discovery.

- U2N relay UE should inform the intermediate relay UE of the chosen resource pool.

With the first option, the rule for resource pool usage would be aligned with the previous release.  However, it is possible that the U2N relay UE would be configured with dedicated signaling with the resource pool to use, while the intermediate relay UE would use the resource pool based on pre-configuration, if it is out-of-coverage, which may result in different resource pool types being used by the two relay UEs.  With the second option, intermediate relay UE and remote UEs would need to monitor both types of resource pools.  With the third option, the U2N relay UE, which may be provided with the resource pool via dedicated signaling may inform the intermediate relay UE of the chosen resource pool, during the initial discovery procedure, at least for Model A discovery.

- Proposal 4	RAN2 should consider the type of resource pools for use by the relay UEs for discovery transmissions.

## 2.2	Relay (re)selection

For relay selection and reselection, the remote UE should use SD-RSRP or SL-RSRP as a measure of PC5 link quality as in legacy. Additionally, we can adopt all the legacy rules for the remote UE to trigger selection and reselection as below [3]:

The U2N Remote UE triggers U2N Relay selection in following cases:

-	Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold;

-	Indicated by upper layer of the U2N Remote UE.

The U2N Remote UE may trigger U2N Relay reselection in following cases:

-	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

-	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

-	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF;

-	Indicated by upper layer.

- Proposal 5	For triggering remote UE to perform relay selection and relay reselection, the legacy conditions should be adopted as baseline.

In addition to the baseline assumptions for triggering relay (re)selection, we should also consider whether the new information provided in the discovery message should impact relay (re)selection.  In particular, we understand SA2 has included in the discovery message both hop count and QoS information which is either Per-hop QoS information or cumulative QoS information (e.g. cumulative delay between Remote UE up to the Intermediate Relay).  It isn’t clear how such QoS information is provided to the relay UEs and how remote UEs, esp. remote UEs attempting initial access would use such QoS information.  Therefore, for Model A discovery, we assume IDLE remote UEs should select relay UE with the least number of hops, as long as the PC5 link quality is above the (pre)configured threshold. For example, we assume the remote UE will select a U2N relay UE over any intermediate relay UE and that discovery message contents will be used to distinguish between the two types of relay UE.  However, it should be further discussed whether this should be left for UE implementation.

- Proposal 6	For Model A discovery under initial connection access, the IDLE remote UEs should select the path with the least number of hops.

- Another scenario for relay reselection occurs when the intermediate relay UE informs the remote UE that its PC5 connection to the U2N relay UE has failed (refer to Figure 2).  According to the legacy procedure, this should trigger the remote UE to perform relay reselection.  As part of the relay reselection procedure, the remote UE may choose another relay UE or keep the PC5 connection to the existing intermediate relay UE.  In case the remote UE cannot detect any available relay UE, it should be allowed to request that the intermediate relay UE should perform relay reselection on behalf of the remote UE.

- Figure 2: Relay reselection upon PC5 connection failure

- Proposal 7	When the remote UE triggers relay reselection after receiving notification from its intermediate relay UE of the PC5 connection failure, the remote UE should be allowed to request the intermediate relay UE to perform relay reselection.

- Besides the condition for the remote UE’s relay reselection, it should also be considered whether the intermediate relay UEs are allowed to perform relay reselection and if so, under what conditions.  Examples of when the intermediate relay UE may trigger relay reselection is when it receives a PC5-S release message from its PC5 connected relay UE or if the PC5 connection (e.g., SL-RSRP level) falls below a configured threshold.  If the intermediate relay UE is allowed to perform relay reselection, it should inform the remote UE the result of the relay reselection.

- Proposal 8	RAN2 should consider whether the intermediate relay UEs are allowed to perform relay reselection and whether the remote UE should be informed the result of the relay reselection.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay discovery and relay (re)selection.  RAN2 is kindly asked to take into account the proposals below:

- Proposal 1	Remote UEs, U2N/intermediate relay UEs are allowed to perform discovery transmissions while in coverage or out-of-coverage regardless of the RRC state.

- Proposal 2	RAN2 should discuss if Uu RSRP threshold(s) should apply to intermediate relay UE’s discovery transmissions.

- Proposal 3	RAN2 should consider whether the intermediate relay UE needs to be PC5-RRC connected prior to discovery transmissions.

- Proposal 4	RAN2 should consider the type of resource pools for use by the relay UEs for discovery transmissions.

- Proposal 5	For triggering remote UE to perform relay selection and relay reselection, the legacy conditions should be adopted as baseline.

- Proposal 6	For Model A discovery under initial connection access, the IDLE remote UEs should select the path with the least number of hops.

- Proposal 7	When the remote UE triggers relay reselection after receiving notification from its intermediate relay UE of the PC5 connection failure, the remote UE should be allowed to request the intermediate relay UE to perform relay reselection.

- Proposal 8	RAN2 should consider whether the intermediate relay UEs are allowed to perform relay reselection and whether the remote UE should be informed the result of the relay reselection.

# References

- RP-241609, “New WID on NR sidelink multi-hop relay”, LG Electronics, InterDigital, FirstNet

- TR 23.700-03, “Study on system enhancement for Proximity based Services”

- TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”