3GPP TSG-RAN WG2 #127					      		 R2-2407206

Maastricht, Netherlands – August 19 - 23, 2024

Agenda item:	8.13.3

Source: 	Kyocera

Title: 	Control Plane under multihop L2 U2N relaying

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key issues for control plane under L2 U2N multihop relaying.

# Discussion

Based on the work item [1], one of the issues to be addressed is highlighted below:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

2.1.	Connection establishment

For the discussion of connection establishment procedure, we could assume the relay discovery and relay selection process has been completed successfully (Steps 1 and 2 in Figure 1).

Figure 1: RRC Setup procedure with multihop relay UEs.

Once the relay UEs are selected for the desired path towards the NW, the PC5 connections for all the sidelink hops must also be established.  Then it is assumed the remote UE will send the SRB0 (RRC Setup Request) via a preconfigured SL-RLC0 via the selected relay UEs towards the serving cell (Step 3).  With regards to Step 4 in Figure 1, we assume the intermediate relay UE will need to forward the remote UE’s RRC Setup Request to the U2N relay UE, but it will also need to send its own RRC setup Request to the cell via the U2N relay UE, if it’s not yet in RRC CONN.  In the legacy U2N case with one U2N relay UE, it is assumed the U2N relay UE that is in IDLE or INACTIVE would need to first establish its own RRC connection before sending the remote UE’s RRC Setup request to its serving cell.  With the addition of the intermediate relay UE, it would be necessary for the intermediate relay UE to send both RRC Setup Requests to the gNB if the intermediate relay UE is also not in RRC CONN.  If we follow the legacy-like procedure, the intermediate relay UE would first send its own RRC Setup Request to the U2N relay UE, and similarly for the U2N relay (as needed if it’s not in RRC CONN). It may also be assumed that the intermediate relay UE will not receive the RRC Setup from the gNB, until the U2N relay UE is RRC CONN.  This would suggest that the intermediate relay UE will only forward the remote UE’s RRC Setup Request to the gNB via the U2N relay UE once it is in RRC CONN.  However, it could also be considered whether such a restriction is really necessary as the intermediate relay UE in IDLE/INACTIVE can still send the remote UE’s RRC Setup Request to the U2N relay UE via a specified SL RLC Channel (similar to the one used by the remote UE) to reduce remote UE’s latency in RRC connection establishment.

- Proposal 1	RAN2 should consider if there are any restrictions for intermediate relay UE to forward the remote UE’s RRC Setup Request in case it is also in RRC IDLE/INACTIVE.

Another issue that should be considered as part of RRC connection establishment procedure, is whether the gNB needs to know which relay UEs are associated with the remote UE’s RRC connection establishment, for the case when all the associated relay UEs are already in RRC CONN.  For example, in Figure 2, Remote UE 1 is already in RRC CONN, and the indirect path consists of two intermediate relay UEs followed by the U2N relay UE. Remote UE 2, currently in RRC IDLE wants to set up RRC connection with the gNB and has selected Intermediate relay UE 1 to connect with the gNB.  The current RRC Setup procedure would allow the U2N relay UE to forward the remote UE’s RRC Setup Request to the gNB; however, the gNB may not know which relay UEs are involved in this remote UE’s RRC Setup Request.

Figure 2: RRC Setup Request while all relay UEs are already in RRC CONNECTED.

- Proposal 2	RAN2 should consider how the gNB should be informed of the relay UEs associated with the RRC Setup Request from the remote UE, if all the relay UEs associated with the indirect path is already in RRC CONN.

2.1.2	Failure condition handling

The failure conditions for multihop relay should follow the same principles and failure reporting as for the legacy U2N failure conditions.  Currently, the NotificationMessageSidelink message already allows for certain indications to be sent from the relay UE to the remote UE.  These include the case for Uu RLF, relay UE HO, relay UE cell reselection, RRC failure, including establishment and resume failures. Also, for U2U relay, it was further added that indication of SL-RLF may also be indicated, which is particularly useful for the multihop relay UE scenario which wasn’t an issue with the legacy U2N relay scenario.  Additionally, the SidelinkUEInformationNR message allows the U2N relay UE to report the SL-RLF to the gNB.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

For the multihop relay scenario, although there’s only one Uu interface, there are multiple PC5 hops, which depends on the number of relay UE in the indirect path.  Therefore, when the U2N relay UE detects SL-RLF or is notified of the SL-RLF from a PC5 connected intermediate relay UE, it should inform the gNB not only of a SL-RLF, but also which relay UE detected the SL-RLF.  This will prevent the gNB from releasing a relay UE that is still has good connection to the gNB and continue to provide relay service to other remote UEs.

- Proposal 3	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

The same condition may also be considered for the indication sent to the remote UE.  In case the NotificationMessageSidelink message is sent to the remote UE, the relay UE should also include in the message the relay UE that was the cause for the indication.

- Proposal 4	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

2.1.2	RRC Connection Re-establishment

For multihop relay scenario, the baseline assumption should be similar to the legacy U2N case, i.e., upon the detection of SL-RLF, the U2N Remote UE may perform relay reselection and perform RRC connection re-establishment. It may be further considered if the intermediate relay UE that is still PC5 connected to the remote UE can perform the re-establishment upon request from the remote UE, since the intermediate relay UE may be closer to the cell and have a better chance of connecting to a U2N relay UE, if not directly to the gNB.

- Proposal 5	RAN2 should consider if the intermediate relay UE that is still PC5 connected to the remote UE can perform RRC Re-establishment upon request from the remote UE.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay on the control plane.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	RAN2 should consider if there are any restrictions for intermediate relay UE to forward the remote UE’s RRC Setup Request in case it is also in RRC IDLE/INACTIVE.

- Proposal 2	RAN2 should consider how the gNB should be informed of the relay UEs associated with the RRC Setup Request from the remote UE, if all the relay UEs associated with the indirect path is already in RRC CONN.

- Observation 	The existing indications in the failure NotificationMessageSidelink message and SidelinkUEInformationNR message can be largely reused for the multihop relay scenario.

- Proposal 3	U2N relay UE should inform the gNB of the intermediate relay UE that detected the SL-RLF.

- Proposal 4	The remote UE should be informed via the NotificationMessageSidelink message which relay UE is the cause for the indication.

- Proposal 5	RAN2 should consider if the intermediate relay UE that is still PC5 connected to the remote UE can perform RRC Re-establishment upon request from the remote UE.

# References

- RP-241609 “New WID on NR sidelink multi-hop relay”, LG Electronics, InterDigital, FirstNet