3GPP TSG-RAN WG2 Meeting #127	R2-2407208

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Source:	Panasonic

Title: 	Event triggered reporting enhancements for LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

# Introduction

As per WID [1] objectives to enhance measurement reporting in R-19 LTM, RAN2 had discussed further during RAN2#126 meeting, on introducing event triggered L1 measurement reporting for LTM and agreed some basic design aspects [2], few of them are listed below.

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This contribution discusses more details on event triggered L1 measurement reporting as per LTM procedure measurement reporting requirements. Further in the later section (2.2) the legacy event reporting procedure is evaluated for LTM measurement reporting requirements and listed the foreseen drawbacks which could be useful for consideration when defining event trigger conditions for LTM procedure.

# Discussion

In LTM (referring to R-18 LTM feature in 5G-NR; see figure 1), the early synchronization procedure, specifically the DL synchronization and UL TA acquisition, could be performed upon one or multiple candidate cells before triggering a cell switch command. The TA acquisition procedure is triggered by PDCCH order from the serving gNB/gNB-DU, while the TA value is calculated by the gNB/gNB-DU of each candidate cell which sends the TA value to the serving gNB/gNB-DU via gNB-CU, which could be sent to UE along with the cell switch command.

In one scenario, considering UL TA acquisition is triggered before the cell switch operation to one target cell determined for the intended LTM cell switch, if the TA value as expected from the target cell (target gNB/gNB-DU) is not available on time for the cell switch operation, cell switch is forced to continue as a RACH based operation. Here the reason for early TA acquisition failure can be due to TA acquisition procedure triggered too late, especially in a dynamic channel condition during UE mobility.

In another scenario, considering the UL TA acquisition procedure may be performed much earlier with one or more candidate cells prior to LTM cell switch decision phase, where all the TA acquisitions are ensured to be successful. Consequentially, the computed TA value of each (one or more) candidate cells can become invalid by the time when necessary for LTM cell switch operation towards one target cell, because the early TA acquisition procedure was triggered too early.

Therefore, it is necessary to provide regular monitoring of channel conditions of candidate beams/ cells also during early synchronization procedure to ensure DL synchronization and UL TA acquisition procedures, at least towards the target cell, to be successful and valid for LTM cell switch operation.

Figure 1: LTM procedure (source – TS38.300v18.1; 9.2.3.5 L1/L2 triggered mobility)

Observation 1: Regular channel monitoring using L1 measurement reports are necessary during early synchronization procedure triggered with one or multiple LTM candidate cells, to monitor and avoid the situation of ‘too early’ or ‘too late’ triggering of TA value acquisition.

## Regular measurement reporting using other reporting types for LTM

To regularly monitor the channel condition of each beam for LTM procedure during early synchronization execution and cell switch execution, LTMx event trigger conditions must be configurable to be reported using other reporting types, by regularly triggering measurement evaluations and reporting procedure.

Accordingly, the entering condition of LTMx events must be configurable to trigger as one or multiple options among periodic triggering, semi-persistent period triggering or by aperiodic requests sent from the network.

Proposal 1: LTMx event configuration to include other reporting type parameters to report measurements of one or more LTM candidate cells/beams regularly, based on one or more of the following reporting types.

- Periodic triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity parameters in LTM event configuration.

- Semi-persistent triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity parameters in LTM event configuration, and if the semi-persistent activation is sent by the network.

- Aperiodic triggering: LTMx event conditions are triggered and reported once. While new measurement evaluations are reported when aperiodic requests are sent by the network.

## LTMx event configuration for LTM procedures using legacy event reporting procedure.

In previous RAN2 meeting it was agreed to include evaluation condition such as ‘entering condition’ and ‘leaving condition’ for LTMx events assumed just like the legacy event trigger condition evaluation and reporting procedure.

Based on the LTM requirement for channel monitoring (using measurement reports) during early synchronization and cell switch procedure, assuming the entering condition for LTMx events can be shifted/ adjusted to fulfil early synchronization criteria and configured to UE, so that UE starts reporting earlier for each of the evaluated beams as per the early synchronization procedure requirements. And, when such LTMx event is configured to be triggered periodically, then measurement results are continued to be reported for the cell switch procedure requirements.

Example shown in figure 2, where event LTM4 (candidate beam becomes better than absolute threshold) is configured with threshold value shifted lower to fulfil early synchronization criteria along with periodicity configurations (like ‘report amount’ and ‘report interval’) to trigger measurement reporting for early synchronization first and then continued to be reported periodically during cell switch criteria.

Figure 2: Event LTM4 configured for early sync and cell switch measurement reporting

Considering measurement reporting for early synchronization procedure such that TA acquisition is not triggered ‘too late’, the entering condition can be (or likely be) configured to report much earlier which results in longer reporting amounts (or periodicity) to fulfil measurement reporting first for early synchronization scenario and continued to be reported until the time UE enters cell switch scenario. Here the measurement reporting for cell switch procedure is achieved using the same LTMx event configuration.

Observation 2: LTMx event configuration with shifted entering condition reported for early synchronization requirements, must be configured with larger reporting amounts (or periodicity) such that the evaluated beams for reporting does not encounter ‘too late’ situation for TA acquisition, while continuing to report for cell switch requirements using the same event configuration.

Also, when the threshold value is shifted to satisfy early synchronization for measurement reporting, it is likely that larger number of beams are triggered during evaluation of entering condition and moreover such measurements are continued to be reported for cell switch procedure (using the same periodicity) which appears to occupy UL resources unnecessarily. As seen in figure 3.

Figure 3: Illustration of shifting threshold to fulfil early synchronization criteria

Observation 3: Shifted entering condition triggers larger number of beams than necessary, which could be acceptable for early synchronization procedure. But when same number of beams are triggered and reported for cell switch procedure, appears to occupy more UL resources (for reporting) than expected.

Further to avoid triggering of larger number of beams during cell switch procedure, multiple LTMx event configuration could be defined for individual LTM procedures, with one (or more) LTMx event configured for early synchronization reporting (entering condition shifted to fulfil early synchronization criteria), while the other (one or more) LTMx event is configured for cell switch reporting. Now the LTMx event(s) which are dedicated to early synchronization reporting need not be configured with larger reporting amounts (or periodicity), which enables optimization of UL resource utilization during LTM procedure (occupying less resources for measurement reporting).

Observation 4: LTMx event configuration requires (or mandates) to always include multiple LTMx events within LTM event configuration to optimize UL resource utilization during measurement reporting for individual LTM procedures. For example, to configure lower reporting amounts (or periodicity) to save UL bandwidth during early synchronization reporting.

In view of the all the drawbacks seen above (observation 2 to 4), when the LTMx event configuration is adapted for LTM procedures as per the legacy event reporting procedure, it is evident that LTMx event configuration must be designed (re-designed) considering the measurement reporting requirements for individual LTM procedures.

Proposal 2: RAN2 to further study how LTMx events can be configured for early synchronization procedure using LTMx event configurations with the following views,

- Configurable to optimize UL resource utilization when LTMx event conditions are triggered for early synchronization reporting and cell switch reporting.

- Configurable to trigger optimum number of beams for individual LTM procedure, like early synchronization reporting and cell switch reporting.

- Considering single (only one) or multiple (as combination) LTMx event(s) configuration to trigger measurement reporting for LTM procedures, such as early synchronization procedure and cell switch procedure.

# Conclusions

Regular measurement reporting

Observation 1: Regular channel monitoring using L1 measurement reports are necessary during early synchronization procedure triggered with one or multiple LTM candidate cells, to monitor and avoid the situation of ‘too early’ or ‘too late’ triggering of TA value acquisition.

Proposal 1: LTMx event configuration to include other reporting type parameters to report measurements of one or more LTM candidate cells/beams regularly, based on one or more of the following reporting types.

- Periodic triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity parameters in LTM event configuration.

- Semi-persistent triggering: LTMx event conditions are triggered to generate and report measurement results based on the configured periodicity parameters in LTM event configuration, and if the semi-persistent activation is sent by the network.

- Aperiodic triggering: LTMx event conditions are triggered and reported once. While new measurement evaluations are reported when aperiodic requests are sent by the network.

LTM event configuration

Observation 2: LTMx event configuration with shifted entering condition reported for early synchronization requirements, must be configured with larger reporting amounts (or periodicity) such that the evaluated beams for reporting does not encounter ‘too late’ situation for TA acquisition, while continuing to report for cell switch requirements using the same event configuration.

Observation 3: Shifted entering condition triggers larger number of beams than necessary, which could be acceptable for early synchronization procedure. But when same number of beams are triggered and reported for cell switch procedure, appears to occupy more UL resources (for reporting) than expected.

Observation 4: LTMx event configuration requires (or mandates) to always include multiple LTMx events within LTM event configuration to optimize UL resource utilization during measurement reporting for individual LTM procedures. For example, to configure lower reporting amounts (or periodicity) to save UL bandwidth during early synchronization reporting.

Proposal 2: RAN2 to further study how LTMx events can be configured for early synchronization procedure using LTMx event configurations with the following views,

- Configurable to optimize UL resource utilization when LTMx event conditions are triggered for early synchronization reporting and cell switch reporting.

- Configurable to trigger optimum number of beams for individual LTM procedure, like early synchronization reporting and cell switch reporting.

- Considering single (only one) or multiple (as combination) LTMx event(s) configuration to trigger measurement reporting for LTM procedures, such as early synchronization procedure and cell switch procedure.

# References

[1]		RP-241515; RAN#104; Revised Work Item - NR mobility enhancements Phase 4.

[2]		R2-2405701; RAN2#126; Report from session on V2X/SL, R19 NES and MOB.