3GPP TSG-RAN WG2 Meeting #127	R2-2407211

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.4.1

Source:	InterDigital Inc.

Title:	Simulation assumptions and evaluation methodology for RLF prediction

Document for:	Discussion

# 1. Introduction

The RLF/HOF prediction use case was discussed in RAN2-126 and the following was agreed [1]:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is down prioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this contribution we will discuss some of the open issues regarding the simulation assumptions and KPIs.

# 2. Discussion

## 2.1 Model output and KPIs

Similar to what we proposed for the case of measurement events in [2], a model that outputs the probability of an RLF within a given predefined time window is sufficient to evaluate the usefulness/feasibility of RLF prediction and will decrease the complexity/time that will be required to train and test the model (for example, as compared to models that predict the probability distribution/values within multiple time windows or the earliest time the RLF is expected to occur, etc.,). Also, predicting at a given time instance is not

Proposal 1: For both direct and indirect prediction, the output of the AIML model is the probability of an RLF on the serving cell within a given time window.

Similarly, the same metrics/KPIs can be considered as in the case of measurement event prediction, as the RLF prediction is also a classification problem.

Proposal 2: The following to be taken as the baseline metrics/KPIs for the RLF prediction use case: Precision, Sensitivity, Specificity, F1-Score, and Accuracy

## 2.2 Simulation parameters

A UE that is in RRC_CONNECTED performs RLM on the SpCell to detect RLFs. The UE is configured with thresholds to determine whether the radio link being monitored is good/reliable enough, specifically:

- Qout:  the level at which the DL cannot be reliably received and shall correspond to out-of-sync block error rate (BLERout) which is the 10% block error rate of a hypothetical PDCCH transmission

- Qin: the level at which the DL can be significantly more reliably received than at Qout and shall correspond to in-sync block error rate (BLERin), which is 2% block error rate of a hypothetical PDCCH transmission

The UE is also configured with timers and counters that are used to determine the reliability of the link being monitored, specifically:

- n310:  the number of consecutive times that an out of sync (OOS) indication is received at the RRC from the lower layers (e.g. PHY) before RRC starts considering the link being monitored as experiencing reliability problem.

- n311: the number of consecutive times that an in-sync (IS) indication is received at the RRC from the lower layers (e.g., PHY) before RRC considers the link being monitored has become reliable again.

- t310: the duration of the timer that is started upon n310 consecutive OOS indications received from lowers, and stopped upon n311 consecutive IS indications.

If the T310 timer expires before the reception of n311 consecutive in-sync indications from lower layers, RRC will consider the link has failed and declares an RLF (and recovery performed by re-establishment or CHO).

The figure below illustrates the NR RLM and RLF detection mechanisms:

As agreed in RAN2-126 ([1]), RLF prediction can be done in two ways:

- Indirect: UE first predicts the future SINR values, and based on that predicts the RLF

- Direct: UE directly predicts the occurrence of the RLF without predicting future SINR values

In the case of indirect prediction, once the SINR predictions are made in the future, the legacy RLF detection procedure can be applied using the RLF parameters (i.e., Qin/out, N310/11 and T310).

In the case of direct prediction, since no intermediate SINR prediction is made, model training must be performed under different (sets of) RLF parameters. The RLF parameters can either be model inputs or different models need to be trained for different (set of) RLF parameters. Since the different parameters can take multiple values, model training for all the possible combinations can be a very time consuming.

In the HetNet mobility study in release 11, the RLF case was studied [3]. The simulation assumptions therein were to use only a single value for the different RLF parameters (table 5.2.1.3.1 in [3], as shown below):

Table 5.2.1.3.1: The parameters for determine the RLFs and the PDCCH failures.

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 36.331) |
| N310 | 1 |
| T311 | Not used for calibration (since RLF recovery is not simulated in the calibration) |
| N311 | 1 |

Our proposal is to reuse these same set of parameters for the simulation studies of RLF prediction, for both direct and indirect prediction cases.

Proposal 3: The following RLF parameters to be used for the simulations:

- Qout = -8 dB

- Qin   = -6 dB

- N310 = N311 = 1

- T310 = 1 sec.

In addition, most of the simulation assumptions of RRM use case that were agreed in [1][4] can be applicable to the RLF prediction use case. However, few changes for UE distribution, UE speed, and UE trajectory may be considered:

- UE Distribution: To invoke higher probability of RLFs within data samples, a percentage of UEs should be dropped closer to the cell edge. For RLF prediction use case, a combination of options 1 and 2 in Question 2.3.1.3-2 of the RRM prediction email discussion [4] can be considered where x% of UEs are uniformly dropped in the cell and (100-x)% of UEs are dropped in circular disk at cell edge.

- UE Speed: For RLF prediction use case, it can be more challenging and interesting to consider higher speed UEs (e.g., 60 kmh and 90 kmh), compared to the RRM prediction use case.

- UE trajectory: We recommend using option 3 (i.e., “random direction, straight line trajectory”) that was discussed in Question 2.3.1.1-1 of the RRM prediction email discussion [4] for UE trajectory in RLF prediction use case. This is because options 1 and 2 allow random change of direction, and thus can decrease the likelihood of RLFs in the data samples (e.g., UE may stay too long within the good coverage area of a given cell).  In addition, we recommend that inter-cell handover or beam switching should be enabled for fair comparison with non-AI baseline without RLF prediction (disabling inter-cell handover was suitable for AIML beam management study but may not be suitable for AIML mobility).

Proposal 4: For the RLF prediction use case, the following simulation parameters can be considered along with the other RRM prediction use case evaluation parameters:

- UE Distribution: A combination of options 1 and 2 in Question 2.3.1.3-2 [4] can be considered, i.e., x% of UEs are uniformly dropped in the cell and (100-x)% of UEs are dropped in circular disk at cell edge.

- UE Speed: Consider higher speed UEs (e.g., 60 kmh, 90 kmh).

- UE Trajectory: Consider option 3 in Question 2.3.1.1-1 [4] (i.e., “random direction, straight line trajectory”), with inter-cell handover or beam switching enabled for fair comparison with non-AI baseline.

# 4. Conclusion

In this contribution, the simulation assumptions and parameters for the RLF prediction use case were discussed and the following proposals were made:

Proposal 1: For both direct and indirect prediction, the output of the AIML model is the probability of an RLF on the serving cell within a given time window.

Proposal 2: The following to be taken as the baseline metrics/KPIs for the RLF prediction use case: Precision, Sensitivity, Specificity, F1-Score, and Accuracy

Proposal 3: The following RLF parameters to be used for the simulations:

- Qout = -8 dB

- Qin   = -6 dB

- N310 = N311 = 1

- T310 = 1 sec.

Proposal 4: For the RLF prediction use case, the following simulation parameters can be considered along with the other RRM prediction use case evaluation parameters:

- UE Distribution: A combination of options 1 and 2 in Question 2.3.1.3-2 [4] can be considered, i.e., x% of UEs are uniformly dropped in the cell and (100-x)% of UEs are dropped in circular disk at cell edge.

- UE Speed: Consider higher speed UEs (e.g., 60 kmh, 90 kmh).

- UE Trajectory: Consider option 3 in Question 2.3.1.1-1 [4] (i.e., “random direction, straight line trajectory”), with inter-cell handover or beam switching enabled for fair comparison with non-AI baseline.

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	R2-2403555, Measurement event prediction, Interdigital Inc., RAN2#125bis, April 2024

[3]	TR 36.839, Mobility enhancements in heterogeneous networks, Release 11, Dec 2012

[4]	R2-2404955, Summary of [POST125bis][021][AIML mobility ] Simulation assumptions and methodology, OPPO, RAN2-126, Fukuoka, Japan, May 2024.