3GPP TSG-RAN WG2 Meeting #127	R2-2407212

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.2.3

Source:	Nokia

Title:	Paging procedures for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions

Document for:	Discussion and Decision

# 1	Introduction

In RAN #103, the study item on Ambient Internet of Things (AIoT) solutions in NR was revised in RP-240826. In RAN2 #126, the following agreements were made related to Ambient IoT paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

The following agreements were also made with respect to Stage 2 general aspects of Ambient IoT.

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

In this contribution, we further discuss the considerations on paging for Ambient IoT and make some observations and proposals focusing on Topology 1.

# 2	Discussion

## 2.1	AIoT paging procedures

There are two types of traffic currently within scope, as defined in the general scope of the study item: DO-DTT (device-originated – device-terminated-triggered) and DT (device-terminated). In the case of DO-DTT traffic type, the device has data that it must send to the network; however, the device cannot initiate communication with the network on its own, but it can send data only in response to a trigger from the network. As defined by the scope of the study item, there is no consideration of RRC states for AIoT devices. Therefore, the term “paging” in the context of AIoT devices has no relationship with RRC states.

In RAN #126, some agreements (noted in Section 1) were made regarding the procedure to supporting the above traffic types in different application scenarios. The following use cases are considered.

Use Case: “Inventory only”

In this use case, associated with DO-DTT traffic, the base station triggers one or more AIoT devices to respond with their respective device IDs. Here it is agreed that the first step will consist of AIoT paging, which will be followed by a step consisting of device ID transmission. Therefore, the AIoT paging message is the initial trigger message for the procedure and indicates which device or devices should respond, and the devices then respond with an indication of the respective device ID(s). There are no further steps associated with this procedure.

Use Case: “Inventory and command”

According to the current agreements noted in Section 1, the steps of this procedure consist of the steps in Case A for “inventory” followed by additional steps associated with a “command”. Thus, the first two steps consist of an AIoT paging message followed by device ID transmission. In the subsequent step, the base station sends data or a command to one or more devices. In the last step, the device sends a corresponding response. The AIoT paging message is the initial trigger message for this procedure as well and indicates which device or devices should respond, and the devices then respond with an indication of the respective device ID(s)

The command may consist of either a “read” command or a “write” command. In case of a “read” command, associated with DO-DTT traffic, the base station commands one or more AIoT devices to respond with data (e.g., sensor data). The device or devices accordingly send data to the base station based on the command. In case of a “write” command, associated with DT traffic, the base station sends some kind of information or data to one or more AIoT devices. The device or devices may send corresponding feedback by way of response. Since the “command” steps follow the “inventory” steps, the information from the “inventory” steps may be used in the “command”, i.e., the IDs of devices that responded to the paging message may be indicated in the command message.

Use Case: “Command only”

There are no agreements yet on the steps associated with the procedure for this use case other than that RAN2 will study this use case. However, our understanding is that in this case, there are no steps associated with “inventory”. Therefore, the base station may directly send a command and/or data to one or more devices in the initial trigger message.

Proposal 1: In case of “inventory only” and “inventory and command” procedures, the AIoT paging message indicates whether the response shall contain device ID.

It is clear from the above cases that the “inventory and command” procedure can be distinguished from the “inventory only” procedure through adding the steps associated with sending a command or data to the device(s) and subsequently receiving data back from the devices. Therefore, in the “command only” use case, there is no need for separate steps associated with the “inventory” procedure. In other words, in this use case the network may want to directly send commands to devices without performing an inventory. For example, devices in this use case may be stationary or always within range of the base station. Therefore, it is not necessary to have separate steps for the purpose of obtaining device ID(s).

Proposal 2: RAN2 to assume the “command only” procedure does not include the steps associated with device ID delivery during “inventory” / “inventory & command” scenarios.

In the paging scenarios in cellular network, the RAN does not maintain any timer to track the device responses to AIoT paging or initial trigger messages. It is up to CN to retransmit the paging message with different escalation levels such as extending paging over larger area. For AIoT scenarios, the device mobility is not a prime consideration, but the reachability may be an issue to get a response to the paging message. In such cases, whether the reader can attempt for a specific number of retransmissions before the AMF attempts retransmission needs to be discussed.

Proposal 3: RAN2 to discuss trigger conditions for paging retransmissions by readers as well as the associated procedures.

## 2.2	AIoT paging message

There are two aspects involved in the triggering of communication by the reader through AIoT paging:

- Indication of paged device(s);

- Other information like

  - controlling access related parameters

  - data to be written by the device

We can again consider the three scenarios discussed in Section 2.1 for the above two aspects.

Indication of device ID(s):

Based on current agreements, the following cases can be studied for the AIoT paging message:

- A message containing the ID of a single device.

  - The network may send an AIoT paging message to inventory a single device, e.g., for the purpose of proximity detection.

- A message containing a group ID that maps to multiple devices.

  - The network may send an AIoT paging message to only a specific group of devices either for the purpose of inventory only to perform an inventory and subsequently send a command to only the devices that responded. It is understood that the group ID may be obtained through a mask of the device ID or some method of pre-configuration.

- A message that does not contain an ID, in which case it is addressed to all devices that can receive the AIoT message.

  - Similar to the previous case, the network may send an AIoT paging message to all devices that can receive the message either for the purpose of inventory only to perform an inventory and subsequently send a command to only the devices that responded. Details of the indication in the AIoT paging message for this case should be studied further.

- A message containing multiple IDs of AIoT devices. The need for this case should be confirmed based on SA2 discussion.

  - From RAN2 perspective, this is similar to the second case with the exception that multiple devices that have not been pre-configured with a group ID and hence has similar applicability. Thus, subject to SA2 confirmation for the need for this case, it should be supported.

Thus, in our view, each of the above cases is valid for consideration by RAN2. The network may need to send an AIoT paging message one or more devices to detect their presence or initiate communication with them.

Observation 1: RAN2 to discuss which of the following device identification options shall be used AIoT paging:

- device ID of a single device,

- (FFS further SA2 inputs),

- “broadcast” ID representing all devices able to receive a given AIoT paging message,

- fixed- and/ or variable-length list consisting of multiple device and/or group IDs (subject to SA2 support of this case).

From a practical point of view, it may be also beneficial to indicate the devices excluded from the list of paged devices which are not / no more expected to respond (e.g. since their response was obtained in the past or is of lower priority).

Proposal 4: RAN2 to discuss whether to an AIoT paging message may also indicate device / group IDs of AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore it.

Other information

To ensure that devices respond using the appropriate form of random access (contention-based or contention-free) as well as the correct resource, access type and the associated resources shall be indicated. For example, when contention-free access is selected by the reader, resource indication may indicate the granted resource (e.g., time slot) relatively to current time (e.g., by using nextSlotOffset information). Alternatively, when pure random access is selected, a specific set of suitable resources (e.g., the next N slots for an initial response attempt) or the appropriate random access configuration (e.g., backoff parameters).

Similarly, service type may need to be also indicate the procedure type and length (e.g., number of subsequent steps until service termination). For example, the “command” scenario requires more steps compared to pure “inventory” use case.

Proposal 5: RAN2 to agree that an AIoT paging message contains at least one of the following

- access type (“Contention-based random access”, “Contention-free access” )

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

- associated response resources and related parameters (FFS details).

Proposal 6: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device by default means “Contention-free access”,

- an AIoT paging of a device group by default means “Contention-based random access”.

As part of the “write” command service, dynamic configuration updates could be sent to one or more devices. The information sent may comprise the full configuration or only the part of the configuration that has changed (differential or “delta” information). The information can include parameters for device access in response to inventory or “read” command requests. In case of paging multiple devices, parameters for random access (e.g., similar to a Q-parameter in the RFID standard) can be provided. Alternatively, when a single device is paged for a “read” command service request, scheduling parameters for fixed resource allocation can be provided. If dynamic configuration update through the paging procedure is supported, further study is needed on the frequency of such updates.

Proposal 7: RAN2 to discuss the signaling of  MAC parameters / configuration / updates during paging procedures, including the parameters needed to determine backoff time (if needed).

It is further observed that a paging request may be retransmitted and / or reconfigured. For example, the targeted devices may be changed (e.g. based on priority if the response time is protracted due large-scale contention implying long / frequent backoff) or MAC parameters may be updated (e.g., backoff time is shortened if contention is low).

To this end, it would be beneficial to uniquely identify individual paging requests, e.g. by using a paging index or ID that is preferably unique over some time. Alternatively, single-bit indication of a new paging occasion may be used, similarly to New Data Indication.

Paging request identification is also needed when multiple paging requests occur simultaneously (be it by originating from the same reader, or multiple independent readers).

Proposal 8: RAN2 to agree that individual paging requests are identified for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Alternatively, a paging request may be terminated if a sufficient amount of devices have responded, or the available resources will become unavailable (e.g., the need to reserve them for SL communications). Also, massive contention and contention may result in excessive response times. If an upper-layer deadline elapses, there is no need to further collect and forward data from AIoT devices.

Proposal 9: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

As discussed in Section 2.1, there is no command or other data sent in the AIoT paging message in the “inventory only” and “inventory and command” use cases. For the inventory step, only device ID(s) may be indicated based on which AIoT devices send response messages indicating their respective IDs. While no data or command is included in the AIoT paging message in these cases, control information related to the inventory procedure (e.g., random access parameters) may be included.

The “inventory and command” procedure employs what may be called a two-step approach. In this approach, the indication of device/group ID is provided in an AIoT paging message in the first step (corresponding to inventory), and indication of information related to data communication is provided in a separate command message in the second step. This information may comprise not only a command but also DT data (in the case of “write” command) specific to one or more devices.

On the other hand, the “command only” procedure is a one-step approach for communication with AIoT devices. In this approach, both the device/group ID indication and the indication of the information related to data communication (e.g., “command”, DT data) are provided in the initial trigger message. In many use cases, the “command only” procedure for communicating with devices may be preferable since a separate inventory step may be unnecessary and would increase the signaling overhead.

However, including the DT traffic data in the initial trigger message may lead to a failure to deliver the data if the device is not reachable. Moreover, delivering the traffic directly in the initial trigger message may lead to some security issues and it may be necessary for SA3 to analyze the security issues associated with this approach.

Proposal 10: RAN2 to consider inclusion of the network information such as DT traffic data and/or command in the initial trigger message.

Proposal 11: If inclusion of network information in initial trigger message is agreed, RAN2 to inform SA3 to analyze the security issues related to this option.

## 2.4	Consideration of different Ambient IoT devices for paging

We adopt the following terminologies agreed in RAN1 for the devices that are within the scope of the study.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.Devices 1 and 2a are the same in terms of basic capabilities – both perform uplink transmission only via backscattering – with the difference between them being that Device 2a is also capable of DL and/or UL amplification. On the other hand, Device 2b is capable of generating an uplink transmission internally. Nevertheless, the uplink transmission time is under network control for all devices. While the backscattering transmission time for Devices 1 and 2a is determined by the activation signal, the uplink transmission time for Devices 2b may be scheduled or configured by the network.

In the legacy paging procedure, the AMF may initiate the procedure by sending a paging message to the gNB of the cell where the UE is expected to be camping. The gNB then pages the UE. If the UE is expected to respond to the paging, it does so by initiating the random access procedure. In the case of Device 2b, it may be possible to use the legacy random access procedure, whereas in the case of Devices 1 and 2a, the network controls the paging response procedure. The objective is to develop a harmonized air interface design. It would be possible to harmonize the paging procedure for the different Ambient IoT devices in terms of the high-level messages.

However, the differences in the capabilities of the Ambient IoT devices may result in some differences in how devices access the network in response to paging. For example, given that uplink transmission of Device 2b is internally generated rather than backscattered, the transmission can be delayed with respect to the triggering message. As a result, if the network has prior knowledge on the type of device being paged, configuration and control information related to random access or resource allocation that is contained in the paging message may be different.

Therefore, the content of paging message may depend on the targeted device when the device type is known to the reader. However, there may scenarios in which the network has no prior knowledge of the device. In this case, a common paging message can be used for all devices. Subsequent message content can be different based on the device.

Observation 2: Paging procedure for different Ambient IoT devices (1, 2a, 2b) can be harmonized, but content of paging message can be device dependent when prior knowledge of device type is available to the reader.

Proposal 12: RAN2 to study differences in AIoT paging messages for Device 1/2a and Device 2b when prior knowledge of device type is available to the reader.

# 3	Conclusion

In this contribution, the considerations on paging for Ambient IoT are discussed. The following observations and proposals are made.

Observation 1: RAN2 to discuss which of the following device identification options shall be used AIoT paging:

- device ID of a single device,

- (FFS further SA2 inputs),

- “broadcast” ID representing all devices able to receive a given AIoT paging message,

- fixed- and/ or variable-length list consisting of multiple device and/or group IDs (subject to SA2 support of this case).

Observation 2: Paging procedure for different Ambient IoT devices (1, 2a, 2b) can be harmonized, but content of paging message can be device dependent when prior knowledge of device type is available to the reader.

Proposal 1: In case of “inventory only” and “inventory and command” procedures, the AIoT paging message indicates whether the response shall contain device ID.

Proposal 2: RAN2 to assume the “command only” procedure does not include the steps associated with device ID delivery during “inventory” / “inventory & command” scenarios.

Proposal 3: RAN2 to discuss trigger conditions for paging retransmissions by readers as well as the associated procedures.

Proposal 4: RAN2 to discuss whether to an AIoT paging message may also indicate device / group IDs of AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore it.

Proposal 5: RAN2 to agree that an AIoT paging message contains at least one of the following

- access type (“Contention-based random access”, “Contention-free access” )

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

- associated response resources and related parameters (FFS details).

Proposal 6: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device by default means “Contention-free access”,

- an AIoT paging of a device group by default means “Contention-based random access”.

Proposal 7: RAN2 to discuss the signaling of  MAC parameters / configuration / updates during paging procedures, including the parameters needed to determine backoff time (if needed).

Proposal 8: RAN2 to agree that individual paging requests are identified for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Proposal 9: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

Proposal 10: RAN2 to consider inclusion of the network information such as DT traffic data and/or command in the initial trigger message.

Proposal 11: If inclusion of network information in initial trigger message is agreed, RAN2 to inform SA3 to analyze the security issues related to this option.

Proposal 12: RAN2 to study differences in AIoT paging messages for Device 1/2a and Device 2b when prior knowledge of device type is available to the reader.