3GPP RAN WG2 Meeting #127	R2-2407213

# Maastricht, Aug 19th – 23rd, 2024

Agenda Item:	8.7.2.

Source:	InterDigital

Title:	Multi-modality support for XR

Document for:	Discussion

# 1. Introduction

In this contribution, we discuss multi-modality support for XR according to the WI objective as follows [1]:

Objective: Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. Including aspects such as: - potential enhancements based on multi-modal information awareness depending on traffic direction (UL/DL) - can the multi-modal information be provided from the UE - other enhancements for multi-modal traffic not strictly related to multi-modality awareness, e.g. power saving, scheduling

RAN2#126 [2] made the following agreements:

Support Multi-modality awareness in RAN in Rel-19 for UL and DL. Ask SA2 whether/what information could be provided to RAN

# Enhancements for multi-modal flows

Multi-modality was initially identified by SA1 for the tactile and multi-modal communication service [3] and has been discussed in Rel-18 SA2 [4]. In Rel-18, SA2 specified a multi-modal service ID, which is provided by the application to the CN to indicate the inter-dependency of different data flows to a multi-modal service. In Rel-18 the multi-modality information is not provided by the CN to the RAN, and as such, the Rel-18 enhancements in RAN/AS did not address aspects specific to multi-modality. RAN2 asked SA2 in [5] what multi-modal information can be provided from SA2 to the RAN (e.g., MMSID and/or synchronization thresholds) in order to allow coordinated handling of the flows at RAN. The reply from SA2 in [6] was that there is no consensus in SA2 in providing the multi-modality ID to NG-RAN.

## Multi-modal awareness at RAN

According to SA1 [3], for immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e., viewers detecting lack of synchronization), particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application. Transmission of the coordinated flows (e.g., audio, visual and tactile) translate into a synchronization requirement between the flows. These types of synchronization are typically handled by the application, but awareness of the coordinated flows at the lower layers may also be beneficial to the user experience.

One way to address the issues related to multi-modality in the RAN is to use an alternative QoS associated with the multi-modal flows during uplink and downlink transmissions. For example, if there are dependencies between some flows and in the event the QoS of a video flow cannot be met temporarily such that an alternative QoS has to be applied, the QoS of a dependent audio flow can also be adjusted so that a similar delay is experienced by all the dependent flows. The alternative QoS approach, which is specified in SA2 [4], can consist of a prioritized list of multiple QoS levels (e.g., combination of PDB, PER) associated with a flow that the application is able to support and make adaptations accordingly.

For downlink traffic, any dependency/synchronization requirements and exposure of related information to the RAN can be left to SA2/RAN3. As it is, there is currently no consensus in SA2 to provide the multi-modality ID to NG-RAN [6]. For the uplink, the network may not be aware of dependency/synchronization requirements of the traffic generated at the application in the UE. If the UE is aware of any dependency/synchronization requirements between dependent flows in the uplink, reporting such information to the network may assist the network in resource provisioning and scheduling. The UE may report multi-modality information to the network via the UAI framework. Such information may include any parameter that characterizes the dependency/synchronization requirements between the flows, including the multi-modal service ID, an indication of association between flows and any synchronization requirements between the flows.

Observation 1:	Awareness of dependency/synchronization information in the uplink is useful for resource provisioning and scheduling.

Proposal 1:	UE to report on dependency/synchronization information of dependent uplink multi-modal flows when applicable.

Proposal 2:	Enhance UAI to report on dependency/synchronization information of dependent uplink multi-modal flows.

Proposal 3:	Dependency/synchronization information includes the multi-modal service ID, an indication of association between flows and any synchronization requirements between the flows.

## Scheduling enhancements from multi-modal awareness

The objectives for this agenda include scheduling enhancements for multi-modal traffic not strictly related to multi-modality awareness. Enhancements are being discussed in the scheduling agenda on overriding/adjusting the priority of logical channels based on the delay/deadline information in case these logical channels have delay-critical data to prevent any negative impact to the overall system capacity from data with a smaller remaining time not meeting its respective PDB/PSDB due to it being mapped to a lower priority LCH. For multi-modal applications, there may be a similar negative impact to the capacity since there exists a synchronization requirement between dependent flows, in addition to the absolute delay requirement for individual PDUs/PDU sets.

Observation 2:	Awareness of dependency/synchronization information is useful to minimize delay for synchronized transmission of dependent flows.

Proposal 4:	Discuss enhancements to LCP to consider synchronization requirements between dependent flows when multiplexing data in a MAC PDU.

## Power saving enhancements from multi-modal awareness

Support of multiple active CDRX cycles was discussed in Rel-18 XR but was not agreed since the motivation for it was not strong as multi-modal flows were not in scope for Rel-18. In the context of multi-modal flows where the UE may support multiple flows with different traffic patterns, it can be beneficial to support multiple active CDRX configurations corresponding to the different flows. For example, for a traffic flow with relatively low arrival rate and small payload sizes (e.g., audio data flow), a dedicated CDRX configuration with low periodicity and short ON duration can be applied instead of using a single CDRX configuration with high periodicity and long ON duration that is configured to support multiple traffic patterns. Essentially, by using multiple active CDRX configurations that match with the traffic patterns of different flows it is possible to tailor the CDRX parameters/configurations to the individual flows and realize both power savings gains and meet the QoS requirements of each flow.

Proposal 5:	Study and enhance multiple active DRX configurations to achieve power saving gains for multi-modal flows.

# Conclusion

In this contribution, the following observations are made:

Observation 1:	Awareness of dependency/synchronization information in the uplink is useful for resource provisioning and scheduling.

Observation 2:	Awareness of dependency/synchronization information is useful to minimize delay for synchronized transmission of dependent flows.

Based on this, the following proposals are made:

Proposal 1:	UE to report on dependency/synchronization information of dependent uplink multi-modal flows when applicable.

Proposal 2:	Enhance UAI to report on dependency/synchronization information of dependent uplink multi-modal flows.

Proposal 3:	Dependency/synchronization information includes the multi-modal service ID, an indication of association between flows and any synchronization requirements between the flows.

Proposal 4:	Discuss enhancements to LCP to consider synchronization requirements between dependent flows when multiplexing data in a MAC PDU.

Proposal 5:	Study and enhance multiple active DRX configurations to achieve power saving gains for multi-modal flows.

# 4. References

[1]	RAN2#127 Agenda, XR enhancements Ph3

[2]	RAN2#126 Chair notes, May 2024

[3]	3GPP TS 22.261, Service requirements for 5G system

[4]	3GPP TR 23.700-60, Study on XR and media services

[5]	R2-2405782, LS to SA2, May 2024

[6]	S2-2406965, [draft]LS on multi-modality awareness at RAN