3GPP RAN WG2 Meeting #127	R2-2407215

# Maastricht, Aug 19th – 23rd, 2024

Agenda Item:	8.7.5

Source:	InterDigital

Title:	RLC enhancements for XR

Document for:	Discussion

# 1. Introduction

In the last RANP #103, the revised WID for XR is agreed [1], with the following objective for user plane enhancements:

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In the previous meetings [2][3], agreements are made on RLC enhancements for XR and provided in the appendix. In this contribution we further discuss RLC AM enhancement for small packet delay budget. Specifically, we further discuss enhancements to avoid unnecessary retransmissions and timely RLC retransmissions for a RLC PDU.

# Discussion

  - 2.1 Avoiding unnecessary retransmissions

In the last meeting, for avoiding unnecessary RLC AM retransmission, RAN2 agreed to further study Rx-initiated and Tx-initiated approaches and choose one of the two. In the following, we provide our analysis of the two approaches.

Rx-initiated approach:

In this approach, for proper advancing of the transmitting window, the receiver may indicate the abandoned SDUs to the Tx (e.g., indicate in a status report the abandoned SDUs as ACKs). Afterward, the transmitter may discard the abandoned SDUs and advance its transmitting window. One remaining issue for this approach is to determine when an SDU should be abandoned. One possibility discussed in the last meeting is to use a timer (e.g., reordering timer) to determine whether an SDU is abandoned. Specifically, the Rx may start a timer upon detection of an SN gap in the reception buffer and send a report indicating the abandoned SDUs (e.g., the SDUs within SN gap) upon the expiry of the timer.

In our view, one benefit of this approach is simplicity as it may rely on the existing report (e.g., status report) procedure to advance the transmitting window. There may be no spec impact for the Tx as it just processes the status report as in legacy. Even if the status report is not received successfully by the Tx entity, e.g. upon HARQ failure, the Tx entity can still retransmit the unnecessary SDUs that will be eventually discarded by the Rx entity eventually once out of the window. Therefore, this approach does not have a big specification impact. However, one drawback of this approach is that the Rx entity is not aware of the delay budget of a SDU at the transmit entity as it is not aware of the arrival time of each SDU at the Tx. Afterall, this discussion on avoiding unnecessary retransmissions stemmed from avoiding retransmissions of packets that have exceeded the PDB, which is better known at the transmitter rather than the receiver. Hence, it may be a challenge for the Rx to properly determine when to provide the status report for abandoning certain SDU.

Observation 1: 	The Rx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: Simplicity and less specification impact. Rely on existing status report at the receiver to indicate abandoned SDUs as ACKs.

- Drawback: the Rx entity may not be aware of the delay budget of a SDU to properly determine whether/when an SDU should be abandoned.

Tx-initiated approach:

The purpose of avoiding unnecessary retransmissions is to avoid retransmissions of the SDUs that have exceeded the delay budget, which is known at the Tx not the Rx. It is because Tx has full knowledge of when/whether an SDU is obsoleted and should be abandoned (e.g., based on expiry of a discard timer). Upon abandoning one or more obsoleted SDUs, the Tx can notify the receiver about the obsolete SDUs, and the Rx may update the receiving window according to the information from the Tx. Therefore, the Tx-initiated approach can completely avoid unnecessary retransmissions.

However, RAN2 need to further discuss how to report the abandoned SDUs and the subsequent procedure if the report is not decoded at the Rx. It is because if the report is not decoded at the Rx, it may create an SN gap and stall the receiving window. Moreover, RAN2 may need to further discuss what the Rx to report in a status report when it receives a notification about abandoned SDUs from the Tx.

Observation 2: 	The Tx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: the Tx entity is fully aware of the PDB and whether/when a SDU should be abandoned. Therefore, it can avoid unnecessary retransmissions more accurately.

- Drawback: the Tx-initiated approach can require more specification impact, especially if the report is not decoded at the Rx entity, potentially creating an SN gap and stalling the RLC AM receive window.

  - 2.2 Timely RLC retransmissions

In the last RAN2 meeting, it was agreed to consider two options (autonomous retransmission, polling enhancement) to support timely RLC retransmission. Status report enhancement was discussed; however, it was agreed to focus enhancement on the uplink. In the following, we discuss the necessity/benefits to support each option.

Autonomous retransmission:

In the current operation of RLC Acknowledgement Mode in NR, the UE first performs the initial transmission of an RLC PDU. Afterwards, the UE only triggers retransmission of an RLC PDU if it receives an RLC status report from the gNB indicating the NACK status for the RLC PDU. For RLC status reporting, the gNB can send a NACK status for an RLC PDU when detecting an SN gap between two consecutively received RLC PDUs [4]. Such a procedure may cause a long delay for a retransmission of an RLC PDU which may not be suitable for an RLC PDU with a small packet delay budget (PDB). This is because, after such a long delay, the retransmission of the RLC PDU may arrive beyond the PDB of the PDU and, hence impacting the QoS.

Therefore, enhancing the RLC re-transmission for the operation of the RLC AM can be beneficial. To enhance the RLC re-transmission for an RLC PDU with small PDB, the delay gap between the initial transmission and the re-transmission of the RLC PDU can be reduced. One possibility is to allow the UE to retransmit an RLC PDU without waiting for the RLC status reporting from the network. For example, the UE can retransmit the RLC PDU if the remaining time of the PDU is smaller than a configured threshold. Such retransmission can help improve the successful reception of the RLC PDU with small delay budget. Alternatively, the UE may fail to transmit an RLC PDU due to an improper scheduling of transmission parameters such as the resource size, MCS, beam, etc. Therefore, with multiple HARQ retransmissions, the UE may still fail to successfully transmit the RLC PDU. In such cases, it is beneficial for the UE to retransmit another RLC PDU using another set of transmission parameters. Specifically, the UE may retransmit the RLC PDU, if the number of HARQ retransmissions for the RLC PDU exceeds a threshold.

For autonomous RLC retransmission based on the remaining time of a PDU, the network can properly configure a delay threshold to minimize the capacity loss while still guarantee the QoS (e.g., reliability) of the service. Regarding retransmission based on the number of HARQ retransmissions, capacity loss is minimal as the UE may still need to retransmit the PDU as it has not been received successfully.

Proposal 1:	Support the UE to retransmit a RLC PDU if the remaining time of the PDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Polling enhancement:

Regarding the polling procedure in NR, the UE is allowed to poll the RLC reception status of the gNB by setting the polling bit in the RLC header to one in the following cases:

    - The amount of data without polling is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte).

    - Its buffer (initial and retransmission buffer) will be empty after the current RLC PDU.

    - Its window is stalling (e.g., the gap between the current SN and the ACKed SN is greater than the window size).

In XR applications, the receiver decodes the received data per PDU set. Therefore, the reception status of each PDU set is more important than the reception status of each PDU in a PDU set. Moreover, in an XR application, the PDU sets may arrive at the UE periodically with a certain jitter. It is also expected that the amount of data per PDU set may vary from one PDU set to another. Specifically, the number of PDUs and the number of bytes per PDU set can be different among PDU sets.

If the current polling procedure is used, in which the UE polls the gNB after a configured number of PDUs or bytes, it is most likely that the UE may poll the RLC status of the gNB in the middle of a PDU set. As a result, the reception status of each PDU set may not be reported. To reduce the delay in determining the reception status of a PDU set, the UE can be configured with a smaller values of pollByte and/or pollPDU. However, this configuration may result in high signaling overhead as the UE may need to receive many more RLC status reports when the values of pollByte and/or pollPDU is small. Therefore, it may be beneficial to enhance the polling procedure, which may help the UE to receive the RLC status of one or more PDU sets. For example, the polling bit can be set based on the transmission of a PDU set (e.g., the last PDU of a PDU set) or it can be set periodically based on the PDU set traffic pattern.

Observation 3: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Proposal 2:	Support polling enhancements to allow the UE requesting the status report after transmissions of each uplink PDU set.

# Conclusion

In this contribution, the following observations are made:

Observation 1: 	The Rx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: Simplicity and less specification impact. Rely on existing status report at the receiver to indicate abandoned SDUs as ACKs.

- Drawback: the Rx entity may not be aware of the delay budget of a SDU to properly determine whether/when an SDU should be abandoned.

Observation 2: 	The Tx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: the Tx entity is fully aware of the PDB and whether/when a SDU should be abandoned. Therefore, it can avoid unnecessary retransmissions more accurately.

- Drawback: the Tx-initiated approach can require more specification impact, especially if the report is not decoded at the Rx entity, potentially creating an SN gap and stalling the RLC AM receive window.

Observation 3: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Based on this, the following proposals are made:

Proposal 1:	Support the UE to retransmit a RLC PDU if the remaining time of the PDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Proposal 2:	Support polling enhancements to allow the UE requesting the status report after transmissions of each uplink PDU set.

# References

- RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, 2024-03.

- RAN2#125bis chair notes.

- RAN2#126 chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: Agreements on RLC enhancement for XR

RAN2#125b Agreements

- Agreements: We focus on RLC AM. RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR. RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

RAN2#126 Agreements

Agreements: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed, or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic