3GPP RAN WG2 Meeting #127	R2-2407216

# Maastricht, Aug 19th – 23rd, 2024

Agenda Item:	8.7.1.1

Source:	InterDigital

Title:	Discussion on LSs from SA2 and RAN3

Document for:	Discussion

# 1. Introduction

RAN2 received an LS from SA2 on dynamic signaling support for periodicity and time to next burst [1] and an LS from RAN3 on DC signaling enhancements for split bearer [2]. In this contribution, we discuss the responses for the two LS’s.

# Reply LS to S2-2407351

In this section, we discuss the feedback for the LS on dynamic signaling support for periodicity and time to next burst [1].

The first question for RAN2 in the LS is as follows:

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signalling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

We can wait for SA4 to determine whether it is possible for the application server to provide periodicity information dynamically via in-band signalling. In any case, GTP-U is not in scope of RAN2 specifications and usefulness of dynamic periodicity indication for the downlink traffic is a gNB implementation aspect.

Proposal 1: 	Reply to SA2 for the first question with the following:

    - GTP-U is not in scope of RAN2 specifications and usefulness of dynamic periodicity indication for the downlink traffic is a gNB implementation aspect.

    - There is no specification impact to RAN2 for dynamically indicating periodicity via in-band signalling in the downlink.

SA2 further requests feedback from RAN2 to the second question as follows:

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

We can wait for SA4 to determine whether it is possible for the application server to provide the time to next burst dynamically in the first packet of the burst. If it is possible, it may be useful to the RAN for scheduling and power saving purposes. Any potential signalling impact would be up to RAN3.

Proposal 2: 	Reply to SA2 for the second question with the following:

    - Wait for SA4 to determine if it’s possible for the application server to provide the time to next burst dynamically in the first packet of the burst.

    - Any resulting signalling impact would be up to RAN3.

# Reply LS to R3-243957

In this section, we discuss the feedback for the LS on UL PSI based PDU discarding in NR-DC [2].

The issue addressed in the LS is as follows:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

This issue is currently being discussed in RAN3 where some companies are in favour of specifying a message on the Xn interface to ensure that there is coordination between the MN and SN. On the other hand, a handful of companies are of the view that there is no need to specify any message and the UE can simply follow the latest received MAC CE for the ACT/DEACT of the PSI based SDU discard.

RAN2 has not addressed split bearer in the context of XR so far, and does not have time allocated to that end in Release 19. If RAN3 were to specify a message on the Xn interface, it would be useful in ensuring that the MN and SN are always coordinated on the ACT/DEACT of the PSI-based SDU discard. If RAN3 decides not to specify any message on the Xn interface, the likelihood of the PDCP entity in the UE receiving inconsistent commands from the MN and SN nodes is still low, relegating this scenario to a corner case. In the unlikely event that it does happen, there is little impact since the PSI level to do discarding at PDCP is up to UE implementation. Furthermore, nothing gets broken in this situation, the UE follows the latest received MAC CE for the ACK/DAECT of the PSI based SDU discard. We therefore think there is no need to specify any UE behaviour for this case.

Proposal 3: 	Reply to RAN3 with the following:

    - The likelihood of the PDCP entity in the UE receiving inconsistent commands from the MN and SN nodes is low.

    - PSI level that triggers discarding at PDCP is up to UE implementation, resulting in little impact in the unlikely event of inconsistent commands from the MN and SN nodes.

    - RAN2 does not think any UE behaviour needs to be specified for handling uplink PSI based SDU discard for split bearer.

# Conclusion

In this contribution, the following proposals are made:

Reply LS to SA2 on dynamic signaling support for periodicity and time to next burst

Proposal 1: 	Reply to SA2 for the first question with the following:

    - GTP-U is not in scope of RAN2 specifications and usefulness of dynamic periodicity indication for the downlink traffic is a gNB implementation aspect.

    - There is no specification impact to RAN2 for dynamically indicating periodicity via in-band signalling in the downlink.

Proposal 2: 	Reply to SA2 for the second question with the following:

    - Wait for SA4 to determine if it’s possible for the application server to provide the time to next burst dynamically in the first packet of the burst.

    - Any resulting signalling impact would be up to RAN3.

Reply LS to RAN3 on XR DC signaling enhancements for split bearer

Proposal 3: 	Reply to RAN3 with the following:

    - The likelihood of the PDCP entity in the UE receiving inconsistent commands from the MN and SN nodes is low.

    - PSI level that triggers discarding at PDCP is up to UE implementation, resulting in little impact in the unlikely event of inconsistent commands from the MN and SN nodes.

    - RAN2 does not think any UE behaviour needs to be specified for handling uplink PSI based SDU discard for split bearer.

# References

- S2-2407351, “LS on FS_XRM Ph2”, SA2, 2024-05.

- R3-243957, “LS on UL PSI based PDU discarding in NR-DC”, RAN3, 2024-05.