3GPP TSG-RAN WG2 #127	R2-2407217

Maastricht, NL, August. 19th – 23th, 2024

Agenda Item:	7.13.2

Source:	Ericsson

Title:	Addressing SONMDT issues

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address some of the remaining SONMDT issues, including the absence of the plmn-IdentityList in logged MDT configuration in SNPN networks and some miscellaneous corrections on the CEF report procedural text.

# 2. Discussion

## 2.1 number of PNI-NPN identities configuration

According to RAN3 NGAP TS 38.413 the maximum number of CAG identities (PLMN + NID) to be configured as part of MDT area scope is 256. This is highlighted in the following excerpt from the TS 38.413.

Range bound; Explanation. maxnoofCellIDforMDT; Maximum no. of Cell ID subject for MDT scope. Value is 32.. maxnoofTAforMDT; Maximum no. of TA subject for MDT scope. Value is 8.. maxnoofCAGforMDT; Maximum no. of CAG IDs for MDT scope. Value is 256.. maxnoofMDTSNPNs; Maximum no. of SNPNs in the MDT SNPN list. Value is 16..

However, the current implementation in the RRC TS 38.331 (version 18.1.0) is not aligned with the RAN3 solution. More specifically, the current solution in the RRC spec is incapable to support the scenario that the network wants to configure MDT measurement collection for more than 12 PNI-NPN from more than 12 PLMNs.

RAN3 has investigated this issue and agreed that for the sake of reducing UE memory consumption network configures the UE only 144 NPN identities (i.e., PLMN + CAG) with maximum 12 PLMNs that can be repeated in the configuration. This is shown in the following excerpt from RAN3 reply LS [R3-243892].

RAN3 answer: RAN3 agreed to keep the number of PNI-NPN identities (PLMN ID+CAG) supported in RAN3 specifications to 256. However to alleviate the UE memory consumption issue highlighted by RAN2, for logged MDT, RAN3 specifications have been enhanced with amendments in the semantics of the CAG List for MDT , stating that this list contains a maximum of 144 Public Network Integrated NPNs with a maximum of 12 different PLMN identities where a PLMN ID may be repeated more than once (see attached CRs).

Based on the above reply in RAN2 we can keep the current implementation and no change is required.

- RAN2 agree that no change on the number of PNI-NPN identities in MDT configuration is needed i.e., maximum (12 x 12) PNI-NPN identities (i.e., 12 PLMNs and 12 CAGs per PLMN). E022 is closed.

## 2.2 plmn-IndentityList in SNPN

The current procedural text mandates to collect Logged MDT measurements in SNPN, UE needs to be configured with snpn-configIDList IE which is sent as part of AreaConfiguration in LoggedMeasurementConfiguration message.

However, if the UE is registered in an SNPN and OAM initiates the Logged MDT configuration with only plmn-IdentityList some companies had the understanding that the UE would need to log MDT measurements in any SNPN that is served by the PLMNs in the plmn-IdentityList. In such scenario, AreaConfiguration could be absent and according to RRC specification.

RAN3 has investigated this issue and replied the following in the Reply LS [R3-243892] i.e., the plmn-IdentityList cannot be configured for the MDT collection in SNPN networks.

RAN3 answer: plmn-IdentityList applies only to public networks and it shall not be used for SNPNs

Therefore, we propose the following.

- RAN2 capture in the RRC spec that plmn-identityList is not included in case of MDT collection in SNPNs. E268 is closed.

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies, the included PNI-NPN identities or the SNPN identities. If areaConfiguration-r17 is present, the UE shall ignore areaConfiguration-r16. The areaConfiguration-v1800 is a non-critical extension of areaConfiguration-r17. See NOTE 1.

earlyMeasIndication If included, the field indicates the UE is allowed to log measurements on early measurement related frequencies in logged measurements.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs. The network does not include this field when the UE is configured with MDT configuration in SNPN access mode.

sigLoggedMeasType If included, the field indicates a signalling based logged measurement configuration (See TS 37.320 [61]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggerd MDT configuration.

## 2.6 RIL [E257] CEF report logging under T300 Expiry.

The procedural text for logging the CEF report under T300 Expiry is not properly written and some indention seems to be required. In addition for the sake of appending the  VarConnEstFailReport to the   VarConnEstFailReportlist it is enough to compare the registered snpn-identity with the snpn-identitty in the  VarConnEstFailReport; and it is not needed to check any of the entry in VarConnEstFailReportList;

In fact, we could follow the same procedural text written for the CEF report collection in the public network (highlighted in yellow colour) as shown in the following excerpt from 3GPP RRC spec. Notelably the current procedural text does not work when the CEF report list is empty.

- The CEF report procedural text (on checking the registered SNPN with the existing SNPN identity of the CEFR report list) is not applicable when the CEF report list is empty.

2>	if the UE supports multiple CEF report:

3>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

3>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in any entry of VarConnEstFailReportList:

4>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

5>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

Hence, we propose the following:

- RAN2 agree that similar to the procedural text for CEF in public network UE compares the registered snpn-identity with the snpn- identity logged in VarConnEstFailReport when appending the VarConnEstFailReport to the VarConnEstFailReportlist.

In addition, based on the SONMDT correction CR [R2-2405968] sent to the plenary meeting, the following change (changing VarConnEstFailReportlist to VarConnEstFailReport) was expected to be applied on the new version of the RRC specification version 18.2.0.

However, in the new version the change is not applied (apparently it is missed). Therefore, we propose to solve this issue in the next version.

- RAN2 agree to fix the missing correction in [R2-2405968] (changing VarConnEstFailReportlist to VarConnEstFailReport) in the next version of the RRC TS.

- RAN2 agree to the draft CRs in the annex of the paper.

# Conclusion

We have the following observations in the previous section.

- The CEF report procedural text (on checking the registered SNPN with the existing SNPN identity of the CEFR report list) is not applicable when the CEF report list is empty.

We have the following proposals.

- RAN2 agree that no change on the number of PNI-NPN identities in MDT configuration is needed i.e., maximum (12 x 12) PNI-NPN identities (i.e., 12 PLMNs and 12 CAGs per PLMN). E022 is closed.

- RAN2 capture in the RRC spec that plmn-identityList is not included in case of MDT collection in SNPNs. E268 is closed.

- RAN2 agree that similar to the procedural text for CEF in public network UE compares the registered snpn-identity with the snpn- identity logged in VarConnEstFailReport when appending the VarConnEstFailReport to the VarConnEstFailReportlist.

- RAN2 agree to fix the missing correction in [R2-2405968] (changing VarConnEstFailReportlist to VarConnEstFailReport) in the next version of the RRC TS.

- RAN2 agree to the draft CRs in the annex of the paper.

# Annex

3GPP TSG-RAN WG2 Meeting #127	R2-240nnnn

# Maastricht, NL, August 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38331 | CR | <CR#> | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR – Absence of plmn-IndeityList in logged MDT configuration in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the following reply from RAN3 received as part of LS [R3-243892], plmn-IndentityList is not configurable in the case of MDT configuration in a UE in SNPN access mode. However RAN2 RRC specification does not reflect the restriction. RAN3 answer: plmn-IdentityList applies only to public networks and it shall not be used for SNPNs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Filed description of the plmn-IndentityList in MDT configuration is enhanced with an amandment that reflects the above RAN3 answer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC spec remain unclear how the plmn-IndentityList should be configured and used in RRC layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies, the included PNI-NPN identities or the SNPN identities. If areaConfiguration-r17 is present, the UE shall ignore areaConfiguration-r16. The areaConfiguration-v1800 is a non-critical extension of areaConfiguration-r17. See NOTE 1.

earlyMeasIndication If included, the field indicates the UE is allowed to log measurements on early measurement related frequencies in logged measurements.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs. The network does not include this field when the UE is configured with MDT configuration in SNPN access mode.

sigLoggedMeasType If included, the field indicates a signalling based logged measurement configuration (See TS 37.320 [61]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggerd MDT configuration.

End of the change

3GPP TSG-RAN WG2 Meeting #127	R2-240nnnn

# Maastricht, NL, August 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38331 | CR | <CR#> | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR – Miscellaneous corrections on CEF report list in procedural text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedural text for appending the CEF report in the CEF report list when the UE is in SNPN access mode is not aligned with the procedural text of the appending the CEF report in the CEF report list when the UE is in public network access mode. In our understanding, checking the resgitered SNPN identity with the SNPN identity logged in VarConnEstFailureReport is sufficient to guarantee that the UE does not store/append CEF report from different SNPNs in the VarConnEstFailureReportList. This was a logic used in Rel-17 for checking the PLMN idnetities that could be adopted for the SNPN. We also observe that the current procedural text for checking SNPN identity in the VarConnEstFailureReportList, does not consider the case that VarConnEstFailureReport is empty. Therefore we think instead of checking SNPN idnentity with SNPN identity stored in any of the entry in VarConnEstFailureReportList, similar to the public network procedural text, we could simply check whether the registered SNPN identity is included in the VarConnEstFailureReport or not. based on the SONMDT correction CR [R2-2405968] sent to the plenary meeting, the following change (changing VarConnEstFailReportlist to VarConnEstFailReport) was expected to be applied on the new version of the RRC specification version 18.2.0. However, in the new version the change is not applied (apparently it is missed). Therefore, we propose to solve this issue in the next version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedural text is corrected to check the registered SNPN identitiy with the SNPN identity included in VarConnEstFailureReport, instead of VarConnEstFailureReportList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour remains unclear when the VarConnEstFailureReportList is empty and the UE tries to append the CEF report to the list upon checking the registered SNPN indentity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs);

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE supports multiple CEF report:

3>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

3>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport:

4>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

5>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

2>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is not equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

End of the change