3GPP TSG-RAN WG2 #127	R2-2407218

Maastricht, August 19 – 23, 2024

Agenda Item:	8.10.2

Source:	Ericsson

Title:	SON support for MRO

Document for:	Discussion/Decision

# Introduction

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

•	Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

•	Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

This contribution further discusses considerations regarding SON feature enhancements for LTM and CHO with candidate SCGs.

# Discussion

## SON for LTM

In the meeting RAN2#126, the following is agreed.

- If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

- Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

- Log the LTM cell ID upon performing recovery to an LTM candidate cell, details TBD e.g. which field.

- Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

Per the agreement from RAN2#126, we will focus on MCG LTM, and discuss the enhancement for the RLF report, the SHR and the RA report for LTM optimization. Considering that the LTM cell switch procedure highly relies on underlying features such as early DL/UL synchronization and a RACH-less access procedure, we will also discuss the possible MRO enhancements for those aspects.

### Enhancement of RLF-Report

Logging LTM recovery cell ID

The agreement to log the LTM cell ID upon performing recovery to an LTM candidate cell leaves which field to use (e.g. the choCellId or a new field) TBD and also does not mention anything about whether the cell ID should be logged for both successful and failed LTM recovery.

For CHO, the choCellId is set after cell selection, if the selected cell is a CHO candidate cell, i.e. before the actual execution of the CHO recovery attempt. Hence, it is set regardless of the outcome (success or failure) of the CHO recovery attempt.

Notably, logging the recovery cell ID irrespective of the outcome of the recovery procedure in conjunction of presence/absence of the reconnectCellID and re-establishment cell ID provides more information to the network about the cell toward which the UE performed CHO recovery. For example, the network can deduce that there is a downlink/uplink coverage mismatch in the selected cell which was an LTM candidate cell. Therefore, we propose the following.

The UE logs the LTM recovery cell ID when it attempts LTM recovery in an LTM candidate cell (similar to choCellID), regardless of whether the LTM recovery attempt succeeds or fails.

Regarding which field to use for logging of the LTM recovery cell ID, reusing the choCellId field would mean that the UE cannot log and report both a CHO recovery cell ID and an LTM recover cell ID in the same RLF report. But a UE may be configured with both CHO and LTM simultaneously, and this may result in, for example, a scenario where the UE fails CHO execution, selects a CHO candidate target cell and attempts CHO recovery, but fails, then selects an LTM candidate cell and attempts LTM recovery (which may or may not succeed). In this scenario, the UE should log both the CHO recovery cell ID and the LTM recovery cell ID in the RLF report. Hence, we need a new separate field for the LTM recovery cell ID.

Introduce a new field for logging of the LTM recovery cell ID.

Logging time since LTM configuration

Time related information will enable the network to optimize the configuration of the LTM candidate cells in a timely manner, avoiding waste of resources at the network side. Therefore, with an analogy with the timeSinceCHO-Reconfig which represent the time since reception of the CHO configuration until failure, a similar time information field for the LTM procedure can be considered. More specifically, if the time elapsed between reception of the last LTM configuration and the radio link failure is too long, LTM resource is reserved in the candidate cells too early which is a resource waste. On the other hand, if the time is too short, there may not be enough time for L1 measurements, LTM evaluation and LTM execution before the radio link failure. Furthermore, the UE may simultaneously be configured for CHO and LTM configured, which means that reuse of the timeSinceCHO-Reconfig field is precluded, since in case of radio link failure, the time info timeSinceCHO-Reconfig for CHO and the similar time info for LTM, e.g., timeSinceLTM-Reconfig should both be included in the RLF-Report.

Enhance the RLF report with the time elapsed from LTM configuration reception to connection failure (similar to the timeSinceCHO-Reconfig field). A new IE, e.g. timeSinceLTM-Reconfig, is introduced.

Logging beam information

The LTM cell switch command includes the target beam to be used by the UE when executing the LTM cell switch. However, the beam indicated in the LTM cell switch command may be sub-optimal and the LTM cell switch may fail due to a bad choice of target beam, the UE may re-establish/recover or reconnect to the same cell but in a better beam or in a different cell.

Currently the UE logs the target cell ID, the re-established cell ID and, when applicable, the reconnected cell ID in the RLF report. Given that the LTM cell switch relies heavily on the beam information, the beam information can also help the network to optimize the future LTM cell switch procedure, in particular in selecting the optimal beam in the LTM cell switch command.

Enhance the RLF report with beam information.

  - The beam to which the UE failed to perform LTM cell switch,

  - The beam indicated in the last successful LTM cell switch command, when failure occurs after an LTM cell switch

  - The beam the UE attempts to recover/re-establish or reconnect in.

Logging early DL/UL sync information in RLF report

In the LTM procedure, before receiving the cell switch command and upon gNB-DU request using TCI State Activation the UE may acquire DL sync toward the LTM candidate cell(s). The UE may activate and deactivate TCI states of LTM candidate cell(s) for DL sync, as triggered by the gNB-DU.

The UE may also obtain UL sync with LTM candidate cell(s) using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, if triggered by the gNB. Given that these new features (as of now designed for the LTM procedure) play important roles in the performance of LTM (in terms of interruption time) we believe optimization of these features is necessary.

In case of RLF/HOF, logging early DL/UL sync related information in the RLF report enables the network to verify if any optimization is needed for the LTM early DL/UL sync procedures. For example, information on whether and when the early DL/UL sync prior was executed with respect to LTM cell switch execution or connection failure could be a useful information to determine possible optimizations of the early DL/UL sync procedures.

- Indication of which UL sync mechanism was used for the early UL synchronization can be useful, as well as whether and when early DL/UL was performed prior to an LTM cell switch execution or connection failure.

We note that if the estimated TA value turns out to be incorrect (irrespective of UL sync method), e.g. it has become outdated because the UE significantly relocated/moved between the time of the UL sync establishment and the time of the LTM execution, this may lead to failure of RACH-less LTM cell switch. In addition if there is no available estimated TA value (because early UL sync was not configured or was configured but failed) at the time of the LTM execution, the UE will execute a RACH-based LTM cell switch procedure, which is likely to lead to longer interruption time than the RACH-less LTM switch procedure. Hence, in our understanding, the early UL sync procedure and in particular TA estimation validity and accuracy will impact the LTM cell switch performance. In light of the above we propose the following.

Early DL sync related information needs to be logged in the RLF report. Details are FFS.

Early UL sync related information needs to be logged in the RLF report. Details are FFS.

### Enhancement of SHR

Logging L1 measurements in SHR

Similar to the inclusion of L1 measurement results in the RLF report, which was agreed in RAN2 #126, provided that LTM procedure decisions are made based on the L1 measurements, in case of near failure (suboptimal) LTM procedure, the network cannot only rely on L3 measurement in the SHR to verify the neighbouring cells’ status (and the neighbouring beams’ status) for the sake of LTM optimization. In other words, since the network makes decisions based on the L1 measurements, enhancing the SHR with the L1 measurement results would provide a better insight for the network to tune suboptimal LTM configurations.

UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

Successful LTM cell switch to wrong beam

As discussed above, the beam indicated in the LTM cell switch command may be wrong or suboptimal. With the wrong or suboptimal beam, the LTM cell switch can still be successful, but beam failure may occur shortly afterwards. The UE then executes the beam failure recovery procedure towards a better beam in the same cell. To report this scenario by SHR may require a comparably big change, as the current SHR determination is performed upon successful completion of the RA or mobility procedure, the SHR availability indication successHO-InfoAvailable is included in Msg5, whereas the BFR may happen shortly afterwards.

An alternative solution to not change the procedure of SHR reporting is to use a NW-based solution, if BFR happens shortly after the successful LTM cell switch, the DU of the target cell informs the source DU about the LTM cell switch to the wrong beam.

RAN2 to discussion the scenario of successful LTM cell switch to wrong beam i.e. BFD followed by BFR.

## SON features for CHO with candidate SCGs

In RAN2#126 meeting, the following was agreed-

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

In our view, the following RLF scenarios should be taken into account when discussing failures of the CHO with candidate SCG procedure:

- The CHO execution conditions is fulfilled, but the CPC execution condition remains not fulfilled, i.e. RLF declared before CPC execution.

- The CPC execution conditions is fulfilled, but the CHO execution condition remains not fulfilled, , i.e. RLF declared before CHO execution

- HOF during “CHO with candidate SCG” execution, in which case of course both the CHO and CPC execution conditions were fulfilled.

For the case 1 and 2 above, the network needs to be informed regarding the time gap between the CHO (or CPC) event fulfilment and declaration of RLF. For example, if the CHO (or CPC) execution condition is fulfilled at t1 and UE declares RLF at t2, UE needs to include the time difference t2-t1 in the RLF report. Furthermore, it is important for the network to understand if the CHO execution condition or the CPC execution condition was fulfilled. With this two information at hand, i.e. time difference t2-t1, and information of which of CHO or CPC was fulfilled, the network can perform the related optimization. For example, if CHO conditions were fulfilled and the time difference between CHO execution and RLF (i.e. time gap t2-t1) is very large, then this may imply that the CPC should be executed earlier and hence that the CPC-related execution conditions should be adjusted. 
For case 3) instead, it is beneficial if the UE could include the time difference t2-t1 between the CHO (or CPC) execution conditions fulfilment and the CPC (or CHO) execution conditions fulfilment, as well as an indication of the execution conditions that were first fulfilled in time. This is basically the same as for the CHO case specified in Rel.17, where the UE includes time gap between CHO events if an HOF occurs. We also note that this information can be useful also for the SHR case. For example, if an SHR is generated due to the UE meeting the T310 SHR threshold, a large gap between CHO and CPC fulfilment can be useful to analyse the root cause of such suboptimal performances.

If the RLF is detected before the execution of the “CHO with candidate SCG”, the UE includes the following information in the RLF-Report:

  - Indication of which execution condition (CHO or CPC) was fulfilled.

  - The time gap between the fulfilled execution condition (CHO or CPC) and the RLF detection.

If HOF is detected during execution of “CHO with candidate SCG” or SHR is generated, the UE includes the following information in the RLF-Report or SHR:

- Indication of which execution condition was fulfilled first in time.

- The time gap between the fulfilled CHO execution conditions and the fulfilled CPC.

Furthermore, if the UE declares RLF after successfully performing CHO with candidate SCG, it will generate an RLF report. It is important for the network to understand from the RLF report that the last successfully executed HO was a CHO with candidate SCG. Hence, a new last handover type should be included in the RLF report.

RAN2 should agree to define a new lastHO-Type in RLF report for CHO with Candidate SCG.

Furthermore, since the UE performs both PCell handover and PSCell change, it is possible that it may generate both an SHR and an SPR, if configured by the network. In our view, the SHR and SPR are correlated, and network needs to consider this correlation while analyzing the reports.

RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Indication of which UL sync mechanism was used for the early UL synchronization can be useful, as well as whether and when early DL/UL was performed prior to an LTM cell switch execution or connection failure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE logs the LTM recovery cell ID when it attempts LTM recovery in an LTM candidate cell (similar to choCellID), regardless of whether the LTM recovery attempt succeeds or fails.

Proposal 2	Introduce a new field for logging of the LTM recovery cell ID.

Proposal 3	Enhance the RLF report with the time elapsed from LTM configuration reception to connection failure (similar to the timeSinceCHO-Reconfig field). A new IE, e.g. timeSinceLTM-Reconfig, is introduced.

Proposal 4	Enhance the RLF report with beam information.

a.	The beam to which the UE failed to perform LTM cell switch,

b.	The beam indicated in the last successful LTM cell switch command, when failure occurs after an LTM cell switch

c.	The beam the UE attempts to recover/re-establish or reconnect in.

Proposal 5	Early DL sync related information needs to be logged in the RLF report. Details are FFS.

Proposal 6	Early UL sync related information needs to be logged in the RLF report. Details are FFS.

Proposal 7	UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

Proposal 8	RAN2 to discussion the scenario of successful LTM cell switch to wrong beam i.e. BFD followed by BFR.

Proposal 9	If the RLF is detected before the execution of the “CHO with candidate SCG”, the UE includes the following information in the RLF-Report:

a.	Indication of which execution condition (CHO or CPC) was fulfilled.

b.	The time gap between the fulfilled execution condition (CHO or CPC) and the RLF detection.

Proposal 10	If HOF is detected during execution of “CHO with candidate SCG” or SHR is generated, the UE includes the following information in the RLF-Report or SHR:

a.	Indication of which execution condition was fulfilled first in time.

b.	The time gap between the fulfilled CHO execution conditions and the fulfilled CPC.

Proposal 11	RAN2 should agree to define a new lastHO-Type in RLF report for CHO with Candidate SCG.

Proposal 12	RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.