3GPP TSG-RAN WG2 #127	R2-2407219

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.3.2.1

Source:	Ericsson

Title:	Simulation results for RRM measurement temporal prediction

Document for:	Discussion

# 1	Introduction

A new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 with the following objectives [1].

- The evaluation of the AI/ML aided mobility benefits should consider HO perf ormance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

In RAN2#125b meeting, it was agreed to reuse temporal prediction case A and case B studied in RAN1 AI/ML PHY TR and two study goals were agreed for the simulation in this SID [2].

RAN2#125bis Agreements 1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction： RSRP difference to the actual measurement Temporal prediction：RSRP difference to the actual measurement measurement reduction rate as one KPI - As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed …. - R2-2404955, Summay of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO). Conclusion 1: 1st study goal of evaluation is to reduce measurement overhead. Conclusion 2: 2nd study goal of evaluation is to enhance handover performance.

In RAN2#126 meeting, priorities for RRM measurement prediction were agreed to well-proceed the simulation in the next a few meetings [3].

| Case number | Prioritization | Evaluation scenario combination | Target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 2nd goal |

In this contribution, the simulation results for those cases highlighted as high priority are discussed.

# 2	Discussion

## 2.1	Simulation scenarios

The following simulation configurations are considered for the discussion:

- Temporal prediction case B

  - randomly dropped UEs in FR1 UMa scenario

  - ISD 500m

  - Option 3 linear UE trajectory

  - Option 1 wrap-around behavior when UEs reach the boundary of the simulation environment

  - UE moving speed: 30km/h, 60km/h and 120km/h for AI/ML model training and evaluation

  - 7*3 network topology

  - gNBs with single 4x8 panel, 4 SSB beams. No mechanical down tilt

- Temporal prediction case A

  - randomly dropped UEs in FR2 UMi scenario

  - ISD 200m

  - Option 3 linear UE trajectory

  - Option 1 wrap-around behavior when UEs reach the boundary of the simulation environment

  - UE moving speed 30km/h, 60km/h and 120km/h for AI/ML model training and evaluation

  - 7*3 network topology

  - gNBs with single 4x8 panel, 8 SSB beams. No mechanical down tilt

See Annex B for more details.

## 2.2	Model input and output

In RAN2#125bis meeting, the following output-input combinations are considered.

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results.

Given the importance of the cell level measurement predictions for L3 mobility (which is the main focus of the study item) in this contribution we focus on Case 2 and Case 3, wherein the L3 filtered cell-level measurements are directly predicted by AI/ML model. This accordingly reduces the labeling effort, since cell-level result can be used as a unified AI/ML model output for both Case 2 and Case 3.

For AI/ML model input, we use L3 filtered cell-level measurement (Point C according to Figure 1) and L1 beam-level measurement (Point A1 according to Figure 1) for Case 2 and Case 3, respectively. This is because Point C after Layer3 filtering for cell quality and Point A1 after Layer1 filtering can eliminate the impact of fast fading which is beneficial for the model training and inference.

In this contribution, Case 2 and Case 3 are studied for evaluation of cell level measurement predictions.

In this contribution, for Case 2, L3 filtered cell-level measurements is used as model input to predict the L3 filtered cell level measurements.

In this contribution, for Case 3, L1 filtered beam-level results is used as model input to predict the L3 cell-level measurements.

For both Case 2 and Case 3, the cluster approach is used i.e., the number of cells for which their measurements are used as input and the number of cells for which the time domain predictions are inferred as output are set to 3 during the training and inference.

In this contribution, for evaluation of Case 2 and Case 3, the cluster approach is used. The number of cells in the cluster is 3 cells.

Besides, how to perform layer1 filtering has impact on the prediction although it’s not standardized (currently, it is a UE implementation). In this contribution, we use the already existing L3 filtering formula in 38.331 section 5.5.3.2 with specific alpha 0.2, i.e., Fn = (1 – a)*Fn-1 + a*Mn, where the parameters are defined in the mentioned TS.

Figure 1.  Measurement model illustrated in TS38.300

For layer1 filtering, the existing L3 filtering mentioned in 38.331 section 5.5.3.2 with specific alpha 20% is reused.

## 2.3	Evaluation methodology

Given that the first goal of the study is to study the RRM measurements for the sake of reducing the measurement overhead, in the following we first present the evaluation of the AIML performance targeting to reduce the measurement overhead. Later we present the evaluation of the AIML performance targeting to enhance the handover performance.

In the following, we refer to Case B from RAN1 AI/ML PHY TR 38.843, as the measurement to prediction pattern used to reduce the measurement overhead. In addition, we use Case A from RAN1 AI/ML PHY TR 38.843, as the measurement to prediction pattern used to enhance the mobility performance.

In general, we evaluate the following cases:

- Case 2 – Case B: prediction cell level measurements from cell level measurements targeting to reduce measurement overhead

- Case 3- Case B: prediction cell level measurements from beam level measurements targeting to reduce measurement overhead

- Case 2- Case A: prediction cell level measurements from cell level measurements targeting to mobility performance improvement

- Case 3- Case A: prediction cell level measurements from beam level measurements targeting to mobility performance improvement

### 2.3.1 	First goal: Prediction to reduce measurement overhead (Case B)

Temporal prediction case B aims to reduce measurement overhead while a minimum prediction accuracy should be met. To calculate the measurement overhead reduction rate, observation window and prediction window are defined. There will be one or multiple measurements samples in observation window taken as model input at training and inference, and there will be one or multiple skipped measurements in the prediction window taken as the label/output. The reduced measurement overhead using time domain prediction is calculated by

num of skipped measurementsnum of sampled measurement+num of skipped measurement

Temporal prediction case B is simulated in FR1 UMa scenarios. The RRM measurement periodicity is 200ms and period for beam-level measurement is 40ms. There are cell-level measurements at each time instance numbered with {T0, T0-1, T0-2…} as shown in Figure 2. We fix the prediction step to 200ms which is one period of L3 RRM measurement, and we only predict the cell-level result for one single future time instance (which is T0 (current time) + 200ms).

Cell level measurement prediction based on cell level measurements (Case 2)

As mentioned above in case 2, we use cell level measurements for both model input and the label/output. Specifically, we use cell level measurements sampled every 200ms in observation window to predict the cell-level measurements which is 200ms later from the end of the observation window, namely: current time + 200ms. For example, as the 50% measurement reduction rate case illustrated in Figure 2, we use 1 cell-level RSRP measurement sampled at time instance T0 (current time) as model input to predict cell-level results at time instance T0+1 which is 200ms later from time instance T0. As the 33.33% measurement reduction rate case illustrated in Figure 2, model input is 2 cell-level RSRP measurements sampled at time instance T0 and T0-1 respectively, and model output is cell-level results at time instance T0+1 which is 200ms later from time instance T0 and 400ms later than the time instance T0-1.

Figure 2.  L3 filtered cell level measurement predictions from L3 filtered cell level measurement results; Case 2 and Case B. Cell level measurement sampling rate is 200 [ms]. Prediction window is 200 [ms]. Observation window is varying from 200 [ms] to 600 [ms].

We consider the number of sampled cell-level results in observation window and one skipped cell-level results in prediction window when we calculate the measurement overhead reduction. By increasing the observation window length to sample more cell-level results while fixing the prediction window and prediction step, we evaluate the prediction accuracy with different measurement overhead reduction rates.

We evaluated the RSRP difference (prediction error compared to the ground through information) for various scenarios targeting to reduce the measurement overhead in the range of 10%~50%. The evaluation results are summarized in Table .1.

Table 1. Case 2 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results. RSRP difference values are in dB.

| Measurement overhead reducion rate |  | 50% | 33.33% | 25% | 20% | 16.67% | 10% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE moving speed | 30km/h | 0.653 | 0.415 | 0.355 | 0.324 | 0.347 | 0.349 |
|  | 120km/h | 1.283 | 0.804 | 0.726 | 0.696 | 0.722 | 0.733 |

Table 1 summaries the AIML evaluations for the scenario Case 2 – Case B. As shown, in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (so-called prediction error) of less than 1 dB while reducing the measurement overheads up to 50%. The RSRP difference is slightly higher in high mobility scenarios (120km/h) – almost two times higher compared to the RSRP difference evaluated in low mobility scenarios. However, our evaluation shows that in high mobility scenarios the AIML model can still maintain a very high accuracy (less than or close to 1 dB RSRP difference) while the measurement reduction rate is up to 50%.

For Case 2-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%.

For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 33%.

For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is just slightly above 1 dB when the measurement reduction rate is 50%.

Cell level measurement prediction based on beam level measurements – (Case 3)

As mentioned above in Case 3, we use beam-level measurements for model input and cell-level results for the label/output. Specifically, we use beam-level results sampled every 40ms in observation window to predict the cell-level measurements which is 200ms later from the end of the observation window (i.e., T0 (current time) + 200ms).

For example, as the 83.3% measurement reduction rate case illustrated in Figure 3, we use 1 beam-level measurements as model input within the observation window. Same as case 2, we predict cell-level results at time instance T0+1 which is 200ms later from time instance T0. As the 50% measurement reduction rate case illustrated in Figure 3, we use 5 beam-level measurements as model input where those 5 beam-level results are sampled every 40ms within the observation window. Same as case 2, we predict cell-level results at time instance T0+1 which is 200ms later from time instance T0.

Difference from case 2 is, we considered the number of sampled beam-level results in observation window and skipped beam-level results in the prediction window when we calculate the measurement overhead reduction rate. By increasing the observation window length to sample more beam-level results while fixing the prediction window and prediction step, we simulate the prediction accuracy with different measurement overhead reduction rate. The simulation results are provided in the following.

Figure 3.  L3 filtered cell level measurement predictions from L1 filtered beam level measurement results; Case 3 and Case B. Cell level measurement sampling rate is 200 [ms] and beam level measurement sampling rate is 40 [ms]. Prediction window is 200 [ms].

We evaluate the RSRP difference (so called prediction error) with 10%~83.3% measurement overhead reduction rate. Simulation results is summarized in Table .2.

Table 2. Case 3 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results.

| Measurement overhead reduction rate |  | 83.3% | 71.4% | 62.5% | 50% | 33.33% | 25% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE moving speed | 30km/h | 1.139 | 0.941 | 0.807 | 0.544 | 0.419 | 0.325 |
|  | 120km/h | 1.914 | 1.751 | 1.527 | 1.089 | 0.78 | 0.536 |

Table 2 summaries the AIML evaluations for the scenario Case 3 – Case B. As shown, the RSRP difference (prediction error) shows a higher gain (less RSRP difference) when the observation window is large. In other words, the larger observation window is, the more accurate prediction is. However there might be a saturation point at which adding more historical measurement samples may not assist to achieve a better gain.

In general, we observe that similar to Case 2, in Case 3 and in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (so-called prediction error) of less than 0.5 dB when the reduction rate is up to 50% and in case the reduction rate increases to 83%, the RSRP difference is still around 1 dB. Similarly, the RSRP difference is slightly higher in high mobility scenarios (120km/h) – almost two times higher compared to the RSRP difference evaluated in low mobility scenarios. In addition, our evaluation shows that in high mobility scenarios the AIML model can still maintain good accuracy (less than or close to 1 dB RSRP difference) while the measurement reduction rate is up to 50%. The results shows that the RSRP difference degrades only one more dB in scenarios wherein the measurement rate is reduced up to 83%.

For Case 3-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%. In case the reduction rate increases to 83%, the RSRP difference is still around 1 dB.

For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with a good accuracy, i.e., RSRP difference is less than or around 1 dB while reducing the measurement overheads up to 50%.

For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is still below 2 dB when the measurement reduction rate is 83%.

We also observed that involving more beam-level results in time domain is slightly helpful for model to predict accurately when we compare column with measurement overhead reduction rate “50%”, “25%” and “10%” in table 1 and table 2.

Simulation results show that adding beam level measurements to the input (for the cases of 50%, 33% and 25%) better accuracy is achievable in Case 3 – Case B compared to the Case 2 - Case B wherein only cell level measurement is considered as input to the model.

### 2.3.2	Second goal: Prediction to improve handover performance (Case A)

Temporal prediction with measurement to prediction pattern denoted as Case A aims to improve handover performance. Different from the temporal prediction Case B, we fixed the number of measurements to 10 in observation window to evaluate the prediction accuracy with different length of prediction window. The simulation is conducted in FR2 and using UMi model with the RRM measurement periodicity set to 480ms.

Cell level measurement prediction based on cell level measurements (Case 2)

As mentioned above in Case 2, we use cell-level measurement results for both model input and the output/label. Specifically, we use cell-level results sampled every 480ms at each time instance depicted with {T0-2, T0-1 , T0} in Figure 4. As shown the observation window fixed to 10x480ms to predict the cell-level results which is (N x 480 ms) after the end of the observation window T0 wherein N  {1, 2, 4, 8, 12}. For example, as the case of prediction step 0.48s illustrated in Figure 4, we use 10 cell-level RSRP measurement sampled at time instance T0~T0-9 as model input to predict cell-level measurements at time instance T0+1 which is 480ms after time instance T0. As the case of prediction step 0.96s illustrated in Figure 4, we predict cell-level results at time instance T0+2 by using 10 cell-level RSRP measurement sampled at time instance T0~T0-9 as model input.

Figure 4.  L3 filtered cell level measurement predictions from L3 filtered cell level measurement results; Case 2 and Case A. Cell level measurement sampling rate is 480 [ms]. Prediction window is N x 480 [ms] wherein prediction window step size N  {1, 2, 4, 8, 12}.

We evaluate the prediction accuracy (measured as RSRP difference) with different prediction window step size N leading to prediction window size between 0.48s to 5.76s. The evaluation results are provided In the following.

Table 3. Case 2 – Case A: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results.

|  | RSRP difference (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction window size | 0.48s | 0.96s | 1.92s | 3.84s | 4.80s | 5.76s |
| UE moving speed 60km/h | 0.361 | 0.391 | 0.444 | 0.642 | 0.693 | 1.04 |
| UE moving speed 120km/h | 0.431 | 0.533 | 0.577 | 0.799 | 1.109 | 1.16 |

The evaluation shows that in a scenario with UE mobility set to 60 km/h the AIML model can predict the RSRP value with up to 1 [dB] difference compared to the ground truth value when prediction window size is less than 5.76 seconds. In a higher mobility scenario (i.e., when UE speed is 120 km/h) the RSRP difference is slightly higher and increasing by increasing by increasing the size of the prediction window. However, the results show still good accuracy (slightly above 1 dB) when the prediction window size is 4.8 seconds. Therefore, we have the following observations:

Maximum prediction step is 5.76s for UE moving speed 60km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Maximum prediction step is 4.80s for UE moving speed 120km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Cell level measurement prediction based on beam level measurements (Case 3)

As mentioned above in Case 3, we use beam-level measurements for model input and cell-level measurements is the label/output. Specifically, we use beam-level results sampled every 20ms in observation window fixed to 10*20ms to predict the cell-level measurements which is 480ms later from the end of the observation window (i.e., T0 (current time) + 480ms). For example, as the case of prediction step 0.48s illustrated in Figure 5, we use 10 beam-level RSRP measurements sampled in the observation window as model input to predict cell-level results at time instance T0+1 which is 480ms later from time instance T0. As the case of prediction step 0.96s illustrated in Figure 5, we use 10 beam-level RSRP measurements sampled in the observation window model input to predict cell-level results at time instance T0+2 which is 0.96 s later from time instance T0.

Figure 5.  L3 filtered cell level measurement predictions from L1 filtered cell level measurement results; Case 3 and Case A. beam level measurement sampling rate is 20 [ms]. Prediction window is N x 480 [ms] wherein prediction window step size N  {1, 2, 4, 8, 12}.

We simulate the prediction accuracy with different length of prediction step. The simulation results are provided in the following. For temporal prediction case A – Case 3 we evaluated the RSRP difference with prediction step 0.48s~1.92s. Simulation results is summarized in Table 4.

Table 4. Case 3 - Case A: L3 filtered cell level measurement predictions from L1 filtered cell level measurement results; Case 3 and Case A. beam level measurement sampling rate is 20 [ms]. Prediction window is N x 480 [ms] wherein prediction window step size N  {1, 2, 4, 8, 10, 12}.

|  |  | RSRP difference (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Prediction step |  | 0.48s | 0.96s | 1.92s | 3.84s | 4.80s | 5.76s |
| UE moving speed 60km/h | AIML, 8 beams as input | 1.77 | 2.60 | 3.34 | 3.84 | 4.34 | 4.58 |
| UE moving speed 120km/h | AIML, 8 beams as input | 3.19 | 3.67 | 4.11 | 4.53 | 4.91 | 5.28 |

Unlike the good prediction performance of Case 2- Case A, in scenario of Case 3 – Case A, the RSRP differences are above 1 dB for all prediction steps.

Some remarks:

Given that the results in case of Case 3 – Case A shows a substantial performance degradation compared to the results in Case 2- Case A, some remarks are in order.

For example, we believe RAN2 should study which set of beams should be included as input to the model for the Case 3 – Case A. In our simulation, we used the measurement results of the 8 beams of the cells as input to the model, while the actual cell level RSRPs are derived by linear combination of the 3 best beams of each cell (i.e., 5 beams are filtered as part of beam consolidation procedure). This might have been a reason to get such degradation in the model performance in that case.

In addition, beams are spatially different (transmitted to non-orthogonal directions) and such phenomenon might create uncorrelated behaviour among the input that finally affects the performance of the AIML model. Therefore, we propose the following.

RAN2 investigate the impact of the set of beams used as input in the Case 3. i.e., which set of beams to use as input to the model.

- Finally, we would like to request RAN2 to consider the results in the Annex to be included in the TR of the study item.

RAN2 include the results in the Annex A of the paper in the TR of the study item.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	In this contribution, Case 2 and Case 3 are studied for evaluation of cell level measurement predictions.

Observation 2	In this contribution, for Case 2, L3 filtered cell-level measurements is used as model input to predict the L3 filtered cell level measurements.

Observation 3	In this contribution, for Case 3, L1 filtered beam-level results is used as model input to predict the L3 cell-level measurements.

Observation 4	In this contribution, for evaluation of Case 2 and Case 3, the cluster approach is used. The number of cells in the cluster is 3 cells.

Observation 5	For layer1 filtering, the existing L3 filtering mentioned in 38.331 section 5.5.3.2 with specific alpha 20% is reused.

Observation 6	For Case 2-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%.

Observation 7	For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 33%.

Observation 8	For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is just slightly above 1 dB when the measurement reduction rate is 50%.

Observation 9	For Case 3-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%. In case the reduction rate increases to 83%, the RSRP difference is still around 1 dB.

Observation 10	For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with a good accuracy, i.e., RSRP difference is less than or around 1 dB while reducing the measurement overheads up to 50%.

Observation 11	For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is still below 2 dB when the measurement reduction rate is 83%.

Observation 12	Simulation results show that adding beam level measurements to the input (for the cases of 50%, 33% and 25%) better accuracy is achievable in Case 3 – Case B compared to the Case 2 - Case B wherein only cell level measurement is considered as input to the model.

Observation 13	Maximum prediction step is 5.76s for UE moving speed 60km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 14	Maximum prediction step is 4.80s for UE moving speed 120km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 15	Unlike the good prediction performance of Case 2- Case A, in scenario of Case 3 – Case A, the RSRP differences are above 1 dB for all prediction steps.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 investigate the impact of the set of beams used as input in the Case 3. i.e., which set of beams to use as input to the model.

Proposal 2	RAN2 include the results in the Annex A of the paper in the TR of the study item.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2404955, Summay of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO).

RAN2 Chair Note, RAN2#126

# Annex A – Simulation results and observations for TR

Table 1. Case 2 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results.

| Measurement overhead reducion rate |  | 50% | 33.33% | 25% | 20% | 16.67% | 10% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE moving speed | 30km/h | 0.653 | 0.415 | 0.355 | 0.324 | 0.347 | 0.349 |
|  | 120km/h | 1.283 | 0.804 | 0.726 | 0.696 | 0.722 | 0.733 |

- For Case 2-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%.

For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 33%.

For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is just slightly above 1 dB when the measurement reduction rate is 50%.

Table 2. Case 3 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results.

| Measurement overhead reduction rate |  | 83.3% | 71.4% | 62.5% | 50% | 33.33% | 25% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE moving speed | 30km/h | 1.139 | 0.941 | 0.807 | 0.544 | 0.419 | 0.325 |
|  | 120km/h | 1.914 | 1.751 | 1.527 | 1.089 | 0.78 | 0.536 |

For Case 3-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 50%. In case the reduction rate increases to 83%, the RSRP difference is still around 1 dB.

For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with a good accuracy, i.e., RSRP difference is less than or around 1 dB while reducing the measurement overheads up to 50%.

For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) RSRP difference (prediction error) is still below 2 dB when the measurement reduction rate is 83%.

Table 3. Case 2 – Case A: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results.

|  | RSRP difference (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction window size | 0.48s | 0.96s | 1.92s | 3.84s | 4.80s | 5.76s |
| UE moving speed 60km/h | 0.361 | 0.391 | 0.444 | 0.642 | 0.693 | 1.04 |
| UE moving speed 120km/h | 0.431 | 0.533 | 0.577 | 0.799 | 1.109 | 1.16 |

Maximum prediction step is 5.76s for UE moving speed 60km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Maximum prediction step is 4.80s for UE moving speed 120km/h if we consider 1 dB as acceptable threshold for RSRP difference.

# Annex B – Simulation settings

| Report parameters |  | Temporal prediction case B |  | Temporal prediction case A |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 2 | Case 3 | Case 2 | Case3 |  |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |  |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 |  |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 30Km/h, 120Km/h |  | 60Km/h, 120Km/h |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |  |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Not involved |  |  |  |  |
|  | Whether LOSsoft is modeled or not | Modelled |  |  |  |  |
|  | spatial consistency option (A or B) | Option B |  |  |  |  |
|  | Number of TX beams | 4 for FR1 UMi, |  | 8 for FR2 UMi |  |  |
|  | Number of RX beams | 1 | 1 | 1 |  | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50%, 33.33%, 25%, 20%, 16.67%, 10% | 83.3%, 71.4%, 62.5%, 50%, 33.33%, 25% | - |  | - |
|  | Observation window(Note3) | 0.2s, 0.4s, 0.6s, 0.8s, 1.0s, 1.8s | 0.04s, 0.08s, 0.12s, 0.2s, 0.4s, 0.6s, | 4.8s |  | 0.2s |
|  | Prediction window (Note3) | 0.2s | 0.2s | 0.48s, 0.96s, 1.92s, 3.84s, 4.80s, 5.76s |  | 0.48s, 0.96s, 1.92s, 3.84s, 4.80s, 5.76s |
|  | Any other parameters (Note 4) |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | ~43000 |  |  |  |  |
|  | Testing | ~10000 |  |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level results | Beam-level results | Cell-level results |  |  |
|  | Model output(Note 6) | Cell-level results |  |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | Convolution, LSTM |  |  |  |  |
|  | Model complexity in a number of parameters (M) | 1.343M | 1.338M | 1.381M | 1.387M |  |
|  | Model complexity in model size (e.g. Mbyte) | 5.12M | 5.10M | 5.267M | 5.291M |  |
|  | Computational complexity [FLOPs] | 8.03M | 12.05M | 40.17M | 40.23M |  |
| Metrics | Average L3 cell level RSRP difference (dB) |  |  |  |  |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) |  |  |  |  |  |