3GPP TSG-RAN WG2 Meeting #127	R2-2407220

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.2.4

Source:	Nokia

Title:	Random access for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions

Document for:	Discussion and Decision

# 1	Introduction

In RAN#103, the study item “Study on solutions for Ambient IoT (Internet of Things) in NR” is revised in RP-240826. The random access procedure is listed as one of the RAN2-led functionalities that are needed to be studied to enable DO-DTT and DT data transmission for A-IoT. To facilitate studying the access procedures, the following agreements were made during the RAN2#125bis meeting:

Agreement

1	RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

2	We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

3	Random Access is triggered by the reader

4	Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

5	Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

6	Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

7	For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

The overall objective of the SI is to study a harmonized air interface design for all device types (defined as devices 1, 2a, and 2b in R1-2401767) and for all topologies (described in details in R1-2403663) according to the below agreements:

Agreements

1	Unless explicitly stated all agreements apply to all device types and for both topologies.

2	From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

To define the procedures of contention-based 3-step and 2-step access RAN2 agreed to the following during RAN2#126 meeting:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

Agreements on 2 step CB RA

1   A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

# 2	Discussion

## 2.1	Structure for A-IoT Random Access Procedure

AIoT utilises random access to enable communication with a large number of potentially unknown devices within a single “inventory” and/or “command”. RAN2 has already agreed general steps for these procedures in terms of 1-4 step random access although some steps are subject of further debate. Nevertheless, an AIoT device must be provided multiple resources by the reader in order to complete the access procedure in a single occasion.

Observation 1: An access occasion is represented by the set of resources based on which a device can execute “inventory” and/or “command” requests.

We can assume an “inventory” or “inventory and command” process comprises data communication between the reader and multiple of AIoT devices triggered within the reader by a request by the CN/AIoT access function. The random access procedure within the AIoT device starts when an AIoT paging message is received.

During the AIoT device response phase, the reader may issue update of access related information i.e. through the paging message, or other means. As of this, it is possible to dynamically configure the duration of the AIoT response phase, thus adapting it to the number of devices. It may also be possible to define different access restrictions e.g. using an allow list, power information, or as to control the number of device access’ between paging updated within a response phase.

Proposal 1: The information related random access for AIoT devices can be provided during AIoT paging and / or updated during the AIoT device response phase (e.g., by using subsequent / repeated paging messages). FFS details.

Proposal 2: The information related to access may consist of resources, time, device ID(s), or other information.

When the reader is not aware of the exact IDs and/or number of IDs of the devices to which it can communicate, and try to collect ‘blind’ information/inventory, it is important to answer the following questions while determining the content of each step of the RA procedure:

- how long should the AIoT response phase last

- whether / when to update the access related parameters

Hence, it is important to ensure that the reader provides the A-IoT device with information to enable the determination of a response phase i.e. through an identifier to serve the purpose to control the RA attempts by each device such that only a single successful RA attempt is required from each device during a session.

The procedure identification may be used to ensure that any device successfully accessing network within a particular procedure knows that it has accessed network for this inventory and/or command procedure and does not need to attempt access even if it receives another access indication signal from the reader in subsequent access rounds.

Proposal 3: RAN2 to study which information needs to be available to the AIoT access function and the reader in order to configure the access related parameters correctly (e.g. CW duration, backoff parameters, total time to respond, expected number of devices).

## 2.2	Contention-free Access

In some scenarios, the reader may be indicated (through a service request) to trigger only a single specific device. In such case, the device may perform a contention-free access procedure without colliding with the access transmission of other devices. Hence, no need to go through the steps of contention-resolution. When multiple or all devices are targeted in the initial trigger message, it may not be reasonable to use a contention-free access procedure due to scalability issues with the massive deployment of users which is expected to be the case with the A-IoT devices.

Furthermore, using the contention-free procedure when multiple devices are triggered may lead to an inefficient resource utilization (if reserved resources are not used, if the traffic is sporadic, or even if the user density is low) and increase the complexity issues. Prioritizing some devices to access the network in a contention-free manner may lead to fairness issues as well.

Observation 2: The usage of contention-free access for multiple or all devices simultaneously may lead to resource inefficiency, scalability and fairness issues as well as may increase the network and device complexity.

Proposal 4: RAN2 to decide whether communication with a single device is contention-free by default.

Figure 2 shows an example of a 1-step like contention-free access procedure. In particular, the reader sends a trigger message including the single device ID to request the target device to start the access procedure. The device whose ID matches the ID sent in trigger message responds to the reader by sending Msg1. The reader’s response to Msg1 could be simple acknowledgement of receiving Msg1 but device ID could be sent as well. Note that the reader may use the device ID to determine whether the device is present in its connectivity region or not. The 1-step like contention-free access procedure can be described as follows:

- Msg0: the reader triggers a single A-IoT device to start the access procedure. This message may include the ID of the target device and/or control information to enable the device completing the access procedure.

- Msg1: the very first D2R transmission consists of the data transmission. This message is transmitted using the contention-free access resources indicated by the reader in Msg0.

  - whether this message can also include the device ID and whether it can be followed by a reader’s response that includes the reader control information for the next D2R transmission and/or the updated control information for the next access occasions/rounds are FFS.

Proposal 5: For a single-device access, one-step access procedure should be studied with the trigger message containing the target device ID.

Proposal 6: For the contention-free access, the first D2R transmission Msg1 consists of the data transmission and is transmitted using the contention-free access resource indicated by the reader in Msg0.

Proposal 7: For contention-free access, there is no need for a reply from the reader to msg1.

Figure 3: An illustration of 1-step contention-free access procedure.

It is possible to use the same structure for contention-free random access as well, where devices in each round are assigned particular RA occasions for network access.

When the network has complete information about identification (IDs) of all the devices in coverage area of a reader, designing rounds and RA occasions is trivial as network can divide IDs in small groups/rounds and in each round number consists of RA occasions equals the number of devices in the group. The devices can use contention-free or contention-based RA as configured.

## 2.3	A-IoT Random Access for Large Number of Devices

The A-IoT devices will be large in number with small transmission requirements, thus, random access procedure should be as simple as possible with minimum signalling overhead for initial access. With no RRC state definition for A-IoT devices, the connectionless access should be studied where initial access is performed to allow for transmission of limited data in a short period of time and the procedure terminates itself when data transmission completes after one or more transmissions.

With the dense deployment of A-IoT devices and the expected high network load, it is quite essential to study the restriction of A-IoT access based on the network and A-IoT device-related factors such as stored energy availability in A-IoT devices. Studying the A-IoT access barring not only reduces the number of connection request failures and congestion levels but also to reduce unnecessary transmission on both the network and device sides.

Proposal 8: RAN2 to study necessary enhancements such as access barring to the existing access mechanism to support the functionality of random access for A-IoT devices.

Another consideration that needs to be studied is the multiplexing of multiple A-IoT devices within one access procedure to meet the expected device density that can reach orders of magnitude higher than the existing 3GPP IoT technologies. During the RAN1#116-bis meeting, RAN1 has agreed to:

Agreement

Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS.

Agreement

Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

By enabling multiple A-IoT devices to access the cell simultaneously through FDM and/or TDM, the collision probability during the access procedure can be substantially decreased. RAN2 is recommended to consider using FDM and/or TDM along with other multiplexing methods including the use of orthogonal preambles and spatial multiplexing techniques for Msg1 in the contention-based access procedure (whether it is 2-step or 4-step like).

Proposal 9: RAN2 to study multiplexing multiple Msg1 from multiple A-IoT devices within one access occasion using FDM and/or TDM as baseline. FFS for other multiplexing techniques.

In case the FDM is supported for multiple D2R transmissions of Msg1, the reader may be able to successfully receive multiple Msg1 from multiple devices at the same time slot. In such scenarios, it is essential to enable the reader to respond to multiple devices at the same time by enabling the frequency-domain multiplexing of its R2D responses to these devices (i.e., Msg2).

Proposal 10: When FDM is supported for Msg1, RAN2 to study multiplexing multiple Msg2 from the reader to multiple A-IoT devices within one time slot using FDM as baseline. FFS other multiplexing techniques.

## 2.6	A-IoT Device Availability

To ensure the availability of the A-IoT device during the transmission and reception procedures, the following agreement was made during the RAN#103 meeting in RP-240854 (Section 8):

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1]

  - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds

    - Note: this value can be revisited in future RAN plenary meetings, if necessary

In general, an A-IoT device may experience failure during the random-access procedure due to their need to harvest energy between transmissions. In particular, A-IoT devices may not harvest enough energy between two successive transmissions of the D2R access messages, or before the start and the completion of access (whether contention-based or contention-free) procedures. Not having enough energy for completing the access procedure may result in unexpected behaviour and can cause radio link failures.

In addition, A-IoT devices (e.g., device 1 and device 2a) may not have the capability to send their D2R transmissions with constant transmission power which means that the maximum transmission power they could achieve at any given moment could vary. Backscattering the D2R signals with variable transmission power may negatively affect the access procedure for A-IoT devices. For example, A-IoT devices may not have sufficient power to backscatter Msg1 with successive increases in power. Failure to complete a successful access procedure not only impacts the network performance but also creates unnecessary collisions and spectral inefficiency. It may also lead to excessive drain of stored energy at A-IoT devices.

Proposal 11: RAN2 to study how A-IoT devices can successfully complete the access procedure under the constraint of device energy availability.

In the cellular network, A-IoT devices will be distributed throughout the network area. Some A-IoT devices will be far from the reader, while others will be closer. For A-IoT devices (e.g., device 1 and device 2a) that are far away (e.g., at the cell edge in Topology 1), to backscatter a signal that can be successfully received and decoded by the reader, the A-IoT device needs to be provided with a carrier wave that is transmitted at high power levels.

Activating A-IoT devices with high-power levels may create severe interference and may also cause excessive energy waste for near A-IoT devices. For this reason, farther A-IoT devices would have to harvest more energy to transmit a signal that is sufficient for the distant reader to successfully receive the A-IoT backscattering.

Proposal 12: RAN2 to consider a measure of proximity (e.g., expressed as signal strength) between an A-IoT device and the reader and/or CW node when studying A-IoT access procedure.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: An access occasion is represented by the set of resources based on which a device can execute “inventory” and/or “command” requests.

Observation 2: The usage of contention-free access for multiple or all devices simultaneously may lead to resource inefficiency, scalability and fairness issues as well as may increase the network and device complexity.

Proposal 1: The information related random access for AIoT devices can be provided during AIoT paging and / or updated during the AIoT device response phase (e.g., by using subsequent / repeated paging messages). FFS details.

Proposal 2: The information related to access may consist of resources, time, device ID(s), or other information.

Proposal 3: RAN2 to study which information needs to be available to the AIoT access function and the reader in order to configure the access related parameters correctly (e.g. CW duration, backoff parameters, total time to respond, expected number of devices).

Proposal 4: RAN2 to decide whether communication with a single device is contention-free by default.

Proposal 5: For a single-device access, one-step access procedure should be studied with the trigger message containing the target device ID.

Proposal 6: For the contention-free access, the first D2R transmission Msg1 consists of the data transmission and is transmitted using the contention-free access resource indicated by the reader in Msg0.

Proposal 7: For contention-free access, there is no need for a reply from the reader to msg1.

Proposal 8: RAN2 to study necessary enhancements such as access barring to the existing access mechanism to support the functionality of random access for A-IoT devices.

Proposal 9: RAN2 to study multiplexing multiple Msg1 from multiple A-IoT devices within one access occasion using FDM and/or TDM as baseline. FFS for other multiplexing techniques.

Proposal 10: When FDM is supported for Msg1, RAN2 to study multiplexing multiple Msg2 from the reader to multiple A-IoT devices within one time slot using FDM as baseline. FFS other multiplexing techniques.

Proposal 11: RAN2 to study how A-IoT devices can successfully complete the access procedure under the constraint of device energy availability.

Proposal 12: RAN2 to consider a measure of proximity (e.g., expressed as signal strength) between an A-IoT device and the reader and/or CW node when studying A-IoT access procedure.