3GPP TSG-RAN WG2 Meeting #127	R2-2407225

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	8.7.2

Source: 	MediaTek Inc.

Title: 	Discussion on multi-modality

Document for:	Discussion and Decision

# Introduction

This document discusses the enhancement directions of first objective in R19 XR WID [1]

-	Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

NOTE:	Check in RAN#105 (check also other WG involvement if needed).

# Discussion

## DL multi-modality awareness at RAN

In Rel-18, SA2 had already discussed multi-modality issues extensively and comprehensively. The results are captured in TS 23.501 [2], clause 5.37.2, as shown below:

-- TS 23.501, clause 5.37.2 --

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The

PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data

flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF. The PCF generates the authorized QoS Monitoring policy for each data flow.

--

Observation 1: SA2 already discussed multi-modal issue and conclude to introduce Multi-model Service ID in TS 23.501.

For synchronization issue between the multi-modal service flows, there are two fields “Timestamp” and “SSRC” in the RTP header. gNB just need to check the “Timestamp” and “SSRC” field, then, we believe that these two information can be used by gNB to address synchronization issue.

- Timestamp: 32 bits, the timestamp reflects the sampling instant of the first octet in the RTP data packet. The sampling instant must be derived from a clock that increments monotonically and linearly in time to allow synchronization and jitter calculations.

- SSRC: 32 bits, identifies the synchronization source.

Observation 2: In RTP header, the Timestamp and SSRC information can be used for synchronization and jitter purposes.

Based on observation 1 and 2, gNB is able to know the multi-modal information (e.g., Multi-modal Service ID) and timing information and synchronization source. gNB is also aware of synchronization thresholds for immersive multi-modality VR applications according to TS 22.261 [3], clause 6.43, as listed below.

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

Because DL transmission is totally controlled and up to gNB implementation, with above information, gNB is able to achieve DL synchronized transmission for multi-modal traffic.

Proposal 1: No additional RAN mechanism is needed for gNB to achieve DL synchronized transmission for multi-modal traffic.

## UL multi-modality awareness at RAN

For UL, the multi-modal information is only aware by application on top of UE, gNB is not aware of it. Therefore, some mechanism are needed to assist gNB to know these information, then, gNB can establish UL QoS flow and DRB mapping and connection according to these information. RAN2 already sent a LS [4] to SA2 to ask for what kinds of existing information can be provided from SA2. RAN2 just wait reply LS from SA2.

Proposal 2: RAN2 to wait for reply LS from SA2 to identify any mechanism/information can be defined for UL multi-modality awareness in RAN.

# Conclusion

In this contribution, we have discussed DL and UL multi-modal traffic, respectively. In summary, we have made the following observation and proposals:

Observation 1: SA2 already discussed multi-modal issue and conclude to introduce Multi-model Service ID in TS 23.501.

Observation 2: In RTP header, the Timestamp and SSRC information can be used for synchronization and jitter purposes.

Proposal 1: No additional RAN mechanism is needed for gNB to achieve DL synchronized transmission for multi-modal traffic.

Proposal 2: RAN2 to wait for reply LS from SA2 to identify any mechanism/information can be defined for UL multi-modality awareness in RAN.

# Reference

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3.

- 3GPP TS 23.501.

- 3GPP TS 22.261.

- R2-2405782, LS on multi-modality awareness at RAN.