3GPP TSG-RAN WG2 #127	R2-2407228

# Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	 7.2.1

Source:	Ericsson

Title:	draft LS reply on synchronization source change at the transmitting anchor UE in SL positioning

Document for:	Discussion

# 1	Introduction

RAN4 in LS “R4-2410352” is asking RAN2 whether RAN2 has introduced or is working on any solutions to inform an SL UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement.

In this paper, we discuss and draft a reply LS to RAN4.

# 2	Discussion

For SL Positioning in Rel-18, RAN2 as such has relied upon SL communication synchronization procedure and has not done any additional enhancements or discussed change of synch source during SL-PRS transmission. It is correct that if Tx UE synchronization source changes during SL-PRS positioning, the ranging/localization estimation may suffer or even the procedure may fail if it takes longer duration to select a new synch source.

However, in Rel-18, RAN2 has not looked into solutions for the issue due to the change of synch source during SL-PRS transmission.

The SL synchronization solution is very rudimentary in Rel-18 and because of lack of time, there has been no enhancements made during Rel-18.

RAN2 to reply that “RAN2 has neither introduced nor working on any solutions to inform a UE performing SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement “.

# 3	draft Reply

3GPP TSG-RAN WG2 Meeting #127 	  R2-24xxxxxx

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title:	[draft] reply LS on synchronization source change at the transmitting anchor UE in SL positioning

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	3GPP TSG-RAN WG2

To:	3GPP TSG-RAN WG4, 3GPP TSG-RAN WG1

Contact Person:

Name:			Ritesh Shreevastav

E-mail Address:		ritesh.shreevastav@ericsson.com

Attachments:	None

- Overall Description:

RAN2 would like to thank RAN4 for the LS and below is the answer from RAN2 for the question asked by RAN4:

RAN2 has neither introduced nor working on any solutions to inform a UE performing SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement.

2. Actions:

To RAN4:

ACTION: 	RAN2 kindly requests RAN4 to consider above answer for their Rel-18 specification work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG2 Meeting #127-bis              China, CN 			October 14 – October 18, 2024

TSG-RAN WG2 Meeting #128                    Orlando, USA 		          November 18 – November 22, 2024

# Conclusion

In the previous sections we made the following observations:

Observation 1	The SL synchronization solution is very rudimentary in Rel-18 and because of lack of time, there has been no enhancements made during Rel-18.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to reply that “RAN2 has neither introduced nor working on any solutions to inform a UE performing SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement “.

# References

R4-2410352