3GPP TSG-RAN WG2 #127	R2-2407229

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	7.24.2.2

Source:	Ericsson, Nokia

Title:	Discussion on LS to RAN3

Document for:	Discussion

# 1	Introduction

In RAN2#126 RAN2 thoroughly discussed the reply-LS (R2-2404123) to RAN3 but never reached agreement on content. The discussion was held under the agenda item 7.24 TEI18, but notes are collected under 7.18 SDT since it heavily focuses on Small Data Transmissions.

This contribution discusses and proposes a way forward.

# 2	Discussion

## 2.1 Background

The incoming LS asked for answer on impact or issues:

To RAN2 group. ACTION: RAN3 kindly asks RAN2 to confirm whether this network-based solution has any RAN2 impact or issue.

RAN3 asks RAN2 if the discussed solution has any RAN2 impact or issues.

The chairs notes are collected beneath as a summary:

R2-2405649 Discussion on RAN3 LS on SDT signalling optimization for partial context transfer Huawei, HiSilicon discussion Rel-18 TEI18 Proposal 1: RAN2 replies to RAN3 that: 1. Fallback from RRC Resume procedure to RRC connection establishment makes the UE release its security context, all of its data radio bearers and its entire RRC configuration (except default values) causing service and data interruption as well as additional procedures to be triggered in both AS and upper layers. 2. Fallback from RRC Resume procedure to RRC connection establishment is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible. 3. The solution which was introduced by RAN2 is very simple and its additional complexity on top of baseline SDT functionality is negligible. 4. The benefits of the NW-based solution proposed by RAN3 over the legacy mechanism (i.e. release the UE to RRC INACTIVE and perform Paging) are unclear. Discussion - Ericsson agrees with observations of ZTE and Huawei. The reply should be that there are no impacts to RAN2 and the concerns are already clear. - Qualcomm has similar understanding as ZTE and Huawei and the impact is that there is data loss. - Intel thinks that RRCSetup clears the data and it doesn’t serve the purpose that RAN3 is trying to achieve. - CATT supports Nokia proposals and thinks that NAS layer retransmission would solve the issue. - LG thinks that there is no spec impact but there are RAN2 impact which is data loss. - Nokia thinks that some of these signaling issues have been there since Rel-15, but the new SDT issue is data loss. => Reply to LS: - No specification impact (for Rel-17 and 18) - However, there are RAN2 issues, like data loss (explain the issue) and Uu signaling overhead due to security being cleared (explain that as from Rel-15 …) [POST126][033][SDT] LS reply to RAN3 (ZTE) Intended outcome: agree to LS by email Deadline: short

During the session, the above notes was captured. As can be noted an email discussion to finalize the LS reply was initiated in [033] but the email discussion never reached consensus and thus was postponed. The main reason for postponing was that some companies think there might be RAN2 impact, although it could never be shown exactly what the impact would be, and companies needed more time.

## 2.2 RAN2 impact

We think there has not been any convincing argument for RAN2 impact, rather, the feeling is that some companies do not want this specific solution. Please note however that whether the solution should be adopted is discussed in RAN3 and RAN3 only asked for impact and issues.

It seems like companies that think there are RAN2 impacts are mostly concerned of a possible reset of MAC when RRCSetup is received during SDT procedure. Important to know is that for this case, the UE is performing small data transmission with timer t319a running and all the actions are known at the cell level and served by a gNB. When the UE receives RRCSetup message after exchange of UL/DL data RLC/PDCP entities are released and all the MAC operation is known to the gNB – hence, there seems to be no MAC issues i.e when the gNB moves that UE to connected mode, gNB already has knowledge of the current MAC state, and does not need to apply reset of MAC layer, just continue at the current state.

As discussed in the session and noted by the chair there are some issues, which should be included in the answer to RAN3, but not any valuation of the solution itself.

RAN2 discusses internally and answers RAN3 if the discussed solution has:

  - Any RAN2 impact

  - Any RAN2 issues foreseen

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN3 asks RAN2 if the discussed solution has any RAN2 impact or issues.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 discusses internally and answers RAN3 if the discussed solution has:

a.	Any RAN2 impact

b.	Any RAN2 issues foreseen