3GPP TSG-RAN WG2 Meeting # 127	R2-2407232

# Maastricht, Netherlands,  August 19th - 23rd , 2024

Agenda Item:	 7.2.6

Source:	Ericsson

Title:	Aggregated Resource Definition and corrections

Document for:	Discussion

# 1	Introduction

During RAN2#126, as part of new MAC CE design for aggregated spatial relation, it was discussed how to identify the aggregated SRS resources and the below aggrement was reached.

The UE is expected to determine which resources can be aggregated based on the RRC configuration and the criteria defined by RAN1.  This decision can be revisited based on contributions to determine if the expected UE behaviour is possible to simplify.

Currently, there is an FFS in the MAC specification:

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. at least one of the C1, C2, and C3 is set to 1. There can be as many as 16 entries of Spatial Relation for Resource IDi depending on the RRC configuration. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within, defined as in Figure 6.1.3.36-2 to 6.1.3.36-5 in clause 6.1.3.36. Spatial Relation for Resource ID1 corresponds to the spatial relation of the first set of  aggregated SRS resources corresponding to the indicated Positioning SRS aggregation ID[ffs what is the first aggregated SRS resource];

# 2	Discussion

Based upon below TS 38.214 paragraph, the UE will have to perform the checking on which SRS resources are overlapping in time domain so that they can be aggregated.

##### 6.2.1.4.2	SRS bandwidth aggregation for positioning measurements

The UE is expected to be configured with linkage information [linkage] on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission. The UE assumes that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation.

It is possible that the NW may list the SRS resources from different linked SRS resource sets in an order such that the UE identifies which are the resources that are aggregated and define the spatial relation also in an order as indicated in R2-2404767.

Further, R2-2404767 highlights that there may be some ambiguity in mapping the aggregated SRS resources to the correct spatial relation definition.

But in this case, it should be possible to resolve it by following a simple order that spatial relations are listed based upon the first SRS resource of first component carrier which can be used for the aggregation and so on. That is in above case it would be mapping of spatial relation #1 to CC1 SRS Resource1, SRS Resource Set2, SRS Resource #3 CC3. That is, spatial relation ID 1 will be for the first aggregated SRS (Resource ID 1 of CC1 rather than of SRS Resource 3 of CC1).

It is also possible to map the aggregated SRS resources to the spatial relation definition without ambiguity and following a simple order that the first aggregated SRS Resource is from the first component carrier first resource that can be used for aggregation and so on.

Thus, a text proposal as below can be formulated to indicate what is the first aggregated resource:

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. at least one of the C1, C2, and C3 is set to 1. There can be as many as 16 entries of Spatial Relation for Resource IDi depending on the RRC configuration. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within, defined as in Figure 6.1.3.36-2 to 6.1.3.36-5 in clause 6.1.3.36. Spatial Relation for Resource ID1 corresponds to the spatial relation of the first set of  aggregated SRS resources corresponding to the indicated Positioning SRS aggregation ID where the first set of aggregated SRS resources is formed in an ascending order of srs-ResourceID such that the first resource from each carrier that fulfil the aggregation criteria [clause 6.2.1.4.2 of TS 38.214] are aggregated at first.

Agree to the Text Proposal for aggregated resource definition as: the first set of aggregated SRS resources is formed in an ascending order of srs-ResourceID such that the first resource from each carrier that fulfil the aggregation criteria [clause 6.2.1.4.2 of TS 38.214] are aggregated at first.

## 2.2	Correction in MAC spec

As per RAN1 agreements below:

Agreement
For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked 
• SRS resources are per BWP per carrier configuration

Thus, a change as below is needed.

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. at least twoof the carriers C1, C2, and C3 is set to 1. There can be as many as 16 entries of Spatial Relation for Resource IDi depending on the RRC configuration. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within, defined as in Figure 6.1.3.36-2 to 6.1.3.36-5 in clause 6.1.3.36. Spatial Relation for Resource ID1 corresponds to the spatial relation of the first set of  aggregated SRS resources corresponding to the indicated Positioning SRS aggregation ID[ffs what is the first aggregated SRS resource];

RAN2 to agree to change from at least one to at least two in Spatial Relation for Resource IDi description.

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is also possible to map the aggregated SRS resources to the spatial relation definition without ambiguity and following a simple order that the first aggregated SRS Resource is from the first component carrier first resource that can be used for aggregation and so on.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree to the Text Proposal for aggregated resource definition as: the first set of aggregated SRS resources is formed in an ascending order of srs-ResourceID such that the first resource from each carrier that fulfil the aggregation criteria [clause 6.2.1.4.2 of TS 38.214] are aggregated at first.

Proposal 2	RAN2 to agree to change from at least one to at least two in Spatial Relation for Resource IDi description.

# References

R2-2404767

TS 38.321 v 18.2.0

TS 38.214 v 18.2.0