3GPP TSG-RAN WG2 Meeting #127	R2-2407234

Maastricht, The Netherlands, August 19th – 23rd, 2024

Agenda item:	7.2.2

Title:	Correction of Notes for Assistance Data Transfer procedures

Source:	Philips International B.V.

Document for:	Discussion

# Introduction

Sections 8.15.x.2.2 of TS 38.305 define the Assistance Data transfer procedures for SL-RTT, SL-AoA, SL-TDOA and SL-TOA positioning respectively.

For simplicity, the procedures are described as an exchange between Endpoint A and Endpoint B. This avoids having to explicitly cover every scenario but, for clarity, some notes are provided to illustrate how the procedures might be used to exchange data between an SL Target UE, SL Anchor UEs and an SL Server UE or LMF.

In our view, the notes are confusing, if not wrong. In this contribution, we explain our concerns and propose some alternative notes.

# Discussion

In the current release, topologies for the procedures for SL positioning described in TS 23.273 and TS 23.586 can be summarised as an LMF or an SL-positioning Server UE in communication with a Target UE (as UE1), which is, in turn, in communication with a number of Located or Anchor UEs (UE2 to UEn, or, for simplicity, UEx). Broadly speaking, the LMF/Server UE manages the positioning process and performs the position calculations and the Target exchanges data and measurements with the Anchor UEs.

TS 23.586 builds on procedures defined in TS 23.273 to include the case where UE1 does not have access to an LMF and uses, instead, a UE acting as SL Positioning Server UE.

## SL-MO-LR

In 6.20.1 of TS 23.273, Figure 6.20.1-1 describes procedures for SL-MO-LR. After discovering and performing a capabilities exchange with UE2 to UEn (UEx), UE1 establishes a link with an LMF, which manages the positioning process. As part of this process, there can be an exchange of assistance information between the UEs and the LMF.

According to step 13 of Figure 6.20.1-1, UE1 and UEx can request Assistance Data from the LMF/Server UE via SLPP, as defined in TS 38.355 and receive a response also via SLPP. For UE1, this is a direct exchange with the LMF via SLPP. For UEx, UE1 serves as a relay, bundling SLPP messages from UEx via a supplementary services message to the LMF. The LMF supplies SLPP response messages to UEx as a bundle to UE1 via supplementary services and UE1 distributes them to their respective UEx recipients as SLPP messages.

Section 6.8 of TS 23.586 shows this procedure adapted for the case where a Server UE is used in place of the LMF. In this case, UE1 and the Server UE bundle SLPP messages between the Server UE and UEx with RSPP. This is further described in steps 8-10 of clause 7.12 of TS 38.305.

In this version of the specification, UE1 is a Target UE.

## Assistance Data transfer procedure

In TS 38.305, sections 8.15.x.2.2 describe the Assistance Data transfer procedure for the four SL positioning and ranging procedures currently defined (SL-RTT, SL-AoA, SL-TDOA, SL-TOA). Figure 2-1 illustrates generic Assistance Data transfer procedure taken from Section 8.15.2.2.2 but common to all.

Figure 2-1: Assistance Data transfer procedure.

The four procedures are essentially identical with some differences in the Assistance Data that may be transferred. They are also accompanied by three notes illustrating potential usage of these procedures. In our view, two of these notes are misleading and need revision.

## Notes 1 and 2.

From the observations above, we can see that the positioning procedures allow for Anchor UEs (UEx) and Target UE (UE1) to act as Endpoint A, and the LMF or Server UE to act as Endpoint B.

Comparison with the notes from 8.15.x.2.2, we see that note 1 is consistent with this understanding:

NOTE 1: Dependent on the scenario, Endpoint A may be a SL Target UE and Endpoint B may be a SL Server UE or LMF.

Note 2, on the other hand, is not:

NOTE 2: Dependent on the scenario, Endpoint A may be a SL Target UE or SL Server UE and Endpoint B may be a SL Anchor UE.

Endpoint A cannot be a SL Server UE. There is no direct link between the Anchor UE and the Server UE. It is also not clear what assistance information a SL Anchor UE would provide to a Server UE. If we look e.g. at the assistance data for SL-RTT in Table 8.15.2.2.2-1, then the only candidate could be “Anchor UE location coordinates”, since the Tx related parameters are covered by Note 3 (see discussion below). However, the “Anchor UE location coordinates” are not assistance information from an Anchor UE to a Target UE or Server UE, but rather from a Server UE or LMF to a Target UE for position calculation. As specified in steps 12-14 and step 17 of the SL-MO-LR procedure in 6.20.1 of TS 23.273, the LMF or Target UE do not use SLPP, but use other mechanism to fetch the absolute location of an Anchor UE, e.g. using supplementary RSPP signalling message. Also, as specified in steps 9-11 of the UE-only procedure in 6.8 of TS 23.586, the Server UE will fetch the absolute location of an Anchor UE using the supplementary RSPP signalling message with the corresponding Application Layer ID of UE2/…/UEn.

Based on this analysis, NOTE 2 is not needed and causes confusion. A more useful note would cover the case wherein the assistance information that is received from the LMF or Server UE by the Target UE is forwarded/relayed/provided by the target UE to an Anchor UE, i.e., a possible revision could be:

NOTE 2: Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a Target UE.

Proposal 1: Revise Note 2 to: "NOTE 2: Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a Target UE”

## Note 3

Although this does not appear in Figure 6.20.1-1 or its relatives, we can anticipate that a UE that transmits SL-PRS signals may share the SL-PRS Sequence ID with any recipients. Thus, either UE1 or UEx, as RX UE, can act as Endpoint A, while the peer UEx or UE1, as TX UE can act as Endpoint B.

This, however, is the opposite of what Note 3 says:

NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS transmitting (Tx) UE and Endpoint B may be a SL-PRS receiving (Rx) UE.

It is difficult to think of a motive for a Tx UE to request assistance data from an Rx UE. On this basis, we would propose to invert the polarity of Note 3 and revise it as follows:

NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.

Proposal 2: Revise Note 3 to: “NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.”

# Conclusion

In this contribution, we make the following proposals.

Proposal 1: Revise Note 2 to: "NOTE 2: Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a Target UE”

Proposal 2: Revise Note 3 to: “NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.”

A draft CR is annexed.

# References

[1]	TS 38.305

[2]	TS 38.355

[3]	TS 23.273

[4]	TS 23.586

3GPP TSG-RAN2 Meeting # 127	R2-240xxxx

# Maastricht, Netherlands, August 19th - 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | xxxx | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Notes for Assistance Data Transfer procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clauses 8.15.x.2.2 describe the Assistance Data transfer procedure for the four SL positioning and ranging procedures currently defined (SL-RTT, SL-AoA, SL-TDOA, SL-TOA). The four procedures are essentially identical with some differences in the Assistance Data that may be transferred. They are also accompanied by three notes illustrating potential usage of these procedures. Two of these notes are misleading and need revision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clauses 8.15.x.2.2, revise Note 2 to: “NOTE 2: Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a SL Target UE” - In clauses 8.15.x.2.2, revise Note 3 to: “NOTE 3: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Existing notes will confuse, rather than clarify, operation of the Assistance Data transfer procesures.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.2.2.2, 8.15.3.2.2, 8.15.4.2.2, 8.15.5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | New |  |  |  |  |  |  |  |  |

Start of Change

##### 8.15.2.2.2	Assistance Data transfer procedure

Figure 8.15.2.2.2-1 shows the Assistance Data transfer operations for the SL-RTT positioning method.

Figure 8.15.2.2.2-1: Assistance Data transfer procedure.

1.	Endpoint A may determine that certain SL-RTT positioning assistance data are desired and sends an SLPP Request Assistance Data message to Endpoint B. This request includes an indication of which specific SL-RTT assistance data are requested.

2.	If step 1 occurred, Endpoint B provides the requested assistance in an SLPP Provide Assistance Data message to Endpoint A, if available at Endpoint B. The assistance data that may be signalled are listed in Table 8.15.2.2.2-1. If any of the requested assistance data in step 1 are not provided in step 2, the Endpoint A shall assume that the requested assistance data are not supported, or currently not available at Endpoint B. If none of the requested assistance data in step 1 can be provided by Endpoint B, Endpoint B returns any information that can be provided in an SLPP message of type Provide Assistance Data which includes a cause indication for the not provided assistance data.
If step 1 did not occur, Endpoint B determines that SL-RTT assistance data needs to be provided to Endpoint A (e.g., as part of a positioning procedure) and sends an SLPP Provide Assistance Data message to Endpoint A.

NOTE 1:	Dependent on the scenario, Endpoint A may be a SL Target UE and Endpoint B may be a SL Server UE or LMF.

NOTE 2:	Dependent on the scenario, Endpoint A may be a SL Anchor UE and Endpoint B may be a SL Target UE.

NOTE 3:	Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx)UE and Endpoint B may be a SL-PRS transmitting (Tx) UE.

[Editorial note: Same changes to be made to 8.15.3.2.2, 8.15.4.2.2 and 8.15.5.2.2].

End of Change