3GPP TSG-RAN WG2 Meeting #127	R2-2407236

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

RAN2#126 agreed on the following:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

This contribution presents our view on some open issues for MBS and PWS.

# 2		Discussion

## 2.1	UE behaviours for the intended service area

In RAN2#125bis, it was agreed that the MBS broadcast intended service area covers a portion of an NTN cell and can cover the area of more than one NTN cells simultaneously. As highlighted in our previous contribution [2], the Work Item Description (WID) clearly states that the primary focus of RAN2 discussions should be on scenarios where the service area constitutes only a portion of an NTN cell, although the service content may be received throughout the entire cell.

RAN2 has agreed to focus in a service area that covers only a portion of an NTN cell.

During RAN2#126, it was agreed that no specific (physical) mechanisms would be defined to prevent the reception of service content outside the intended service area. Therefore, the next step for RAN2 is to clarify the expected UE behavior when it receives the MBS broadcast service while located outside the intended service area or when it exits the intended service area. RAN2 needs to decide whether a UE should not (or may not) establish or release MRB(s) associated with a service restricted to an intended service area when the UE is either not located within the area or is leaving the area.

In RAN2#126, we referred to the normative text guiding the behaviour of legacy MBS UEs (see the following excerpt taken from Clause 5.9.3 of TS 38.331 V18.1.0). When MBS was introduced in Release 17, cell broadcasts applied to entire cells, and it was deemed sufficient to employ "may" in the related normative text without mandating UE behavior. However, considering the new context in NTN and the vulnerability of GNSS receivers to spoofing at the application layer, it is essential to mandate UE behavior for the feature being specified for NTN. Consequently, a UE interested in an MBS broadcast service should be mandated not to establish its MRB(s) if it is located outside the intended service area. Similarly, a UE receiving an MBS broadcast service should release its related MRB(s) upon exiting the intended service area. Notably, how the UE detects its location relative to the intended service area remains a matter of UE implementation.

A UE does not establish MRB(s) associated with an MBS service limited to an intended service area when it is not located within the area.

A UE releases its established MRB(s) associated with an MBS service limited to an intended service area when exiting the area.

#### 5.9.3.2	Initiation

The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in.

The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service.

## 2.2	Provision of intended service area

In adherence to the WID, we advocate adopting a straightforward approach, steering clear of overly complex solutions when it comes to signalling of the intended service area. In RAN2#126, it was agreed to narrow down the options and consider the following possibilities for including the service area information: SIB20, SIB21, MBSBroadcastConfiguration.

First, the role of the above three signalling options in the framework of MBS is as follows:

- SIB20 is used to inform the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, and the time resource for MCCH). If the serving cell sends SIB20, the UE may receive the MCCH of this cell.

- SIB21 is used to inform the MBS Frequency Selection Area Identities of the current/ neighbor frequencies. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session.

- MBSBroadcastConfiguration message (provided in MCCH) is used to inform the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbor cells which are also broadcasting this MBS session). The UE will receive the MTCH for the interested MBS session.

In summary, MCCH’s time and frequency resource information are carried in SIB20, which indicates when the contents of MBSBroadcastConfiguration can be acquired. SIB21 is only used to carry the MBS FSAI for frequency prioritization. Therefore, we think it is more appropriate to use SIB20 or MBSBroadcastConfiguration for signaling the intended service area. We see the following two options:

- Option 1: Introduce the intended service area (with an ID) in SIB20 and use the same ID in the MBSBroadcastConfiguration  (e.g., in the MBS-SessionInfo IE) to associate a broadcast session with that area.

- Option 2: Introduce the intended service area in the MBSBroadcastConfiguration, together with the associated broadcast session information.

Both options work. While Option 2 is self-contained since information about both a service and its associated intended service area is carried in a single message, Option 1 is more aligned to the WI’s objective which clearly states to specify SIB signalling for the intended service area. Another relevant aspect to consider is how efficient each option is to address any possible updates. A possible scenario, albeit uncommon, would be when a new service becomes available and a new intended service area needs to be added to the existing signalling. In case of Option 1, the SI update notification mechanism would be required informing all UEs in the cell, even UEs that are not interested in receiving MBS. For Option 2, a notification is sent in PDCCH addressed to MBS-RNTI to inform MBS-interested UEs, which is more efficient. Hence, we propose the following:

Information about intended broadcast service area is signalled in MBSBroadcastConfiguration.

## 2.3	Public warning systems in NTN

The scope of the work item to include a service area smaller than a cell in NTN is not limited to MBS but also encompasses other broadcast services such as Public Warning Systems (PWS). The two most prominent PWS are analysed in the following sections.

### 2.3.1	CMAS - Commercial Mobile Alert Service

CMAS, also known as the Wireless Emergency Alert (WEA) in the US, is a Public Warning System (PWS) standardized in 3GPP Release 8. This system supports geo-fencing emergency messages, which enables targeting specific geographic areas. To achieve this, the Warning Area Coordinates IE is used to conditionally display emergency messages based on the UE’s location. If the UE is unable to determine its location, it will always display the message, as specified in TS 23.401 section 9.1.3.5.2.

The content of the Warning Area Coordinates IE defined in TS 23.041 is included in SIB8 within the field parameter warningAreaCoordinatesSegment. Each segment is comprised of one or more octets, and it is uniquely identified by the warningMessageSegmentNumber. The UE must collect all segments before deciding to display the emergency message. The Warning Area Coordinates IE is defined in TS 23.401, with encoding specified in ATIS-0700041.

Service providers have the option to choose from different formats for the warning area, including circle, polygon, or custom-defined, and a warning area may consist of multiple circles and/or polygons. The size of the warning area, i.e., the total size of the Warning Area Coordinates IE,  depends on the number of shapes and the chosen formats. In conclusion, the introduction of an intended service area is unnecessary in this case, as the current system already applies to NTN.

The CMAS emergency message geo-fencing feature can already be used in NTN.

### 2.3.2	ETWS - Earthquake and Tsunami Warning System

ETWS is another PWS standardized in 3GPP Release 8. The system is designed to rapidly disseminate emergency information to the public, ensuring timely alerts in situations such as earthquakes and tsunamis.

Emergency Information is broadcast in two levels:

- Primary Notification: Contains the most urgent and essential information, such as "an earthquake is about to happen.“ When a UE receives a Primary Notification, it triggers a preset alert sound and displays predetermined text on the screen to notify users of the imminent danger. This message is sent in SIB6.

- Secondary Notification: Provides supplementary information about the emergency situation, including details like seismic intensity and the epicentre. This message may be segmented if it is large and is sent in SIB7.

Each ETWS SIB message includes at least the disaster type information, such as a Message Identifier and Serial Number. This helps the UE avoid processing multiple messages containing the same emergency information (e.g., avoids alerting the user multiple times for the same warning), thereby preventing users from being overwhelmed with redundant alerts.

Although ETWS also employs a segmented approach, this does not apply to the primary notification. Therefore, the geo-fencing mechanism required in NTN for areas smaller than a cell cannot be simply extended from CMAS. Introducing additional geographic information to filter ETWS primary notifications, or waiting for the UE to obtain its location, may cause delays in providing critical information.

Introducing geographic information for geo-fencing in ETWS primary notification may cause delays.

To devise an appropriate solution, RAN2 should consult SA1 (and possibly CT1) on the requirements for ETWS primary notifications. Therefore, we propose that a LS is sent to clarify these requirements so that RAN2 can determine a suitable approach for extending the geo-fencing mechanism to ETWS. A draft LS is provided in the appendix.

Send an LS to SA1 to clarify the requirements for ETWS primary notification and the possibility to include the intended warning area in NTN.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has agreed to focus in a service area that covers only a portion of an NTN cell.

Observation 2	The CMAS emergency message geo-fencing feature can already be used in NTN.

Observation 3	Introducing geographic information for geo-fencing in ETWS primary notification may cause delays.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE does not establish MRB(s) associated with an MBS service limited to an intended service area when it is not located within the area.

Proposal 2	A UE releases its established MRB(s) associated with an MBS service limited to an intended service area when exiting the area.

Proposal 3	Information about intended broadcast service area is signalled in MBSBroadcastConfiguration.

Proposal 4	Send an LS to SA1 to clarify the requirements for ETWS primary notification and the possibility to include the intended warning area in NTN.

# 4	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

R2-2404841, Support for broadcast services in NR NTN, Ericsson, RAN2#126, Fukuoko, Japan, May 2024.

# Appendix

3GPP TSG-RAN WG2 Meeting #127	R2-XXXXX

Maastricht, Netherlands, August 19th – 23rd, 2024

Title:	LS to SA1 on NTN broadcast requirements for ETWS

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN2

To:	SA1

Cc: 	-

# Contact Person:

# Name:	Ignacio Pascual

E-mail Address:	 Ignacio.pascual.pelayo@ericsson.com

# Attachments:	None

# 1. Overall Description:

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell a broadcast service (e.g., MBS, ETWS) is applicable. CMAS already offers the capability to geofence emergency messages to an area. However, its segmentation approach may require multiple SIBs reception for decoding and may not be suitable for ETWS’ primary notification.

RAN2 would like to know whether the present geofencing solution for CMAS can be reused to address the geographic information requirements in NTN for ETWS without introducing significant delays.

# 2. Actions:

# To SA WG 1:

# RAN2 kindly request SA1 to take the above information into account and provide feedback whether the present geofencing solution for CMAS can be reused to address the geographic information requirements in NTN for ETWS without introducing significant delays.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #127bis		14th Oct. – 18th Oct. 2025					Hefei, China

TSG RAN WG2 Meeting #128		18th Nov. – 22th Nov. 2025				Orlando, US