3GPP TSG-RAN WG2 Meeting #127	R2-2407237

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.9.2

Source:	Ericsson

Title:	Support for store and forward in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusions from SA2 study report [1], in this contribution, we discuss the objectives for store and forward of the work item [2]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

# 2	Discussion

## 2.1	Architecture

SA2 study item on Store and Forward (S&F) architecture [1] has concluded with two possibilities for the final architecture that will be captured in an informative annex:

- Full core network (CN) onboard. The UE attaches, transfers data (e.g., SMS, MO (Mobile Originated), and MT (Mobile Terminated) data) and detaches from each satellite. The impact on RAN2 protocols is expected to be limited.

- Split MME architecture. The MME functionality is split between satellite and ground parts with an interface out of scope of 3GPP. The HSS is always on the ground.

The SA2 SI on S&F architecture has concluded with two possible architectures: full CN onboard and split MME architecture.

During the last meeting, RAN2 reached an initial agreement for the transmission of MO data, which explicitly states that only the eNB is present on the satellite.

- For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

- 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

- 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

- 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

- 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again.

Unfortunately, this is not aligned with any of the architectures presented in the SA2 study conclusion. Although these conclusions are captured as informative, they should serve as the baseline for further RAN2 discussions.

RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion. This reverts the previous agreement to consider a scenario where only the full eNB is onboard the satellite.

## 2.2	Scenarios

Each of the two selected architectures has distinct implications for the services that can be provided. For example, the split MME architecture may require multiple satellite passes to complete the UE attach procedure with the network. In previous RAN2 contributions, this mode has been designated as the multiple satellite pass scenario.

Conversely, a UE is expected to complete data transmission within a single pass of a satellite when the full CN is onboard. This mode has been designated as the single satellite pass scenario in prior RAN2 contributions.

We believe that RAN2 should initially focus on addressing the general scenarios common to both architectures, i.e., the main scenarios, and defer the evaluation of specific impacts and corner cases if time permits.

RAN2 to address general scenarios that may be common for both architectures.

Consider both single satellite pass and multiple satellite pass scenarios.

Include both MO and MT data within scope.

## 2.3	S&F indication signalling details

In RAN2#126, it was agreed that S&F indication can be provided by SIB, with specific details to be discussed in subsequent meetings. A similar conclusion was reached in the SA2 study [1].

When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

In a satellite constellation that employs Store and Forward, we foresee that the status of the feeder link may significantly influence the UE behaviour across different RRC states. From the UE’s perspective, three scenarios are possible:

- Normal operation. Both service and feeder link are concurrently available.

- Store and forward operation. Only service link is available.

- Combination. The feeder link status changes at a certain point in time within the duration of the coverage pass. A clear example is Earth-moving cells.

To effectively manage these three scenarios, the UE must be able to discern the current operational mode through the indication(s) provided in System Information. Consequently, the UE should be aware of the following:

Whether the serving satellite supports S&F connectivity.

The status of the serving satellite’s feeder.

The remaining duration of the current feeder link status.

By providing this information, the UE will be able to adapt its behaviour appropriately under varying conditions. RAN2 can later determine which is the impact to specific procedures. Therefore, it is proposed that:

UE is informed whether its serving satellite is currently operating in S&F mode (without feeder link connectivity) via System Information broadcast.

UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

## 2.4	Support of legacy UEs

In RAN2#126, it was discussed whether a new barring bit should be added to System Information to prevent UEs that do not support S&F from connecting to such network. Unfortunately, SA2 study report does not include any reference to the support of legacy UEs. Some companies claim that they could be supported and handled with legacy NAS codes depending on the specific situation. Prior to reaching any hasty conclusion and introducing any signaling, we consider that RAN2 should ask SA2 whether it would be necessary to bar those UEs from the network.

Send an LS to SA2 to ask whether UEs that do not support S&F enhancements (legacy or Rel-19) should be barred from a S&F network.

## 2.5	Neighbour and upcoming satellites

In their study [1], SA2 has concluded that the MME can provide, during the Attach or Tracking Area Update (TAU) procedure, a list of Satellite IDs over which the UE may later exchange signalling and data. A UE can determine a satellite’s ID from information provided by the eNB in SIB33. This mechanism allows the UE to identify which satellite(s) may have the necessary context to fulfil its connection requirements such as Attach or TAU. This information is beneficial for the UE implementation as (together with the information in SIB32) it helps to plan optimal network access times, thereby saving energy by avoiding unsuccessful connection attempts.

In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

In NAS, the satellite ID list is based on SIB33 information.

Similarly, it would be beneficial for the UE in AS level to have access to the operation mode of not only its serving satellite but also neighbouring and upcoming satellites. For neighbouring satellite, this information may be included in SIB33, along with their respective satellite ID. For upcoming satellites, this information could draw from synergies with the discontinuous coverage feature and be included in SIB32. This additional information can enhance the efficiency of various AS procedures, such as cell re-selection, paging, or PSM, and facilitate power-saving mechanisms up to UE implementation. One example is that a UE that is located close to the cell border may prioritize the reselection to a cell which does not support S&F architecture.

The UE may be informed of the S&F support status of neighbour and upcoming satellites.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The SA2 SI on S&F architecture has concluded with two possible architectures: full CN onboard and split MME architecture.

Observation 2	In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

Observation 3	In NAS, the satellite ID list is based on SIB33 information.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion. This reverts the previous agreement to consider a scenario where only the full eNB is onboard the satellite.

Proposal 2	RAN2 to address general scenarios that may be common for both architectures.

Proposal 3	Consider both single satellite pass and multiple satellite pass scenarios.

Proposal 4	Include both MO and MT data within scope.

Proposal 5	UE is informed whether its serving satellite is currently operating in S&F mode (without feeder link connectivity) via System Information broadcast.

Proposal 6	UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

Proposal 7	Send an LS to SA2 to ask whether UEs that do not support S&F enhancements (legacy or Rel-19) should be barred from a S&F network.

Proposal 8	The UE may be informed of the S&F support status of neighbour and upcoming satellites.

# 5	References

TR 3GPP TR 23.700-29 v19.0.0

RP-241624

# Appendix

3GPP TSG-RAN WG2 Meeting #127	R2-XXXXX

Maastricht, Netherlands, August 19th – 23rd, 2024

Title:	LS to SA2 on the barring of legacy UEs in store and forward networks

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN2

To:	SA2

Cc: 	-

# Contact Person:

# Name:	Ignacio Pascual

E-mail Address:	 Ignacio.pascual.pelayo@ericsson.com

# Attachments:	None

# 1. Overall Description:

RAN2 is considering introducing a barring bit in System Information broadcast so that UEs that do not support Store & Forward architecture refrain from camping in such a network.

RAN2 would like to know whether UEs that do not support Store & Forward Rel-19 enhancements need to be barred from network access when this architecture is used.

# 2. Actions:

# To SA WG 2:

# RAN2 kindly request SA2 to take the above information into account, and provide feedback whether UEs that do not support Store & Forward Rel-19 enhancements need to be barred from network access when this architecture is used.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #127bis		14th Oct. – 18th Oct. 2025					Hefei, China

TSG RAN WG2 Meeting #128		18th Nov. – 22th Nov. 2025				Orlando, US