3GPP TSG-RAN WG2 #127 	R2-2407238

# Maastricht, Netherlands, 19th - 23rd Aug 2024

Agenda Item:	7.7.2

Source:	Ericsson

Title:	Remaining open issues for NR NTN enhancements

Document for:	Discussion, Decision

# 1	Introduction

The aim of this contribution is to address a couple of issues that were marked as possible come backs.

R2-2404158    Further Discussion on Satellite Switch with Resync    vivo    discussion    Rel-18 NR_NTN_enh-Core

Proposal 1: Change the granularity of capabilities softSatelliteSwitchResyncNTN-r18 and hardSatelliteSwitchResyncNTN-r18 as per band level.

Can come back in the next meeting

R2-2404890    Correction on Location-based CHO    OPPO    CR    Rel-18    38.300    18.1.0    0863 -    F    NR_NTN_enh-Core

Come back in the next meeting

# 2	Discussion

## 2.1	Satellite Switch with Re-sync capability

During previous meetings, we extensively discussed UE capabilities for the satellite switch with resynchronization. A minor issue remains unresolved: whether the granularity of UE capability indications for softSatelliteSwitchResyncNTN and hardSatelliteSwitchResyncNTN should change from per-UE to per-band.

To recap, per-band capabilities are specific to individual frequency bands, detailing UE performance and supported feature on those bands, thereby assisting the network in frequency-specific optimizations and resource allocation. In contrast, per-UE capabilities encompass the UE’s overall performance, features, and data rates across all frequency bands.

As highlighted in [1], RAN2 guidelines recommend defining UE capabilities in the same or finer granularity as their pre-requisites. Defining capabilities in coarser granularity may create ambiguity regarding the support of the coarser capability. This discussion relates to an LS response from RAN4 [2] and the presumed requirement for the parallelMeasurementWithoutRestriction-r17 capability to avoid scheduling restrictions during a soft satellite switch. Given parallelMeasurementWithoutRestriction-r17 is defined per-band, some argue that satellite switch with resynchronization feature should also be per band. However, this was already discussed and RAN2 reached the following agreement:

- RAN2 understands that a UE supporting softSatelliteSwitchResync but not supporting e.g. parallelMeasurementWithoutRestriction-r17 in a band can still perform soft satellite switch with scheduling restriction at the source satellite at the switch

In conclusion, the UE's capability report should include the bands where it supports parallel measurement without restriction (per-band) and whether it supports soft satellite switch with resynchronization (per-UE). This information is sufficient for the network to identify UEs that may face scheduling restrictions during the transition. In addition, satellite switch with resynchronization is a feature at the cell level, in which UE behaviour or implementation does not differ depending on the frequency. Thus, this feature should remain a per-UE capability.

Satellite switch with resync related capabilities (softSatelliteSwitchResyncNTN and hardSatelliteSwitchResyncNTN) are defined per UE.

## 2.2	CHO triggering conditions

In the previous meeting, a contribution [3] was made to address the necessity of modifying a text in stage 2 related to the independent configuration of time or location-based trigger conditions from measurement conditions. It was argued in the contribution that CHO does not effectively work in some cases. One listed example is a hard satellite switch scenario in which the UE is configured with a location-based triggered CHO to a cell which will replace the current cell covering the same area. As we stated during the meeting, this scenario does not actually exist. Clearly, the network would not configure a location-based trigger condition in such a scenario where the time condition is more applicable. Furthermore, we believe it is not within the scope of the specification to enumerate all potential scenarios and explicitly specify network configuration for each one that should be considered. Radio conditions are intricate, and networks configure the most optimal CHO trigger condition based on the prevailing conditions. Nevertheless, this contribution serves as an example of how the referenced text  in Stage 2 (presented in the excerpt in the box) may warrant an update.

The text related to the independent configuration of CHO trigger conditions is complex and requires clarification.

Time-based or location-based trigger conditions may be configured independently from the measurement condition for CHO in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible. However, if the service discontinuity gap time length is not zero or negligible, then a time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

This text originates from a compromise reached in IoT NTN session for both NR and IoT in RAN2#122:

- For CHO in NTN (both NR NTN and eMTC NTN, time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition). We add a description/note saying in which scenarios this is reasonable, e.g. at least hard-switch case where gap is assumed to be zero/negligible.

It is evident from the agreement that the current text in stage 2 does not accurately reflect the intention of the compromise. Firstly, the agreement only provides a single example (hard service link switch), but it is evident that the potential scenarios are not limited to this one. Secondly, the agreement does not stipulate that the configuration of a RRM measurement-based condition is mandatory in cases with a non-zero service discontinuity gap. Therefore, to prevent further confusion, we propose an alternative text that eliminates text redundancy and effectively captures the intent behind the original agreement.

Adopt the following clarification to Stage 2 related to the independent configuration of time or location-based CHO trigger conditions from RSRP-based trigger conditions.

# 3	TP to 38.300

##### 16.14.3.2.2	Conditional Handover

The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified.

NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	The RRM measurement-based event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

Time-based or location-based trigger conditions may be configured independently from RRM measurement-based condition in some scenarios. One such example scenario is a service link switch with a zero or negligible service discontinuity gap time length.

It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

When a time-based trigger condition is used, the source gNB may signal the corresponding parameters to a single target gNB via the Source NG-RAN Node to Target NG-RAN Node Transparent Container in an NG-C based handover, see TS 23.502 [22], or Xn based handover. The source gNB signals the corresponding CHO configuration to the UE in the RRC Reconfiguration message during handover.

When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when to start the early data forwarding to the target NG-RAN node.

Time-based CHO can be performed via RACH-less.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The text related to the independent configuration of CHO trigger conditions is complex and requires clarification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Satellite switch with resync related capabilities (softSatelliteSwitchResyncNTN and hardSatelliteSwitchResyncNTN) are defined per UE.

Proposal 2	Adopt the following clarification to Stage 2 related to the independent configuration of time or location-based CHO trigger conditions from RSRP-based trigger conditions.

# References

R2-2404158

R2-2314016

R2-2404890, Correction on Location-based CHO, OPPO, RAN2#126, Japan, May 2024.