3GPP TSG-RAN WG2 Meeting #127	R2- 2407241

Maastricht, Netherlands, Aug 19th– 23rd, 2024

Agenda Item:	8.4.3

Source:	InterDigital, Inc.

Title:	Discussion on RRM measurement relaxation and offloading

Document for:	Discussion

# 1. Introduction

In RAN2#126 following agreements were made [1].

Agreement Entry/exit condition RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we discuss RRM relaxation of UE MR (Main Radio) with offloading condition from MR to LR (Low power wake-up Receiver).

# 2. Discussion

Criterion for offloading

For power saving operation with MR, an entry condition for LP-WUS monitoring is agreed. To satisfy entry condtion for LP-WUS monitoring, the serving cell quality (e.g., RSRP/RSRQ) measurements via MR needs to be above a threshold. When the entry condition is satisfied, the LR may monitor LP-WUS and stop the legacy PO monitoring with MR.

In addition to the entry/exit conditions for LP-WUS monitoring, serving cell measurement offloading is agreed. According to the agreement, when the offloading condition is satisfied, serving cell measurement is fully performed by LR (i.e., serving cell measurement via MR is not required). As the detailed offloading condition is not defined yet, it is not clear that satisfying the offloading condition means satisfying the entry condition of LP-WUS monitoring. If RAN2 considers two different conditions or two different values, it may happen that while serving cell measurement is performed by LR, monitoring the PO is performed by MR. This operation is not desirable to achieve power saving gains with LR.

Hence, RAN2 needs to consider the same criteria (e.g., threshold) for entry condition of LP-WUS monitoring and offloading condition of the serving cell measurement to LR.

Proposal 1: Support the same criteria for entry conditions of LP-WUS monitoring and offloading conditions of serving cell measurement via LR.

Relaxed RRM measurement and offloding

According to IDLE specification [2], relaxed measurement criterion for intra-frequency and inter-frequnecy measurement with MR is described. The triggering condition of RRM measurement relaxation of neighboring cell measurment is related to ‘UE with low mobility’ and/or ‘UE not at cell edge’ criteria.

##### 5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell. ##### 5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

RAN2 agreed that when an offloading condition is satisfied, serving cell measurement is performed by LR (offloaded from MR) and serving cell measurement via MR is not required. In this case, it is desired to deactivate MR for additional power saving. However, when the serving cell measurement is offloaded from MR to LR, there may be no serving cell measurement via MR. Hence, the UE is not able to determine whether the legacy relaxation condition for neighboring cell measurements is satisfied due to absence of the serving cell measurement via MR.

To handle this issue, measured value (e.g., LP-SS) of the serving cell measurement via LR can be used to determine whether to satisfy the conditions of relaxed measurement for neighboring cell measurement (while the offloading condition is satisfied).

Proposal 2: Support LR measurement for relaxation criteria of neighboring cell measurement when the serving cell measurement is offloaded to LR (i.e., no serving cell measurements via MR).

Relaxed RRM measurement and EMR configuration

According to IDLE sepcification [2], neighboring cell measurement relaxation is not allowed when freuqencies (i.e., neighboring cells) are included in Early Measurement Reporting (EMR) configuration (i.e,. MeasIdleConfig) as below:

#### 5.2.4.9 Relaxed measurement ##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8]; - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8]; … NOTE 1: It is up to UE implementation when to start performing relaxed measurements in RRC Idle/Inactive if multiple methods are configured. NOTE 2: It is up to UE implementation which relaxation method to perform based on the "allowed" cases as specified in TS 38.133 [8] for RRC Idle/Inactive if multiple methods are configured. The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

However, the situation can be different when the UE is equipped with LR in addition to MR. For example, when LR is activated, peforming neighboring cell measurement under EMR configuration without RRM relaxation is not desirable to achieve more power saving gains. While serving cell measurement requires measurement of one cell, neighboring cell measurement generally requires measurents of multiple cells. Given the situation, offloading serving cell measurement to LR may require more power consumption if the UE needs to activate both MR (to measure the neighboring cells under EMR configuration) and LR (to measure the serving cell by offloading).

To avoid the situation and corresponding additional power consumption, measurement offloading from MR to LR for neighboring cell measurement under EMR configuration should be supported. For example, the UE can perform neighboring cell measurement via LR when neighboring cell measurement is relaxed via MR (i.e., legacy relaxation conditions with MR are satisfied).

Proposal 3: Support neighboring cell measurement via LR if the EMR is configured and relaxation criteria is satisfied.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support the same criteria for entry conditions of LP-WUS monitoring and offloading conditions of serving cell measurement via LR.

Proposal 2: Support LR measurement for relaxation criteria of neighboring cell measurement when the serving cell measurement is offloaded to LR (i.e., no serving cell measurements via MR).

Proposal 3: Support neighboring cell measurement via LR if the EMR is configured and relaxation criteria is satisfied.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #126.

[2] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[3] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).