3GPP TSG-RAN WG2 Meeting #127	R2- 2407242

Maastricht, Netherlands, Aug 19th– 23rd, 2024

Agenda Item:	8.4.4

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS operation in RRC_CONNECTED mode

Document for:	Discussion

# 1. Introduction

In RAN2#126 following agreements were made [1].

Agreement In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC messag

In this contribution, we discuss procedure for LP-WUS operation in RRC_CONNECTED mode.

# 2. Discussion

Procedure for (de-)activation of LP-WUS monitoring

In RAN1#116, the following agreements were made on (de-)activation of LP-WUS monitoring [2].

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1 agreed that LP-WUS is enabled/disabled by gNB RRC signaliing. FFS whether to support UE assistance. In gNB’s view, needs to know whether the UE is capable of LP-WUS to enable LP-WUS monitoring. For this purpose, RAN2 supports a procedure that UE reports assistance information related capability of LP-WUR to enable LP-WUS monitoring.

Proposal 1: Support a UE assistance procedure (e.g., capability) that LP-WUS monitoring is enabled/disabled by gNB.

Conditions for (de-)activation of LP-WUS monitoring

RAN1 agreed condition(s) as an option of the activation/deactivation conditions for further study. Although entry/exit conditions are satisfied, there may be potential disconnection between LR and gNB. To avoid such disconnection, a timer can be a good solution to make sure the connection between LR and gNB. For example, while a timer is running, a UE can activate LP-WUS monitoring and until the expiry of the timer. Upon the expiry of timer, the UE may activate MR to check potential coverage lost and report the deactivation of LP-WUS monitoring to the gNB.

Proposal 2: Support a timer as one of the conditions to activate/deactivate LP-WUS monitoring. The detailed conditions (e.g., start/stop) are further discussed.

Proposal 3: Upon expiry of timer, report the expiry of timer to deactivate LP-WUS monitoring.

Channel condition for (de-)activation of LP-WUS monitoring

Moreover, channel condition can be considered as one of the activation/deactivation conditions for CONNECTED mode as agreed in RAN1 for IDLE/INACTIVE modes. For activation, potential out of coverage due to the different coverage of LP-WUS monitoring (e.g., smaller coverage) can be avoided by receiving measurement results from a UE via RRM with MR and not allowing activation of LP-WUS monitoring/deactivation of MR when the measurement result from NR signal is not be enough.

For deactivation, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS can be configured. If the measured channel quality satisfies the threshold (e.g., above the threshold), the measurement can be regarded as in-coverage and successful reception. In contrast, if the measured channel quality is not satisfied, LP-WUS reception can be regarded as out-of-coverage and failure. Based on the decision, for a link monitoring perspective, once the UE detects certain number of reception failures, then the UE needs to report these failures to the gNB to deactivate LP-WUS monitoring.

Proposal 4: Support a channel condition as one of the conditions to activate/deactivate LP-WUS monitoring.

Proposal 5: For service continuity, LP-WUS UE can be configured with a threshold (e.g., channel condition) of DL signal for determining LP-WUS in-coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

Reporting procedure for (de-)activation of LP-WUS monitoring

In another example, the UE reporting on information (e.g., detail information is FFS) for activation/deactivation of LP-WUS monitoring to the considered by gNB. Based on the information, the gNB can determine to activate/deactivate an LP-WUS monitoring for the UE. RAN2 can further discuss detailed criteria of the feasibility information, e.g., low mobility, measured channel quality.

For measuring DL signals for activation/deactivation, a UE capable of OFDM-based LP-WUR can measure both NR-SS (existing SSB) and LP-SS (i.e., if supported) simultaneously. In this case, the measured values can be used for the decision of activation/deactivation. Since qualities of NR-SS and LP-SS are different metrics (e.g., RSRP and LP-RSRP, respectively) RAN2 should discuss how to use the different metrics for the decisions. For example, using the higher value or combined values or another method.

Proposal 6: Support to measure and report a procedure (e.g., RRM by MR) that LP-WUS monitoring is activated (or deactivated) by gNB.

PDCCH monitoring triggered by LP-WUS

In RAN1#117, the following agreements were made on LP-WUS based PDCCH monitoring [3].

Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In the RAN2 meeting [1], we discussed regarding the option 1-1 [1]. We need to discuss another options, e.g., option 1-2. Based on the RAN1 agreement, two options, i.e., option 1-2-1 together with option 1-1 and option 1-2-2, can be considered to support PDCCH monitoring outside DRX active time.

Supporting option 1-2-1 together with option 1-1 can lead to higher power use within short span of time. This is because, supporting option 1-1 regardless of triggering PDCCH monitoring outside DRX active time can make MR stay active for a longer duration within a short span of time. Supporting option 1-2-2 can cause issues due to limited coverage of LP-WUS. For example, in the case option 1-2-2 is supported, UEs monitoring LP-WUS have to completely depend on LP-WUS for DL scheduling without any fallback option.

In RAN2 perspective, option 1-2-2 seems to have more specification impact compared to option 1-2-1. For example, if the PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onduration timer, the UE cannot perform the RRM/RLM during the associated active time. RAN2 should discuss how to support the absence of active time due to the triggered LP-WUS monitoring.

Proposal 7: For triggering PDCCH monitoring via LP-WUS ‘option 1-2-1’ is supported.

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support a UE assistance procedure (e.g., capability) that LP-WUS monitoring is enabled/disabled by gNB.

Proposal 2: Support a timer as one of the conditions to activate/deactivate LP-WUS monitoring. The detailed conditions (e.g., start/stop) are further discussed.

Proposal 3: Upon expiry of timer, report the expiry of timer to deactivate LP-WUS monitoring.

Proposal 4: Support a channel condition as one of the conditions to activate/deactivate LP-WUS monitoring.

Proposal 5: For service continuity, LP-WUS UE can be configured with a threshold (e.g., channel condition) of DL signal for determining LP-WUS in-coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

Proposal 6: Support to measure and report a procedure (e.g., RRM by MR) that LP-WUS monitoring is activated (or deactivated) by gNB.

Proposal 7: For triggering PDCCH monitoring via LP-WUS ‘option 1-2-1’ is supported.

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #126 .

[2] Chairman’s notes, 3GPP TSG RAN WG1 #116.

[3] Chairman’s notes, 3GPP TSG RAN WG1 #117.