3GPP TSG-RAN WG2 Meeting #127	R2-2407243

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	Data Collection for Network-Sided Model Training

Document for:	Discussion

# 1. Introduction

In RAN2-126, the following agreements were made regarding the data collection for the training of network-sided models [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

In this contribution, we address some of the open issues highlighted above.

# 2. Discussion

### 2.1 Handling of large size logged measurements

One of the open issues is on how to handle the reporting of the logged measurements, as it is likely that the measurements may not fit into one RRC message.

This is not a new problem. For example, a similar problem has been addressed for application level QoE reporting and 3GPP has already defined SRB4 (and SRB5, in the case of reporting via the SCG in NR-DC) that can support segmentation, where one upper layer QoE report can be sent segmented into up to 16 RRC messages.

Observation 1: SRB4 and SRB5 already support message segmentation at RRC level.

Thus, SRB4 and SRB5 can readily be used for the sending of measurements logged for the training of network side model. If the logged measurements are expected to be very big and even the segmentation supported by SRB4/5 is not enough, further discussion can be made to determine if a new SRB is to be defined.

Proposal 1: For the beam management use case, RAN2 to agree as the baseline that SRB4 (and associated message segmentation) will be used for the sending of multiple instances of L1 logged measurements to the network.

### 2.2 Reporting of logged measurements

On the network side, measurement reports triggered for RRM are initiated and terminated at the gNB while measurement reports for immediate MDT are initiated and terminated at the OAM. However, since there is no interface between the OAM and the UE, the UE will receive configuration for both immediate MDT and RRM measurements from the gNB and sends the measurements also to the gNB.

In the MDT stage 2 description, it is stated that [2]:

For Immediate MDT, RAN measurements and UE measurements can be configured. The configuration for UE measurements is based on the existing RRC measurement procedures for configuration and reporting with some extensions for location information.

Our understanding is that, from the UE’s perspective, current immediate MDT configuration/reporting follows the same measurement configuration/reporting as RRM measurements, except that location information can be included in measurements that are configured for the sake of immediate MDT (by the inclusion of the includeCommonLocationInfo-r16 IE in the measurement reporting configuration, both for periodic and event triggered reporting) [3].

Observation 2: immediate MDT already supports the triggering of measurement reports periodically or based on measurement events.

For the reporting of logged L1 measurements to be used for network side model training, there are two aspects to consider:

- When a given measurement is to be included/recorded in the log?

- When the logged measurements are to be reported?

For both point a) and b), periodic and event-based triggering can be considered. Additionally, for b), the same approach can also be used as in the case of logged MDT or QoE measurement report, i.e., on demand upon explicit request from the network (e.g., via UEInformationRequest/UEInformationResponse).

Our understanding is that the events that are already available for triggering event-based measurement reporting (e.g., Ax, Bx events) are not suitable for the case of data collection for network side model training, as they were designed to support mobility (e.g., identifying when the UE should be handed over, when the UE needs to start performing additional measurements, etc.,). Also, these events are based on L3 measurements, while the measurements under discussion here (at least for the BM case) are L1 measurements.

Observation 3: Legacy measurement events (e.g., Ax, Bx events) are designed for mobility purposes and based on L3 measurements, and thus are not suitable for triggering the logging of L1 measurements or the reporting of the logged measurements.

Additionally, we think event-based triggering of the reporting of the logged measurements is not as important as in the RRM case, as the main use case for them is for offline model training (e.g., not urgent). The same arguments can be made regarding periodic reporting.

Thus, periodic logging of the L1 measurements and reporting based on explicit network request can be considered as baseline. Further discussion can be made regarding the need for event-based logging or/and reporting, e.g., based on input from RAN1

Thus, we propose:

Proposal 2: For the beam management use case, RAN2 to agree on the following as the baseline for the logging and reporting of L1 measurements for network side model training:

- UE can be configured to log the L1 measurements in a periodic fashion.

- UE sends the logged measurements upon explicit request from the network (e.g., via UEInformationRequest/UEInformationResponse).

Proposal 3: For the beam management use case, event-based logging and/or reporting of logged measurements is left FFS.

# 4. Conclusion

In this contribution, we discussed the open issues regarding logging and reporting of L1 measurements for network side model training, and the following observations and proposals were made:

Proposal 1: For the beam management use case, RAN2 to agree as the baseline that SRB4 (and associated message segmentation) will be used for the sending of multiple instances of L1 logged measurements to the network.

Proposal 2: For the beam management use case, RAN2 to agree on the following as the baseline for the logging and reporting of L1 measurements for network side model training:

- UE can be configured to log the L1 measurements in a periodic fashion.

- UE sends the logged measurements upon explicit request from the network (e.g., via UEInformationRequest/UEInformationResponse).

Proposal 3: For the beam management use case, event-based logging and/or reporting of logged measurements is left FFS.

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	TS 37.320, Stage 2 Radio measurement collection for Minimization of Drive Tests (MDT); Overall description, June 2024

[3] 	TS 38.331, RRC specifications (section 6.3.2, ReportConfigNR IE), June 2024