3GPP TSG-RAN WG2 #127	R2-2407245

Maastricht, Netherlands, Aug 19th– 23rd, 2024	revision of R2-2405290

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#103, the WID for network energy savings in Rel-19 was revised [1]. The core part objective based on the revised WID is:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this paper, we address the third objective highlighted in the WID above. During RAN2#125bis agreements were made only for the adaptation of paging occasions. For reference regarding the progress of the discussion in RAN1 on the third objective, we provide the list of RAN1 agreements in the Appendix.

# 2	Discussion

In this section, we consider the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

### 2.1 Adaptation of SSB signals

Currently, SSBs are specified as always-on transmissions whose periodicity can be changed semi-statically. A legacy NR UE may assume a default SSB periodicity of 20ms during initial cell search. Even though the frequency of most broadcast transmissions can be minimized (e.g., with a periodicity of 160ms or larger), SSB transmissions for UE initial cell search are required to be provided every 20ms regardless of whether there is light/no traffic in the cell. This leads to energy consumption in the network, which can be minimized by adapting the SSB periodicity or skipping SSB transmissions on certain occasions.

In TR 38.864 [2], the evaluation results for adaptation of SSB periodicity were captured. The results show that significant energy savings can be achieved on the network side with such adaptation. For example, Figure 1 shows how much energy saving can be achieved only by changing the periodicity of SSB transmissions from 20ms to 40ms. Note that the setup assumes that the other transmissions/receptions are minimized (e.g., SIB1/PRACH is 160ms) to show the full potential of such SSB adaptation.

Figure 1: NW energy saving (NES) when SSB periodicity is increased from 20ms to 40ms.

Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of SSB transmissions. SSBs can, for example, be provided sparsely as a baseline, and upon need, such as prior to/during handover, positioning services (E-CID) or alike, additional SSBs can be provided on-demand. For UEs in RRC idle/inactive mode, having a longer SSB periodicity e.g., > 20ms as a baseline, and activating additional SSBs upon need can also be considered.

Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode. FFS in RRC idle/inactive mode.

For UEs in RRC idle/inactive mode, a non-initial-access cell may have SSB periodicity of 160ms, as captured in existing specifications:

SSB-MTC2-LP-r16 ::= SEQUENCE {

pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need R

periodicity ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

For example, if a capacity cell is overlapped by a coverage cell, the SSB transmission rate of the capacity cell may be every 160ms while the UE will be able to find the overlapping coverage cell for initial access as it is providing SSBs every 20ms. Then through neighbor cell information (SSB-MTC2-LP) provided by the coverage cell the UE may reselect to the low-rate SSB capacity cell.

Figure 2: Deployment according to legacy specifications in which a coverage cell providing initial access functionality provides SSB every 20ms, while the overlapped capacity cell (non-initial-access) provides SSBs every 160ms.

In legacy specifications the periodicity of SSB can be 160ms for non-initial-access cells.

However, in practice such low-rate SSB cells are not deployed. The main reason is that there may be UEs, for example poor coverage UEs, in need of multiple SSBs (up to 3 SSBs) before such UEs can decode the paging message. This is visualized in Figure 3, in which case the UE needs to wake up more than 320ms ahead of the PO for processing the required number of SSBs. Note that PEI would not have helped in this scenario as the PEI occasion would still be 3 SSBs ahead and upon paging the UE would not be able to go back to deep sleep until the PO.

Figure 3: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO.

Instead, additional SSBs (in addition to a baseline SSB transmission rate) can be transmitted on demand prior to the POs for Rel-19 UEs in RRC Idle/Inactive. Through such scheme, which is backwards compatible as legacy UEs can rely on the baseline SSB transmissions, it would then become practical to deploy such cells that are configured with low rate SSB transmissions. Hence, we think that adaptations to SSBs in capacity cells should be supported also in RRC Idle/inactive.

Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Figure 4: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO if SSB adaptations are done. The UE may choose to wake up closer to PO when additional SSBs are provided.

Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

In RRC Idle/Inactive mode, on top of a baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

### 2.2 Adaptation of PRACH occasions and the related signaling

Another important periodic/always-on activity for a cell that supports initial access is PRACH occasions and their periodicity. Similar to the discussion on SSB transmissions, the frequency of PRACH occasions can be changed dynamically rather than only semi-statically as specified today. These resources cannot be dynamically adjusted to accommodate changes in traffic load or save energy at the gNB. The PRACH reception on such cells must continue quite frequently even when the cell has no/light traffic. Dynamic adaptation of the PRACH occasions in the time domain, i.e., without using the system information update notification mechanism, allows energy saving on the network side while not compromising the initial access performance.

In TR 38.864 [2], the evaluation results for adaptation of the frequency of PRACH occasions were captured. The evaluations show that significant energy savings can be achieved by adapting the frequency of PRACH occasions. For example, Figure 5 shows how much energy savings can be achieved by changing the periodicity of PRACH occasions between 10ms and 40ms. Note that it is assumed in the setup that SSBs are broadcast every 20ms (as in legacy) and SIB1 is broadcast every 40ms.

Figure 5: 10ms PRACH occasion periodicity (left), versus another with 40ms PRACH occasion periodicity (right).

Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of PRACH occasions. For UEs in RRC idle/inactive mode, changing the periodicity of PRACH occasions can also be considered. This can be realized by introduction of a second/additional configuration for PRACH occasions for Rel-19 UEs and dynamic signalling, such as DCI or MAC signalling, can be used to activate these additional PRACH occasions per need basis. This would be backwards compatible as legacy UEs can continue to use such carrier when legacy PRACH configuration is provided as part of system information broadcast.

Support activation of additional PRACH occasions dynamically for UEs in RRC idle/inactive and connected modes

### 2.3 Adaptation of paging

The adaptation of paging was discussed during RAN2 #125bis [Ref], and the following agreements were made:

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

The motivation for the existing framework of paging messages is to distribute the paging frames/occasions (PFs/POs) evenly across DRX cycles in idle/inactive modes. This is to balance the load not only from gNB perspective in a cell, but also from UE perspective to receive the paging message within a period intended based on the configured DRX cycle. From network energy saving perspective, such framework does not provide many opportunities for a gNB to sleep. It may become even more challenging to achieve energy savings when paging messages are broadcasted in all cells in a tracking area.

The current paging framework does not provide many opportunities for a gNB to sleep from network energy saving perspective.

For the purpose of understanding how the PF and PO for paging are determined in the current specifications, we recall the following formulas from the TS 38.304 [3]:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns,

where

T: DRX cycle of the UE,

N: number of total paging frames in T,

Ns: number of paging occasions for a PF,

PF_offset: offset used for PF determination, and

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024.

According to the outcome of RAN2#125bis meeting, there are two different potential solutions that RAN2 down-selected for further study of the adaptation of paging occasions in time domain. One of the potential solutions is to bundle PFs, i.e., to configure legacy PFs sparsely and to define new PFs for Rel-19 UEs that can be configured consecutively to the legacy PFs. Figure 6 shows a comparison of legacy (Rel-18) PFs/POs configuration to a potential Rel-19 PO/PF configuration. As illustrated in this figure, legacy UEs can be supported using the existing PO/PF configuration that is complemented by the Rel-19 PO/PF configuration. Another solution considered by RAN2 is to decrease the number of PFs by extending the set of possible values for N in the above highlighted formulas from the TS 38.304, and to compensate the decrease in the number of PFs by increasing Ns (POs per PF).

Figure 6: Legacy PO/PF configuration example (upper figure) vs proposed Rel-19 PO/PF configuration (bundling PFs shown in the lower figure) providing more gNB sleep opportunities.

Concerning the number of POs in current specifications, up to 4 POs may be associated to a PF through parameter Ns. It is important to note that introducing excessive Ns values may negatively impact scheduling of other traffic when PO resources are fully utilized, as can happen during SI update or Public Warning System notifications. Provisioning of additional PFs/POs should be flexible enough to address such scenarios, and hence RAN2 should strive for the solutions that provides such flexibility. If PFs/POs can be condensed with no/minimum impact on UE reachability, it would be possible for a network to save energy, e.g., a gNB can sleep longer between condensed PFs/POs as it may also be possible to minimize broadcast of common signals such as SSBs or PRACH occasions.

If PFs/POs can be condensed with no/minimum impact on UE reachability, it would be possible for a network to save energy.

It is important to observe that the impact on UE reachability can be avoided by bundling the PFs/POs without affecting the length of a paging cycle. This way, the bundling of PFs/POs would affect only the distribution of the PFs/POs on the timeline, but reachability, i.e., paging cycle length, will not be affected. Furthermore, legacy UEs would be still able to use a cell with bundled PFs as illustrated in Figure 6. Therefore, we think it would be beneficial to bundle PFs/POs. Note that we do not intend to introduce a dynamic adaptation mechanism due to its complexity during updates, especially if some POs would be deactivated. We think that semi-static configurations and updates via system information update notification, as in legacy, is good enough.

The impact on UE reachability can be avoided by bundling the PFs/POs in a way that does not affect the length of a paging cycle.

The adaptation of paging occasions in time domain is achieved via bundling PFs/POs.

Rel-19 UEs are notified about an update in the configuration of Rel-19 paging resources via system information update.

Next, we discuss the mechanisms through which bundling of PFs/POs can be achieved. In the context of bundling, there has been two solutions that are under discussion so far.

Solution 1: One solution is to increase the number of POs within a PF by introducing higher values for parameter Ns, i.e., to increase the number of POs that can be associated to a single PF. Although this may provide a considerable gap for a gNB to sleep, we see disadvantages of such solution that relies on the increase of number of POs associated to a single PF via extending the value range for parameter Ns. In order to provide enough paging resources within a single PF, the Ns value may need to be increased significantly, which has two drawbacks. First, as discussed above, introducing excessive values for parameter Ns may impact the scheduling of other traffic when PO resources are fully utilized. Second, introducing excessive values for parameter Ns may result in many new parameters (e.g., one new firstPDCCH-MonitoringOccasionOfPO would have to be introduced for each PO since all POs are associated to a single PF) and hence a complicated configuration.

Solution 2: Another solution is to define new PFs for Rel-19 UEs that are configured by offset(s) which indicates its timing relative to a PF configured for legacy UEs.  Therefore, in Solution 2 POs are distributed in multiple consecutive PFs instead of a single PF. For this reason, when compared with Solution 1, Solution 2 has less impact on the scheduling of the other traffic, but also a shorter gap during which a gNB can sleep.

In the following, we discuss how advantages of Solution 1 and Solution 2 can be utilized while their drawbacks can be mitigated if they are used together to support bundling of PFs/POs. We describe this below as the 3rd solution.

Solution 3: On one hand, if new PFs for Rel-19 UEs are defined, there will be no need for excessive values for parameter Ns since the newly introduced POs can be distributed among multiple consecutive Rel-19 PFs instead of a single PF. On the other hand, if Ns values are moderately increased, the number of new PFs for Rel-19 UEs can be reduced, and consequently the gap during which a gNB can sleep can be relatively increased, compared to Solution 2. Observe also that as in the case of legacy POs, these few new extra POs can be spread within a single PF via parameter firstPDCCH-MonitoringOccasionOfPO in a way that the scheduling of other traffic is not significantly affected. Furthermore, in order to avoid an excessive number of new parameters in the ASN.1, the distribution of POs within a single PF can be the same for all new PFs defined for Rel-19 UEs in the same BWP. Note that this is already the case in the legacy configuration of the PFs, i.e., each of the PFs in the same BWP has the same distribution of the POs.

Figure 7 shows an example of how Solution 1, Solution 2 and Solution 3 can be used to support bundling of PFs/POs. As illustrated in the figure, Solution 3 can support the same paging capacity with fewer POs per PF and fewer PFs for Rel-19 UEs compared to Solution 1 and Solution 2, respectively. When comparing Solution 3 with Solution 1 illustrated in Figure 7, one can make the following observations. First, the density of the POs per single PF is significantly smaller (i.e., 6 POs in a PF defined for Rel-19 UEs in the case of Solution 3 compared to 12 POs in the PF in the case of Solution 1), and hence the number of slots for scheduling the other traffic is higher in the case of Solution 3, which provides an advantage in terms of delay. Second, Solution 3 requires a configuration with 6 PO-specific parameters (e.g., firstPDCCH-MonitoringOccasionOfPO) instead of 12 in the case of Solution 1, and Solution 3 has simpler configuration than Solution 1. When comparing Solution 3 with Solution 2, one can observe that the gap during which a gNB can sleep is one PF longer for Solution 3 than for Solution 2.

Support bundling of PFs/POs by introducing PFs for Rel-19 UEs that are consecutive to the associated legacy PFs and increasing the number of POs in a PF by extending the value range for Ns moderately.

PFs for Rel-19 UEs are configured by an offset relative to the legacy UEs.

Figure 7: An example of how Solution 1, Solution 2 and Solution 3 t can be used to support bundling of PFs/POs.

Another aspect that was discussed during the RAN2#125bis meeting is the compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs in the case of paging adaptations. The first question that was raised is whether a network should prevent the access of legacy UEs via barring in the case of a cell that uses paging adaptations. This, of course, depends on which solutions are chosen for supporting paging adaptations. Our opinion is that minimizing the impact on legacy UEs should be one of the important criteria that RAN2 takes into consideration when designing paging adaptation mechanisms. As discussed above we are supportive of solutions, such as bundling of PFs/POs, which have the flexibility of supporting legacy UEs with no/minimum impact.

Legacy UEs are not barred in a cell where PF/POs are adapted for Rel-19 UEs..

The second question that was raised is whether the Rel-19 UEs should be allowed to use legacy PFs/POs along with the new Rel-19 PFs/POs. Our opinion is that the network should have a flexibility to decide whether Rel-19 UEs can use only Rel-19 PFs/POs or both Rel-19 PFs/POs and legacy PFs/POs. For example, an operator may want to control the load on different PO/PFs depending on estimated population of legacy vs Rel-19 UEs. On one hand, in case of high legacy UE paging load, the network may want to configure the Rel-19 PFs/POs to use only Rel-19 PFs/POs such that the congestion on legacy paging resources is minimized. On the other hand, in the case of a low load, the network may want to configure the Rel-19 PFs/POs to use both Rel-19 PFs/POs and legacy PFs/POs such that the utilization of legacy PFs/POs is maximized.

It should be possible for the network to configure Rel-19 UEs to use legacy PFs/POs, i.e., while some other Rel-19 UEs use Rel-19 PFs/POs or configure all Rel-19 UEs to use Rel-19 PFs/POs.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

Observation 2	In legacy specifications the periodicity of SSB can be 160ms for non-initial-access cells.

Observation 3	Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Observation 4	Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

Observation 5	Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

Observation 6	The current paging framework does not provide many opportunities for a gNB to sleep from network energy saving perspective.

Observation 7	If PFs/POs can be condensed with no/minimum impact on UE reachability, it would be possible for a network to save energy.

Observation 8	The impact on UE reachability can be avoided by bundling the PFs/POs in a way that does not affect the length of a paging cycle.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode. FFS in RRC idle/inactive mode.

Proposal 2	In RRC Idle/Inactive mode, on top of a baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

Proposal 3	Support activation of additional PRACH occasions dynamically for UEs in RRC idle/inactive and connected modes

Proposal 4	The adaptation of paging occasions in time domain is achieved via bundling PFs/POs.

Proposal 5	Rel-19 UEs are notified about an update in the configuration of Rel-19 paging resources via system information update.

Proposal 6	Support bundling of PFs/POs by introducing PFs for Rel-19 UEs that are consecutive to the associated legacy PFs and increasing the number of POs in a PF by extending the value range for Ns moderately.

Proposal 7	PFs for Rel-19 UEs are configured by an offset relative to the legacy UEs.

Proposal 8	Legacy UEs are not barred in a cell where PF/POs are adapted for Rel-19 UEs..

Proposal 9	It should be possible for the network to configure Rel-19 UEs to use legacy PFs/POs, i.e., while some other Rel-19 UEs use Rel-19 PFs/POs or configure all Rel-19 UEs to use Rel-19 PFs/POs.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

# Appendix

### RAN1#116 Agreements

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

### RAN1#116bis Agreements

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2