3GPP TSG-RAN WG2 #127	R2-2407248

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management use cases

Document for:	Discussion

# 1	Introduction

The topic of NW-side data collection was discussed during the Rel.18 SI, and different technical enhancements to support this feature were included in the TR 38.843. In this paper, we first provide an overview of the Rel.18 discussion, and then we draw some proposals for the Rel.19 work item.

# 2	Discussion

Related to network side data collection, the following agreements were reached during the WI meetings in RAN2.

Agreements from RAN2#125-bis:

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

Agreements from RAN2#126:

- For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

2	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

In the following, we further develop the expected technical impacts related to this topic, taking into account the above agreements and the online discussion during the last meeting.

## 2.1 How to handle the case of collected data exceeding single RRC message size

The first issue to address from the above agreements is the “FFS how to handle case when single RRC message is not sufficient”. 
Reporting multiple instances of logged L1 measurement result from UE to gNB is a useful feature that allows the UE to avoid continuously transmitting reports of individual measurement results which will not be used anyhow for “real-time” decisions at the gNB, thereby saving the UE power consumption and data overhead. However, it can happen that by doing that all the data collected by the UE may not fit a single RRC message. It was discussed then by RAN2 how to address this issue, e.g. by supporting RRC segmentation for such use case.
We first notice that UL RRC segmentation is supported in legacy for the transmission of UECapabilityInformation and MeasurementReportAppLayer, which is used for the transmission of QoE-related information. For the case of QoE, the data to be transmitted are application layer measurements, and as such, they are generated by the application layer and injected in the RRC layer that in turn would send this data to the gNB as an octet string. Hence, the segmentation is motivated by the fact that the application layer may pass to RRC a bunch of application layer data (in XML format) which could not fit the maximum supported size of a PDCP SDU. Thus, segmentation is needed at RRC in such case. 
Similar argument holds for the case of UECapabilityInformation: capabilities information reflects fundamental UE properties and, similar to the QoE case, they are generated without taking into the maximum supported size of a PDCP SDU. Capabilities are preconfigured properties of the device, which are configured by the chipset or UE manufacturer and stored in the UE memory. Considering that the capabilities envisaged by the RRC specifications and that a UE can support could be much larger than what a single RRC message can convey, then a standardized RRC segmentation procedure is needed to allow the UE to segment the various capability IEs into different messages, and to allow the decoder at gNB side to combine all the capability IEs into a single UE capability object associated to the UE.

UL RRC segmentation is used in legacy when the data to be transmitted are generated outside the RAN (i.e. as in QoE) and hence no control of the maximum supported size of a PDCP SDU can be guaranteed; or when the amount of information to be transmitted in a single container is so large that the message exceeds the maximum supported side (as in UECapabilityInformation).

The case of AIML data collection is different in our view. Data are generated in the RAN at RRC level, they do not come from higher layers and they are not preconfigured in the device. In our case, data just includes the radio measurement results performed at different points in time. Even though we have not agreed yet on the details of the data that the UE should collect, it is very unlikely that information associated to the radio measurement results taken at a certain point in time will exceed the maximum supported size. As an example, at a given point in time the UE may store the RSRP results, cell/beams information, possibly location/time info, but certainly such info will not exceed the maximum supported PDCP size. 
Hence, rather than on QoE-/UECapabilityInformation-like approach we should get inspiration from the logged MDT. As a reference the logged measurement report in logged MDT looks as follows in TS 38.331:

Legacy logged measurement report used for logged MDT: LogMeasReport-r16 ::= SEQUENCE { absoluteTimeStamp-r16 AbsoluteTimeInfo-r16, traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), logMeasInfoList-r16 LogMeasInfoList-r16, logMeasAvailable-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableBT-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableWLAN-r16 ENUMERATED {true} OPTIONAL, ... } LogMeasInfoList-r16 ::= SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

In logged MDT, the logged measurement results are included in the logMeasInfoList. If UE cannot transmit in the RRC message all the logged measurements stored by the UE, the UE signals to the network via logMeasAvailable flag that there are still logged data available for transmission to the gNB. No standardized RRC segmentation is specified for this case. It is simply the UE implementation that based on the current size of the UL RRC message determines how large should be the information included in the logMeasInfoList. That is possible because the size of the entries of the logMeasInfoList (i.e. LogMeasInfo) are very much below the maximum supported size.

In the logged MDT, the logged measurements stored by the UE are signalled in a list of logged measurement results (i.e. logMeasInfoList). The UE implementation can determine how many entries to include in this list such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is used in MDT.

The logged MDT use case is very similar to our use case in AIML. We do not need to specify a standardized RRC segmentation procedure for the reporting of collected data for NW-side model training. The collected data can be included in a measurement results list, where each entry corresponds to measurements performed at different points in time. If the overall amount of collected data exceeds the maximum PDCP SDU size, it is the UE implementation that makes sure to populate such measurement result list accordingly. This would reduce a lot the standardization effort needed to define an RRC segmentation procedure.

The reporting of AIML collected data for NW-side model training resembles the reporting of logged MDT measurements, because the collected data are generated at RAN level (unlike QoE), and the information associated to measurements results collected at a given point in time do not exceed the maximum size of PDCP SDU (unlike UECapabilityInformation).

Given the above considerations, we propose the following:

The collected AIML-specific data stored by the UE are signalled in a list whose entries include information associated to radio measurements taken at different points in time (as for the logged MDT).

The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements).

Once discussed/agreed the above, RAN2 can further discuss how to transmit the remaining collected AIML data to the gNB. To this end, we can again get inspiration from the logged MDT, i.e. the UE indicates to the gNB that there available collected data to transmit, and then the gNB can further request the UE to transmit such remaining samples. Or the UE can simply continue the transmission of the remaining samples in following RRC messages (e.g. based on the priority of the SRB allocated to data collection), or postpone the transmission to the following transmission occasion for the AIML reporting, e.g. in case the UE is configured to periodically report the collected data.

In case all the collected data do not fit into a single RRC message, RAN2 to discuss:

  - Whether and how the UE should inform the network about remaining collected data to be transmitted.

  - When the UE should transmit remaining collected data, e.g. in following RRC message(s) based on SRB priority, upon gNB request, at the next periodic transmission occasion, etc.

## 2.2 When the UE should perform/stop the radio measurements for NW-side data collection

For the case of beam management, the NW may configure the UE with a specific set of CSI-RS/SSB resources specifically configured for training purposes. On such resources, the UE will then start performing layer-1 measurements and log the measurement results and any possible useful information associated to it. From the point of view of the gNB, this set of CSI-RS/SSB resources used for training purposes may correspond to the set A and set B, because (similar to the UE-side data collection) during the NW-side data collection the gNB would need to know the results on all the beams so that during the inference the gNB just needs to configure the set B and derive via the AIML model the prediction results on the set A. At the same time, the UE would need to perform the legacy measurements for legacy “real time” scheduling decision on another set of CSI-RS/SSB resources.

The UE should perform measurements based on a L3 configuration indicating the resources (CSI-RS/SSB) specifically configured for NW-side data collection purposes.

When the logging should start is something that RAN2 should discuss. The logging may start upon receiving such configuration from the NW, and it may last for some time depending on NW decisions, or it may start upon fulfilling certain events (as for MDT). For example, if the network needs to collect data for training a model to improve the system performances in regions with poor coverage, it might be useful to configure the UE to start the data collection when the UE ends up in poor coverage regions, or in proximity of cell borders.

RAN2 to consider the following approaches for the starting of the NW-side data collection procedure:

  - The UE starts the data collection upon receiving a L3 configuration for NW-side data collection

  - The UE starts the data collection upon fulfilling certain events, e.g. the UE starts the data collection when it ends up in poor coverage regions.

As mentioned above, the data collection can continue for certain time configured by the network (as in logged MDT), or it can continue until certain events are fulfilled. For example, the data collection can continue until the UE is in poor radio coverage reason. Further, we need to consider potential UE-side issues that may cause the stopping of the data collection. For example, the UE can become memory limited, in which case the data collection could stop.

RAN2 to consider the following mechanisms for the stopping of the NW-side data collection procedure:

  - The UE continues data collection for a certain configurable time (as in logged MDT)

  - The UE continues data collection until certain events are fulfilled, e.g. until the UE is in poor radio coverage

  - UE-side reasons, e.g. UE becoming memory limited

## 2.3 How the UE should report the collected data

In the last RAN2#126 meeting, it was agreed to support the periodical reporting. It is currently FFS whether event-based or “on-demand” approaches should be used. 
One could argue that, since there is no latency requirement involved in the transmission of the AIML-related collected data, a periodical reporting would be sufficient. We note however that in some cases the data collection could be stopped by the UE at any point in time. For example, the UE may become memory limited, or the UE may exit certain radio conditions such that the data collection would be stopped by the UE. In this case, it is useful if the UE could transmit the collected data or at least indicate to the network that the data collection is stopped. For example, if the UE stops the data collection due to UE memory becoming full, this is definitely an information that the network should know as soon as possible, so that the network can start retrieving the data in order to free up the UE memory, and to avoid that the UE needs to skip the logging of some radio measurements. Hence, both the event-driven reporting of collected data and the on-demand report of collected data may be useful to allow the UE to quickly free up its memory, and continue the data collection.

Further, the on-demand report gives to the gNB some further flexibility on when to get the collected data, especially as proposed in Proposal 3b if the collected data do not fit into one RRC message.

Event based reporting of collected data at the stopping of data collection can be configured.

On-demand reporting of collected data is supported.

## 2.4 UE memory capabilities

During the last RAN2 meetings, we have discussed at length the problem of UE memory limitations that may arise when performing the data collection. It was proposed to introduce a minimum memory requirement (as in MDT) for the AIML data collection, or to define in a UE capability the maximum memory that a UE can support for the logging of data related to NW-side data collection. The latter approach gives some more flexibility to the UE implementation. These two alternatives can be considered and discussed by RAN2.

RAN2 to discuss the following options for defining UE memory limitations for a UE supporting the data logging:

  - Introduce a minimum requirement on the UE memory size for the logging of data related to NW-side data collection

  - Define in a UE capability the maximum memory values that a UE can support for the logging of data related to NW-side data collection

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UL RRC segmentation is used in legacy when the data to be transmitted are generated outside the RAN (i.e. as in QoE) and hence no control of the maximum supported size of a PDCP SDU can be guaranteed; or when the amount of information to be transmitted in a single container is so large that the message exceeds the maximum supported side (as in UECapabilityInformation).

Observation 2	In the logged MDT, the logged measurements stored by the UE are signalled in a list of logged measurement results (i.e. logMeasInfoList). The UE implementation can determine how many entries to include in this list such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is used in MDT.

Observation 3	The reporting of AIML collected data for NW-side model training resembles the reporting of logged MDT measurements, because the collected data are generated at RAN level (unlike QoE), and the information associated to measurements results collected at a given point in time do not exceed the maximum size of PDCP SDU (unlike UECapabilityInformation).

Based on the discussion in the previous sections we propose the following:

Proposal 1	The collected AIML-specific data stored by the UE are signalled in a list whose entries include information associated to radio measurements taken at different points in time (as for the logged MDT).

Proposal 2	The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements).

Proposal 3	In case all the collected data do not fit into a single RRC message, RAN2 to discuss:

a.	Whether and how the UE should inform the network about remaining collected data to be transmitted.

b.	When the UE should transmit remaining collected data, e.g. in following RRC message(s) based on SRB priority, upon gNB request, at the next periodic transmission occasion, etc.

Proposal 4	The UE should perform measurements based on a L3 configuration indicating the resources (CSI-RS/SSB) specifically configured for NW-side data collection purposes.

Proposal 5	RAN2 to consider the following approaches for the starting of the NW-side data collection procedure:

a.	The UE starts the data collection upon receiving a L3 configuration for NW-side data collection

b.	The UE starts the data collection upon fulfilling certain events, e.g. the UE starts the data collection when it ends up in poor coverage regions.

Proposal 6	RAN2 to consider the following mechanisms for the stopping of the NW-side data collection procedure:

a.	The UE continues data collection for a certain configurable time (as in logged MDT)

b.	The UE continues data collection until certain events are fulfilled, e.g. until the UE is in poor radio coverage

c.	UE-side reasons, e.g. UE becoming memory limited

Proposal 7	Event based reporting of collected data at the stopping of data collection can be configured.

Proposal 8	On-demand reporting of collected data is supported.

Proposal 9	RAN2 to discuss the following options for defining UE memory limitations for a UE supporting the data logging:

a.	Introduce a minimum requirement on the UE memory size for the logging of data related to NW-side data collection

b.	Define in a UE capability the maximum memory values that a UE can support for the logging of data related to NW-side data collection