3GPP TSG-RAN WG2 #127	R2-2407250

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.1.4

Source:	Ericsson

Title:	UE-side Data Collection

Document for:	Discussion

# 1	Introduction

The topic of UE-side data collection was discussed at length during the Rel.18 SI, and several different options were discussed, some of them with impact in 3GPP and some not. However, eventually no clear conclusion was reached, and according to the new Rel.19 WID the study of UE-side data collection alternatives has to continue in Rel.19.

# 2	Discussion

Data collection is an essential step to allow UE-side model training. The UE needs to perform and collect radio measurements which will then be used to train UE-side models. A critical question that arose during the Rel.18 SI is whether the collected data should be transferred to the entity performing the UE-side model training in a transparent or non-transparent way with respect to 3GPP.

In RAN2#126, RAN2 further progressed on this topic with a handful of potential solutions shown below.RAN2 concluded that option 1a is a transparent option with respect to the MNO, whereas options 1b/2/3 are not transparent. In particular, in option 1a the termination entity is always a data collection entity outside the MNO (also called OTT server), whereas in option 1b/2/3 there should be at least a termination entity inside the MNO, also called first termination entity.

The outcome of this discussion was captured in TR 38.843:

##### 7.2.1.3.2 Data collection for UE-side model training The following options were discussed in RAN2: 1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact. 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |

RAN2 also discussed during the last RAN2#126 meeting whether there is the need to define the final termination entity. In our understanding the final termination entity is the entity which eventually will perform the UE side model training, which can be either inside or outside the MNO. Whether the final termination entity is located inside or outside the MNO depends on the specific deployment, and 3GPP can simply assume that both solutions are possible in a real-world scenario. RAN2 does not need to take any assumption on that. 
In particular:

- For option 1a), the final termination entity is the OTT server (this is already clear in the current version of the TR).

- For option 1b/2/3, the final termination entity can be either the Server for data collection for UE-side model training (inside the MNO), or the OTT server (outside the MNO)

We propose to clarify the above in the TR.

Clarify in the TR that for option 1b/2/3, the final termination entity can be either the Server for data collection for UE-side model training (inside the MNO), or the OTT server (outside the MNO).

## 2.1 Controllability

Controllability and visibility were discussed at length during the past meetings. Here, we present our views, and try to address some of the FFSs.

Related to the controllability, first of all we would like to focus on the following definition of controllability as included in the revised TR:

- Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed.

In our view, the above definition of controllability is not complete, since it only focuses on the management of the data transfer. Controllability is a broader concept not only limited to the data transfer. There are in our view two levels of controllability that the MNO can exercise:

- The MNO's/network’s ability to manage/control the data collection process for UE-side model training, including e.g. the ability to initiate/terminate the data collection process:

  - The MNO/network should be aware of the fact that a data collection session for the UE-side model training needs to start, and that a data collection session is ongoing. This is because network resources will be consumed to ensure a proper UE-side model training, so the MNO needs to supervise this procedure, and be able to allow/reject it depending on network conditions.

- The MNO's/network’s ability to manage/control the data collection transfer and the associated traffic flow

  - Once the data collection session is completed, the MNO should be aware of the fact that the collected data needs now to be transferred from the UE. The MNO can thus control (allow/disallow, prioritize/deprioritize) this procedure and manage the related traffic.

The latter bullet is already captured in the current definition of full controllability in the TR. However, the first bullet is not currently considered. It is important that the MNO's/network’s is not only limited to the data transfer, rather the MNO/network should be able to control and manage the entire data collection procedure, including both the management of the data transfer (already captured) and the management/control (e.g. initiation/termination) of the data collection for UE-side model training before the actual data transfer.

Clarify in the TR that the full controllability of the UE-side data collection implies the MNO´s ability to manage both the data collection process (including e.g. the ability to allow/reject/initiate/terminate/pause the data collection process) and the data transfer (as already agreed).

It was discussed in RAN2#126 how to achieve controllability in the different options. The discussion appeared to be controversial, and it might be hard for RAN2 to agree on the technical details of how to achieve controllability, mainly because many options have impact outside RAN2. It would be instead preferrable from RAN2 point of view to assume that AIML-specific full controllability should be ensured in all the options (except option 1a). Whether the controllability should be full or partial or completely absent should be ultimately a decision of the MNO, and it may depend on the specific deployment. It should not depend on potential limitations of a specific option. Thus, from RAN2 point of view, a requirement is that for all the options 1b), 2), and 3, it should be possible to have partial/full controllability, and if RAN2 is not clear on how to achieve that, then SA WGs should be involved.

From RAN2 point of view, AIML-specific controllability full or partial should be possible in all the options (except option 1a). The level of controllability should depend on the MNO decision. RAN2 consult SA2 to discuss the details of how controllability can be achieved for the various options.

## 2.2 Visibility

Related to the visibility, our view is very similar to the one on the controllability. Assuming that the first termination entity is inside the MNO, as it is the case for option 1b/2/3, then there should be ways for the MNO to achieve visibility. On the other hand, if the first termination entity is outside the MNO (as for option 1a) then no standardized visibility can be achieved, e.g. it can be achieved as part of an SLA, as already agreed.
As for the controllability case, we believe that it might be hard for RAN2 to agree on all the details on how to achieve visibility, considering that many options have major impacts in other WGs. It would be instead preferrable from RAN2 point of view to assume that for all the options (except 1a) it should be possible to achieve partial or full visibility. RAN2 should then acknowledge that how to achieve visibility is out of RAN2 scope at least for option 1b/2 and for option 3 in case a UP solution is adopted.
Furthermore, it is proposed removing from the TR the option of “no standardized” visibility for option 1b/2/3, because if no standardized visibility is required in a certain deployment, then option 1a should be used.

Standardized full or partial visibility should be possible in all the options (except option 1a). RAN2 consult SA2 to discuss the details of at least option 1b/2 and 3 (if UP solution is adopted).

No standardized visibility should be considered only for option 1a.

## 2.3 Analysis of the options

In the following, we further analyse the technical details of the options agreed in the SI.

#### 2.1.1 Option 1a)

For option 1a), we have already clarified in the TR that no 3GPP spec impact is expected. Visibility/controllability can be achieved via SLA, but that is not something that RAN2 needs to further discuss.

The only thing that is worth clarifying in the TR is the controllability of the option 1a. In particular, it was captured in the TR that “no AI/ML specific controllability” is considered, which makes sense considering the definition of option 1a). However, as a “solution for network controllability” it was captured in the brackets that “the OTT server can directly request data from the UE”. This is one option, but as part of an SLA, the MNO may have ways to interact with the training entity (OTT server) and achieve “no AI/ML specific” controllability. 
It is proposed then that we clarify in the TR, that controllability (besides visibility) can be achieved as part of an SLA. It is also proposed to clarify in the row “Solution for network controllability” that no solutions for network controllability related to option 1a will be discussed by RAN2, as for the case of visibility.

For option 1a, clarify in the TR, that controllability (besides visibility) can be achieved as part of an SLA, and that RAN2 does not need to discuss solutions for NW controllability and visibility.

#### 2.1.2 Option 1b)

In option 1b), data is uploaded in a non-transparent way via user plane. The data should be first terminated in the server for UE-side data collection, inside the MNO, and optionally the data can be further forwarded to an entity outside the MNO for the purpose of UE-side model training.

With the above assumption, then it should be possible for the MNO (and the network more in general) to have controllability of the data collection process and visibility of the data, given that the data will be firstly terminated inside the MNO.
This means that the MNO can control the data collection process and configure accordingly the involved network nodes on whether to start/stop/reject the data collection, allow/not allow the data transfer as well as managing the data transfer traffic, in order to ensure for example the QoS and limit the network overload. For example, the operator can configure a gNB/UPF with criteria to allow/disallow the transfer of data and also to control the amount of data that are transferred from the UE to the server. 
All these aspects are out of RAN2 scope, as highlighted in Section 2.1.

With Option 1b), it is possible for the MNO to control (e.g. traffic management, QoS policies, admission control, etc) the UP traffic associated to the data collection, as it would do for any other UP service injected into the 3GPP network. Details on how to achieve controllability are up to SA2.

Option 1b) in itself does not have any limitation in the level of visibility that can be achieved. For example, upon receiving a service request from the UE for data collection, the operator can configure the network to setup different QoS flows, i.e. a QoS flow for the data that should be visible to the MNO, and a QoS flow for data that should not be visible (if there are data that cannot be visible to the MNO). The UE will then encrypt data accordingly, to ensure that certain data can be visible to the server for data collection for UE-side model training (inside the MNO). All these aspects are out of RAN2 scope, as highlighted in Section 2.2.

With Option 1b), it is possible for the MNO to decide whether to have full/partial or no visibility of the UP traffic associated to the data collection. Details on how to achieve visibility are up to SA2.

#### 2.1.3 Option 2

It was already discussed that for option 2, the MNO can achieve controllability and visibility of the data collection process, though the control plane. In this option, the CN node in charge of controlling this procedure needs to interact with the “server for data collection for UE-side model training” (inside the MNO) or with the OTT server (in case the training is performed outside the MNO premises) which are the possible final termination entities of the data transfer. Hence, there should be ways for the “server for data collection for UE-side model training” or for the OTT server to inform the CN node that the data collection should start/stop, so that the CN node can determine whether to allow/reject the data collection procedure and also whether to establish/release the control plane for the data collection transfer.

In option 2, to achieve controllability, the first termination entity (a CN node) should be able to interact with the “server for data collection for UE-side model training” (for the case of final termination entity inside the MNO) and with the OTT server (for the case of final termination entity outside the MNO). Details up to SA2.

Related to the control plane, the natural choice seems to be NAS signalling, which is also captured in the TR. However, NAS is not a RAN2 protocol and any assumption/decision on the usage of NAS cannot be made by RAN2. In particular, the NAS protocol might not be dimensioned to carry large amount of data, which could be the typical case for the transfer of data collected for AI/ML purposes. The evaluation of a NAS-based solution should be done in SA2, and RAN2 cannot exclude a-priori a UP-based solution whose feasibility can be studied by SA2.

The feasibility of a NAS-based solution for the option 2 should be evaluated by SA2. RAN2 does not preclude a UP-based solution.

#### 2.1.4 Option 3

In option 3, the data are collected by the OAM which can have the control of the data collection session and have visibility of the transferred data. For the moment, RAN2 has mainly considered CP as the candidate solution for option 3. We note that a CP solution-based solution for option 3 can have issues in terms of scalability, and in terms of standardization efforts in RAN, especially considering that many different RAN AIML functionalities/use cases will be introduced by 3GPP in coming releases.

Hence, similar to option 2, a UP solution can also be considered, in which the UP data traffic is terminated in the OAM, based on network configuration. How to realize this can be discussed in SA WGs, e.g. SA2/SA5.

For option 3, RAN2 does not preclude a UP-based solution. SA2/SA5 to evaluate feasibility.

Furthermore, as for option 2, the OAM needs to interact with the “server for data collection for UE-side model training” (inside the MNO) or with the OTT server (in case the training is performed outside the MNO premises), because they are the possible final termination entities of the data transfer. If the training entity is the OAM itself, this interaction is obviously not needed, but if the training entity is outside the OAM, then the OAM node needs to interact with the training entity (final termination entity) in order to allow/reject the data collection procedure and to establish/release the CP/UP for the data collection transfer. This type of interaction between the OAM and the training entity should be discussed in SA WGs.

In option 3, if the training entity is outside the OAM, the OAM should be able to interact with the “server for data collection for UE-side model training” (for the case of final termination entity inside the MNO) and with the OTT server (for the case of final termination entity outside the MNO). Details up to SA2/SA5.

## 2.3 Other SI objectives

The WID contains other RAN2-related objectives such as:

- Necessity and details of model Identification concept and procedure in the context of LCM

- Model transfer/delivery

However, RAN2 did not have time to discuss the above objectives which instead were considered in the RAN1 discussion. Given this we propose the following:

Related to the SI objectives of model-based LCM and model transfer/deliver, no changes to TR were considered by RAN2 during the Rel.19 SI discussion. Conclusions are up to RAN1.

## 2.3 Text proposal for TR 38.843

The above considerations are captured in a TP in the Annex of this contribution. The TP takes as baseline the latest version of TR 38.843 including the agreements from the last RAN2#126 meeting.

Agree the TP to TR 38.843 in the Annex 1.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in the TR that for option 1b/2/3, the final termination entity can be either the Server for data collection for UE-side model training (inside the MNO), or the OTT server (outside the MNO).

Proposal 2	Clarify in the TR that the full controllability of the UE-side data collection implies the MNO´s ability to manage both the data collection process (including e.g. the ability to allow/reject/initiate/terminate/pause the data collection process) and the data transfer (as already agreed).

Proposal 3	From RAN2 point of view, AIML-specific controllability full or partial should be possible in all the options (except option 1a). The level of controllability should depend on the MNO decision. RAN2 consult SA2 to discuss the details of how controllability can be achieved for the various options.

Proposal 4	Standardized full or partial visibility should be possible in all the options (except option 1a). RAN2 consult SA2 to discuss the details of at least option 1b/2 and 3 (if UP solution is adopted).

Proposal 5	No standardized visibility should be considered only for option 1a.

Proposal 6	For option 1a, clarify in the TR, that controllability (besides visibility) can be achieved as part of an SLA, and that RAN2 does not need to discuss solutions for NW controllability and visibility.

Proposal 7	With Option 1b), it is possible for the MNO to control (e.g. traffic management, QoS policies, admission control, etc) the UP traffic associated to the data collection, as it would do for any other UP service injected into the 3GPP network. Details on how to achieve controllability are up to SA2.

Proposal 8	With Option 1b), it is possible for the MNO to decide whether to have full/partial or no visibility of the UP traffic associated to the data collection. Details on how to achieve visibility are up to SA2.

Proposal 9	In option 2, to achieve controllability, the first termination entity (a CN node) should be able to interact with the “server for data collection for UE-side model training” (for the case of final termination entity inside the MNO) and with the OTT server (for the case of final termination entity outside the MNO). Details up to SA2.

Proposal 10	The feasibility of a NAS-based solution for the option 2 should be evaluated by SA2. RAN2 does not preclude a UP-based solution.

Proposal 11	For option 3, RAN2 does not preclude a UP-based solution. SA2/SA5 to evaluate feasibility.

Proposal 12	In option 3, if the training entity is outside the OAM, the OAM should be able to interact with the “server for data collection for UE-side model training” (for the case of final termination entity inside the MNO) and with the OTT server (for the case of final termination entity outside the MNO). Details up to SA2/SA5.

Proposal 13	Related to the SI objectives of model-based LCM and model transfer/deliver, no changes to TR were considered by RAN2 during the Rel.19 SI discussion. Conclusions are up to RAN1.

Proposal 14	Agree the TP to TR 38.843 in the Annex 1.

# Annex: Text Proposal to TR 38.843

NEXT CHANGE

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). According to RAN2 there might be 3GPP specification impact.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. According to RAN2 there might be 3GPP specification impact.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. According to RAN2 there might be 3GPP specification impact.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that for the different options, the data content visibility was discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility could be achieved via SLA (Service Level Agreement), as well as controllability. However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| Final termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training; or OTT server | Server for data collection for UE-side model training; or OTT server | OAM; or Server for data collection for UE-side model training; or OTT server |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) Feasibility analysis of UP tunnel not in RAN2 scope | CP tunnel (provided that the data volume remains within the RRC signalling capacity) Feasibility analysis of UP tunnel not in RAN2 scope |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel Feasibility analysis of UP tunnel not in RAN2 scope | RRC layer for CP tunnel Feasibility analysis of UP tunnel not in RAN2 scope |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Full controllability can be achieved. (Note 1) | Full controllability can be achieved (Note 1) | Full controllability can be achieved (Note 1) |
| Solution for network controllability | N/A (no AI/ML specific controllability) | Example: per PDU sessions | Via NAS procedure Feasibility analysis of NAS not in RAN2 scope | Via RRC procedure Feasibility analysis of UP tunnel not in RAN2 scope |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility (Note 3) | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. How to achieve full/partial visibility not in RAN2 scope. (Note 3) | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. How to achieve full/partial visibility not in RAN2 scope. (Note 3) | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. How to achieve full/partial visibility with the UP tunnel not in RAN2 scope (Note 3) |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage the data collection procedure of the UE, and the data transfer from the UE to the server for UE-side data collection. This includes initiating, terminating, and fully managing the data collection procedure of the UE and the transfer of collected data from the UE to the server for UE-side data collection. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Interaction between the first termination entity and the final termination entity is not in the scope of RAN2 discussion. |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.