3GPP TSG-RAN WG2 #127	R2-2407251

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

During the last RAN2#126 meeting and the email discussion [POST126][032], RAN2 has progress on the topic of LCM, in particular on the inference discussion. However, many LCM aspect still need to be discussed or clarified.

In this contribution, we share our views on the following LCM aspects:

- UE capability reporting

- Applicability reporting: signalling impact of proactive and reactive approaches

- AIML functionality activation/deactivation/fallback

- Performance monitoring

- Triggering of data collection for training, and resource configuration for training.

# 2	Discussion

The following agreement were reached during RAN2#126:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

In this contribution we will elaborate on the above agreements addressing the FFS, and taking into account the outcome of the email discussion [POST126][032].

## 2.1	UE capability reporting

For UE capability reporting, it has been agreed in RAN2#125bis meeting that supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability, and from RAN2 point of view, the conventional framework of UECapabilityEnquiry/UECapabilityInformation can be used for the UE to indicate to the network the AIML supported functionalities.

The UECapabilityEnquiry/UECapabilityInformation framework is used by the UE for the reporting of the AIML supported functionalities.

## 2.2	Applicability reporting: Proactive/reactive approaches

In RAN2#126 meeting, it has been agreed that UE determines whether a functionality is applicable as a baseline for BM use case, and the existing UAI framework is used at least for proactive reporting of applicable functionality, and reactive reporting of applicable functionality is FFS.

Agreements - For BM use case, as a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive.

The related agreements in RAN2#125bis meeting for reactive reporting of applicable functionality include

Agreements for positioning and beam management - Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality. - FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

We would like to first stress the difference between the proactive and reactive approach. To this end, we take the flow chart diagrams proposed in the email discussion [POST126][032] as baseline.

Figure 1: Reactive approach (left), proactive approach (right)

The proactive reporting would instead imply the UE proactively indicating the need for a change in the radio measurements (e.g., a new set of A/B resources) to keep the AI/ML functionality applicable or simply to indicate whether a functionality that previously was considered applicable become inapplicable or the other way around. The UE may for example indicate which recommended resources based on the NW-side additional conditions the gNB should activated in order to make the AI/ML functionality applicable. This is in line with the spirit of UEAssistanceInformation, according to which the UE can indicate to the gNB its preferred/recommended configuration for different use cases (e.g. DRX configuration, MIMO configuration, etc.). Or simply the UE can indicate the “inapplicability” of an AI/ML functionality that was previously in use. However, in the proactive approach, the step-5 is assumed to be always needed. The UE may indicate in the step-4 that an AIML functionality is applicable (based on NW-/UE-side additional conditions, model availability etc.), however this does not imply that the AIML model/functionality is already configured by the gNB. Rather, the gNB based on the received UEAssistanceInformation (step-4) may decide whether to configure the AIML functionality, and in such case the necessary inference configurations.

The proactive applicability reporting consists of:

  - Step-3: UE receives information from the gNB to evaluate the applicability status of certain AIML functionality(ies). Such information may include the NW-side additional conditions and the AIML functionality(ies) to be evaluated by the UE.

  - Step-4: Based on the information received in the step-3, on the UE additional conditions, on the AIML model(s) availability, the UE evaluates the applicability status (applicable/inapplicable) of the AIML functionality(ies) and possibly recommendations to the gNB (e.g. the recommended set(s) A/B). The results of such evaluation are finally transmitted to the gNB.

  - Step-5: Based on the information received in the step-4, the gNB may configure the UE with an AIML inference configuration for certain AIML functionality(ies)

On the other hand, in the reactive reporting the gNB can provide the UE with an AI/ML-related resource configuration for the inference (e.g., including for example a CSI measurement configuration with the set(s) A and/or set(s) B, the NW-side additional conditions, etc), and then the UE reacts to it. With this approach, the gNB can signal already in the step-3 which is the CSI measurement configuration for AIML that the gNB can support, so that no further configuration in step-5 is needed. The UE could then simply indicate in the step-4 whether the AI/ML functionality is applicable or not applicable, given the provided configuration. 
Hence in the reactive reporting, as depicted in the figure above (left side), the step 5 may not be needed. The UE can be configured directly in the step-3 with the necessary inference configuration and thus it can activate the AIML model directly after step-3, if the AIML functionality is applicable based on the provided configuration. The step-5 can just be used in case the gNB needs to transmit an update of the AIML inference configuration previously provided in the step-3.

The reactive applicability reporting consists of:

  - Step-3: The gNB configures the UE with the AI/ML inference configuration for certain AIML functionality(ies) (e.g., indicating the set(s) A and/or set(s) B, the NW-side additional conditions, etc).

  - Step-4: Based on the configuration received in the step-3, on the UE additional conditions, on the AIML model(s) availability, the UE evaluates the applicability status (applicable/inapplicable) of the AIML functionality(ies). The results of such evaluation are finally transmitted to the gNB.

  - Step-5: This step is optional. It can be used to provide an updated AIML configuration to the UE.

Hence, given the above discussion there are clear differences between the proactive and reactive reporting, and both of them are needed because they serve different purposes. In particular, the proactive reporting allows the UE to dynamically indicate to the gNB any changes in the applicability status, e.g. an AIML functionality becoming applicable or inapplicable, due to changing radio configurations, UE-side conditions, model availability. This feature is by definition not considered by just leveraging on the reactive approach. On the other hand, in the reactive approach, the UE has the possibility to be configured with the AIML inference configuration straight-away without a further reconfiguration (step-5) which instead is always required in the proactive approach. As already mentioned, the proactive approach just allows the UE to know a set of information, e.g. the NW-side additional conditions, such that on the basis of that, the UE can evaluate the (in)applicability of the AIML functionality(ies), and possibly report to the gNB a recommended/preferred configuration. This does not imply that the UE can already perform the AIML inference based on an applicable AIML model, because first the UE has to receive an AIML inference configuration (comprising for example the CSI measurement configurations with setA/B) from the gNB in the step-5. For example, if based on the NW-side additional conditions received in the step-3, the UE first evaluate that an AIML functionality is not applicable, and then later the UE evaluates that this AIML functionality is applicable (e.g. based on changed UE-side conditions or model availability), then the UE can inform in step-4 (via UAI) the gNB, but then it would be up to the gNB implementation to configure the AIML inference in the step-5.

With the proactive reporting, the UE indicates to the gNB the applicability status (applicable/inapplicable) of an AIML functionality, or a change in the applicability status, and possibly further information (e.g. recommended set A/B of resources that should be configured) to aid the gNB-provided AIML inference configuration in the step-5.

In the proactive reporting, the UE always needs to receive an AIML inference configuration in the step-5 for the applicable AIML functionality, before it can activate the AIML functionality.

With the reactive reporting, the step-3 already conveys an AIML inference configuration related to an AIML functionality, and the UE can apply the AIML inference configuration directly without the step-5 (if the AIML functionality is applicable based on the received AIML inference configuration).

In the email discussion [POST126][032], there was also an attempt to discuss in which message to include the above information in the step-3 and step-4. For the proactive approach, it is quite clear that the UEAssistanceInformation is very suitable, because it is designed to allow the UE to report recommendations/preferences dynamically/spontaneously to the gNB. Hence the otherConfig IE which is used in legacy to configure the UEAssistanceInformation should be used for the UAI configuration (as proposed in the outcome of the email discussion [POST126][032]. However, whether the NW-side additional conditions, or any other configuration information such as the configured set(s) A/B should be conveyed in the otherConfig is too early to decide. That depends also on RAN1 progress, for example related to whether the CSI framework can be used for that.

RAN2 to postpone the discussion on the messages/IEs to use to transmit the information in the step-3 (e.g. where to transmit the NW-side additional conditions, the set(s) A/B, etc).

## 2.3	Functionality activation/deactivation

In RAN2#126 meeting, it has been agreed to support functionality activation/deactivation after inference configuration, and further study initial state of configuration, how to achieve activation/deactivation and deactivation behaviors.

Agreements - RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc.

In our view, as already discussed above, in the reactive approach, the UE can apply the AIML inference configuration already in the step-3, if an AIML functionality is applicable based on such AIML inference configuration. Hence, the UE can consider the AIML functionality also active already after the step-3.

The UE can activate an applicable AIML functionality upon reception of an AIML inference configuration received either in the step-3 or step-5.

In any case, it has to be possible for the network to activate/deactivate dynamically the configured AIML functionality, without deconfiguring it. That is for example because the gNB may perform real-time monitoring of the AI/ML performances, or because the gNB may take decisions at scheduling level for which radio measurements predictions may or may not be needed. Hence, the possibility to dynamically activate/deactivate the AIML functionality via MAC signalling is a useful feature. Upon deactivation, the UE may fallback to a fallback configuration.

The gNB can dynamically activate/deactivate a configured AIML functionality via MAC signalling.

Upon AIML functionality deactivation, the UE may fallback to a “fallback configuration”.

## 2.4	Performance monitoring

This step is needed for the gNB to determine whether the outcome of AI/ML radio measurement predictions are reliable. On this, we note that the UE can independently monitor its own system/link level performance and it can detect or predict potential performance degradation. The UE may report performance results of functionality monitoring to the NW, e.g., accuracy, or other performance metrics.

In our view, which specific layer (layer-1/2/3) should be adopted to report this information may depend on the type of information, and on the amount of information to be reported, and on when this information should be reported. Since all these issues seem to be more in the RAN1 domain, and the impact in RAN2 protocols might be limited, we suggest letting RAN1 drive this topic.

RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

## 2.5	Training

RAN2 has mainly focus on the impact of the AIML inference so far, and we have not discussed yet how the training would impact RAN2 protocols. Training is a fundamental LCM functionality that we should start discussing.

First, there should be a mechanism that allows the UE to inform the gNB about the need to start (or stop) data collection for model training. This step is important because whether an AI/ML model needs to be trained is a decision completely in the realm of the UE (i.e., of the UE training entity to be more specific), and the UE may need assistance from the network to carry out a proper data collection for training. For example, the gNB may need to temporarily provide extra reference signals (besides the ones configured for conventional radio measurements), so that the UE can build a robust AIML model. The applicability reporting, e.g. in UAI, could be used to convey this information: the UE could for example indicate in the UAI that the AIML functionality is not applicable and training is needed.

Second, there should be signalling enabling the configuration of radio resources for training that should coexist with the legacy radio measurement configuration (used for conventional “real time” measurement reports). For example, the gNB may need to provide a specific measurement configuration (CSI-RS/SSBs) for the UE to perform data collection for training and for the related reporting (since these measurements would be needed only for the purpose of UE data collection, the measurement results may also be not reported at all to the gNB).

The UE can be allowed by the gNB to request resources for data collection for UE-side training.

If allowed, the UE can inform the gNB about the need to start/stop a data collection for UE-side model training. FFS how to signal this information.

RAN2 to discuss how to configure radio measurements resources and radio measurement reports for data collection for UE-side training.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UECapabilityEnquiry/UECapabilityInformation framework is used by the UE for the reporting of the AIML supported functionalities.

Proposal 2	The proactive applicability reporting consists of:

a.	Step-3: UE receives information from the gNB to evaluate the applicability status of certain AIML functionality(ies). Such information may include the NW-side additional conditions and the AIML functionality(ies) to be evaluated by the UE.

b.	Step-4: Based on the information received in the step-3, on the UE additional conditions, on the AIML model(s) availability, the UE evaluates the applicability status (applicable/inapplicable) of the AIML functionality(ies) and possibly recommendations to the gNB (e.g. the recommended set(s) A/B). The results of such evaluation are finally transmitted to the gNB.

c.	Step-5: Based on the information received in the step-4, the gNB may configure the UE with an AIML inference configuration for certain AIML functionality(ies)

Proposal 3	The reactive applicability reporting consists of:

a.	Step-3: The gNB configures the UE with the AI/ML inference configuration for certain AIML functionality(ies) (e.g., indicating the set(s) A and/or set(s) B, the NW-side additional conditions, etc).

b.	Step-4: Based on the configuration received in the step-3, on the UE additional conditions, on the AIML model(s) availability, the UE evaluates the applicability status (applicable/inapplicable) of the AIML functionality(ies). The results of such evaluation are finally transmitted to the gNB.

c.	Step-5: This step is optional. It can be used to provide an updated AIML configuration to the UE.

Proposal 4	With the proactive reporting, the UE indicates to the gNB the applicability status (applicable/inapplicable) of an AIML functionality, or a change in the applicability status, and possibly further information (e.g. recommended set A/B of resources that should be configured) to aid the gNB-provided AIML inference configuration in the step-5.

Proposal 5	In the proactive reporting, the UE always needs to receive an AIML inference configuration in the step-5 for the applicable AIML functionality, before it can activate the AIML functionality.

Proposal 6	With the reactive reporting, the step-3 already conveys an AIML inference configuration related to an AIML functionality, and the UE can apply the AIML inference configuration directly without the step-5 (if the AIML functionality is applicable based on the received AIML inference configuration).

Proposal 7	RAN2 to postpone the discussion on the messages/IEs to use to transmit the information in the step-3 (e.g. where to transmit the NW-side additional conditions, the set(s) A/B, etc).

Proposal 8	The UE can activate an applicable AIML functionality upon reception of an AIML inference configuration received either in the step-3 or step-5.

Proposal 9	The gNB can dynamically activate/deactivate a configured AIML functionality via MAC signalling.

Proposal 10	Upon AIML functionality deactivation, the UE may fallback to a “fallback configuration”.

Proposal 11	RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

Proposal 12	The UE can be allowed by the gNB to request resources for data collection for UE-side training.

Proposal 13	If allowed, the UE can inform the gNB about the need to start/stop a data collection for UE-side model training. FFS how to signal this information.

Proposal 14	RAN2 to discuss how to configure radio measurements resources and radio measurement reports for data collection for UE-side training.