3GPP TSG-RAN WG2 Meeting #127	R2-2407259

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda item:	7.6.2

Source:	Samsung

Title:	Various corrections to IoT NTN Rel-18

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

This contribution focuses on various corrections to IoT NTN Rel-18 [1].

# Stage 2 corrections

## Measuring NTN in TN

In RAN2#124 we agreed the following:

- We introduce the possibility to broadcast neighbor NTN cell information (SIBXX) in a TN cell (We remove the restriction in 36.331 that SIBXX can only be sent in a NTN cell. No changes expected to other Stage 3 specs)

The agreement highlighted that broadcasting NTN cell information is possible in a terrestrial cell, which is an important feature for TN-NTN integration. However, until now very little text has been captured to highlight this rather important feature. We propose:

Proposal 1: Agree Stage 2 text proposal in Section 5.1 on measuring NTN cells in a terrestrial network.

## Correction on not measuring NTN cells

In 36.300 18.2.0 [2], the following has been captured:

23.21.4.3	Measurements

... OMITTED ...

For a UE in idle mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is indicated in SIB3/SIB5 but without corresponding satellite information in SIB33.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is included in the measurement configuration but without corresponding satellite information in SIB33.

The text is a copy of NR NTN text that was introduced in Rel-17:

For a UE in Idle/Inactive mode it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB3/SIB4 but not included in SIB19.

For a UE in Connected mode, it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19.

However, the signalling in IoT NTN differs to NR NTN to the degree that the text is incorrect. This is because in IoT NTN the cells are not directly signalled in idle mode. In IoT NTN, satellite IDs are associated with a frequency (inter or intra-frequency). SIB3/5 may or may not have specific cell IDs signalled but these are not tied to specific satellites in SIB33.

Observation 1: In idle mode IoT NTN, NTN cells are not directly signalled.

If the feature is to be applicable to IoT NTN, then there are two options:

1. If none of the satellite IDs in SIB3/SIB5 are present in SIB33 for a specific frequency, a UE does not need to measure the frequency. This is less likely to occur, but still useful.

2. If a satellite ID is present in SIB3/SIB5 but not in SIB33, the UE does not need to use the corresponding satellite assistance information to detect neighbouring cells.

For connected mode, satellite IDs are tied to specific cells, which means that the current text in Stage 2 is correct.

Proposal 2: If none of the satellite IDs for a specific frequency in SIB3/SIB5 are present in SIB33, it is up to UE implementation to measure the frequency.

Proposal 3: Agree Stage 2 correction text.

# Capabilities

## Connected mode measurements using NTN assistance information

In Rel-17, connected mode neighbour cell measurements were essentially left to UE implementation. This is because NTN assistance information, such as ephemeris and TA common was not introduced. The agreement in RAN2#120 was:

Agreements:

- RAN2 doesn’t introduce neighbour cell ephemeris in Rel-17 IoT-NTN, neither for eMTC not for NB-IoT. RAN2 agrees to support this in Rel-18, with details FFS.

In Rel-18, IoT NTN has now introduced a new feature to provide NTN assistance information in SIB33 for measuring neighbouring cells in RRC idle and RRC connected. For connected mode RRM measurements, the UE can now be configured with a satellite ID which configures the UE with NTN assistance information. This needs to be indicated as part of UE capability in order for network to know whether this is configurable or not.

Proposal 4: Introduce non-signalled UE capability for a UE to measure an NTN cell using NTN assistance information in idle mode.

Proposal 5: Introduce a signalled UE capability for a UE to measure an NTN cell using NTN assistance information in connected mode.

## Measuring NTN from a terrestrial network

In RAN2#124, the following was agreed:

- We introduce the possibility to broadcast neighbor NTN cell information (SIBXX) in a TN cell (We remove the restriction in 36.331 that SIBXX can only be sent in a NTN cell. No changes expected to other Stage 3 specs)

For acquiring SIB33 in a terrestrial, a first important question is whether this implies that a UE is able to measure an NTN cell when the UE is in a terrestrial cell. On one hand, TN to NTN mobility is supported in Release 17, as evidenced by a number of corrections [3][4], but there is no NTN assistance information provided to perform the idle, inactive or connected mode measurements. One question is whether the new feature implies that a UE is capable of measuring an NTN cell in a terrestrial cell. We think that the feature to provide NTN assistance information in a terrestrial cell implies that a UE is capable of measuring an NTN cell in a terrestrial cell using the NTN assistance information.

Proposal 6: The UE feature to acquire SIB33 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

As discussed above, IoT NTN release 18 is the first release to introduce NTN assistance information for measuring NTN cells. This means that there may not be a need to have specific UE capability to indicate that the UE is capable of measuring NTN cells while in a TN cell in addition to capabilities to indicate that the UE is capable of measuring NTN cells while in an NTN cell (as proposed by P4 and P5 above). Thus we think that the capabilities to measure an NTN cell using NTN assistance information implies that the UE is capable of this both in an NTN cell and in a TN cell.

Proposal 7: If P4-P5 is agreed, the capabilities indicate capability to measure NTN cell both in an NTN cell and a terrestrial cell.

# RRC corrections

The field satelliteID was introduced in measObjectEUTRA as part of cellsToAddModList in 36.331 V18.1.0. This was introduced in order to allow for an association between an NTN satellite assistance information element in SIB33 and a cell that is configured to be measured in the measurement object. This is key to allow for neighbour cell measurements in IoT NTN, which in turn is an absolutely vital feature for cellular networks. One of the benefits of this is that a UE may be configured with ephemeris in a rather lightweight manner, as opposed to directly configuring dedicated ephemeris in a measurement object, and there is no need to constantly update the dedicated ephemeris, as the satellite ID is associated with SIB33, which the UE may acquire during T318.

As part of enhancements for handover event D2 and conditional handover event D2, it was agreed that ephemeris info, epoch time as well as reference location can be signalled per cell. However, it has yet to be agreed that dedicated ephemeris may be configured for the purpose of connected mode RRM measurements. This was introduced for NR NTN, but we believe that for IoT NTN the conditions are different. First of all, configuring ephemeris info for UEs may represent some significant overhead as each ephemeris element is very large. Second of all, the current maximum number of ephemeris elements that a UE can be configured to measure is currently limited to the size of NeighSatelliteInfoList in SIB33. This is currently limited to 4. If on top of this, a UE is configured with additional ephemeris, then there may need to be discussion on the maximum number of ephemeris that a UE may be configured to monitor in connected mode. This would have to involve RAN4. Measuring an NTN cell is expected to be a lot more difficult than a terrestrial cell given that UE has to synchronize to a large frequency offset as well as synchronize in time. It would for instance not be reasonable to configure an eMTC UE with 6 different ephemeris elements to measure.

Observation 2: If dedicated ephemeris may be signalled in Measurement object, there needs to be a discussion on the maximum number of ephemeris elements that can be configured to a UE.

Therefore we think that for IoT NTN it would be sufficient to only rely on satellite ID, which already has a maximum number elements of 4.

Proposal 8: Dedicated ephemeris element in measurement object is not allowed for connected mode RRM in IoT NTN. Capture this in 36.331.

# Idle mode corrections

In Release 17, t-Service was introduced to ensure that UEs shall measure on neighbouring cells before a quasi-earth fixed satellite moves from an area. This is to prevent the case where the satellite suddenly stops serving an area, and the UE has not performed adequate measurements in anticipation of this. The parameter t-Service thus roughly gives a time when the satellite will stop the area, and the UE shall then perform measurements regardless of any fulfilled relaxation criteria.

In release 18 IoT NTN, a new relaxation criteria was added, where if the distance from the UE to a reference location is smaller than a threshold, the UE allowed to not measure.

In NR NTN, when t-Service is triggered, the UE shall disregard the distance-based measurement relaxation and perform intra-frequency, inter-frequency and inter-RAT measurements:

------------------- Based on 38.304 V18.2.0 -------------------

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

------------------- Based on 38.304 V18.2.0 -------------------

In IoT NTN, the UE such text has not been added to 36.304. This means that the when t-Service is triggered, the UE shall not disregard the distance-based measurement relaxation:

------------------- Based on 36.304 V18.2.0 -------------------

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-Service is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

------------------- Based on 36.304 V18.2.0 -------------------

We think that IoT NTN shall also disregard the distance-based measurement relaxation and perform intra-frequency, inter-frequency and inter-RAT measurements.

Proposal 9: IoT NTN UE shall disregard NTN distance-based measurement when t-Service is triggered, as in NR NTN.

Proposal 10: Agree 36.304 correction text.

s

# Conclusion

In this contribution we have the following observations and proposals:

Observation 1: In idle mode IoT NTN, NTN cells are not directly signalled.

Observation 2: If dedicated ephemeris may be signalled in Measurement object, there needs to be a discussion on the maximum number of ephemeris elements that can be configured to a UE.

Proposal 1: Agree Stage 2 text proposal in Section 5.1 on measuring NTN cells in a terrestrial network.

Proposal 2: If none of the satellite IDs for a specific frequency in SIB3/SIB5 are present in SIB33, it is up to UE implementation to measure the frequency.

Proposal 3: Agree Stage 2 correction text proposal.

Proposal 4: Introduce non-signalled UE capability for a UE to measure an NTN cell using NTN assistance information in idle mode.

Proposal 5: Introduce a signalled UE capability for a UE to measure an NTN cell using NTN assistance information in connected mode.

Proposal 6: The UE feature to acquire SIB33 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

Proposal 7: If P4-P5 is agreed, the capabilities indicate capability to measure NTN cell both in an NTN cell and a terrestrial cell.

Proposal 8: Dedicated ephemeris element in measurement object is not allowed for connected mode RRM in IoT NTN. Capture this in 36.331.

Proposal 9: IoT NTN UE shall disregard NTN distance-based measurement when t-Service is triggered, as in NR NTN.

Proposal 10: Agree 36.304 correction text proposal.

# Reference

RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022

3GPP TS 36.300 “Overall description; Stage 2”, V18.2.0, June 2024.

R2-2308985, RRC Correction on including GNSS validity duration and dedicated SIB31, Samsung, RAN2#123, August 2023.

R2-2313789, Correction on SIB31 signalling only in NTN cell, Samsung, RAN2#124, November 2023.

# Text proposal

## Stage 2

------------------- Based on 36.300 V18.2.0 -------------------

7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters). It can also contain information about E-UTRA and NR idle/inactive measurements;

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency), which can also be used for NR idle/inactive measurements;

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockType26a contains information related to NR bands list which can be used for EN-DC operation with the serving cell;

-	SystemInformationBlockType27 contains assistance information for inter-RAT cell selection to NB-IoT;

-	SystemInformationBlockType28 contains information related to NR sidelink communication;

-	SystemInformationBlockType29 contains information related to common resource reservation;

-	SystemInformationBlockType30 contains information related to disaster roaming;

-	SystemInformationBlockType31 contains information required for accessing an NTN cell;

-	SystemInformationBlockType32 contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33 contains assistance information for neighbouring NTN cells for mobility to an NTN cell.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockType27-NB contains assistance information for inter-RAT cell selection to E-UTRAN and/or GERAN;

-	SystemInformationBlockType31-NB contains information required for accessing an NTN cell;

-	SystemInformationBlockType32-NB contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33-NB contains assistance information for neighbouring NTN cells for mobility to an NTN cell.

OMITTED

23.21.4.3	Measurements

The principles described in clause 10.1.3 apply in NTN unless specified otherwise.

To enable measurements, the network may provide neighbouring cell assistance information via system information. Neighbour cell assistance information may also be provided via system information to enable TN to NTN mobility.

The following can optionally be used for measurements on neighbour cells in RRC_IDLE as specified in TS 36.331 [16]:

-	The timing and location information associated to the serving cell provided in SIB3 and SIB31;

-	Timing information when the neighbour cell starts serving the current geographical area;

-	Location information refers:

-	In the quasi-Earth fixed cell scenario, to the reference location of the serving cell and a distance threshold to the reference location;

-	In the Earth moving cell scenario, to the reference location of the serving cell at the epoch time and a distance threshold to the reference location.

The following measurement triggers can be configured in RRC_CONNECTED for the purpose of RRC reestablishment or handover as specified in TS 36.331 [16]:

-	A time-based trigger condition;

-	A location-based trigger condition.

The time-based measurement initiation may be applicable for the feeder link switchover case for cell (re)selection.

For a UE in idle mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a frequency which is indicated in SIB3/SIB5 but without corresponding satellite information in SIB33.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is included in the measurement configuration but without corresponding satellite information in SIB33.

------------------- Based on 36.300 V18.2.0 -------------------

## Idle mode

------------------- Based on 36.304 V18.2.0 -------------------

5.2.4.2		Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation (quasi-)Earth for fixed cell, referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to TS 36.133 [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SnonIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location. The referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE supports location-based measurement initiation and has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-Service regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-Service is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

5.2.4.2a		Measurement rules for cell re-selection for NB-IoT

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlock31-NB, and if the UE has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and the UE supports location-based measurement initiation for Earth moving cell the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NB-IoT inter-frequencies which are indicated in system information:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlock31-NB, and if the UE supports location-based measurement initiation and has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between UE and serving cell location is shorter than distanceThresh, the UE may choose not to perform inter-frequency measurements.

-	Else, the UE shall perform inter-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and the UE supports location-based measurement initiation for Earth moving cell the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform inter-frequency measurements.

-	Else, the UE shall perform inter-frequency measurements.

-	Else, the UE may choose not to perform inter-frequency measurements.

-	Else, the UE may choose not to perform inter-frequency measurements.

-	Otherwise, the UE shall perform inter-frequency measurements.

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3-NB, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE shall perform intra-frequency or inter-frequency measurements before the time t-Service regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP or Srxlev > SnonIntraSearchP. The exact time to start measurements before t-Service is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33-NB may be used to decide on when to start measurements.

------------------- Based on 36.304 V18.2.0 -------------------