3GPP TSG-RAN WG2 Meeting #127	R2-2407255

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.7.2

Source:	Samsung

Title:	Miscellaneous corrections

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide miscellaneous corrections and discussions.

# Discussion

## PUCCH repetition for Msg4 HARQ ACK

According to the RAN1 agreement and RRC parameters list [1], for PUCCH repetition for Msg4 HARQ-ACK, a RSRP threshold can be configured.

The parameter numberOfPUCCHforMsg4HARQACK-RepetitionsList is configured to enable this feature. And a UE capable of PUCCH repetition of Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK in the following cases:

- if numberOfPUCCHforMsg4HARQACK-RepetitionsList is configured but rsrp-ThresholdPUCCHforMsg4HARQACK is not, or;

- if numberOfPUCCHforMsg4HARQACK-RepetitionsList and rsrp-ThresholdPUCCHforMsg4HARQACK are configured, and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdPUCCHforMsg4HARQACK"

However, the current NOTE 3 in Table 6.2.1-2c in TS 38.321 does not include the first case. We propose a correction for this note.

NOTE 3:	For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured, and if rsrp-ThresholdMsg4HARQ-ACK is configured, the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK.

Proposal 1: Modify NOTE 3 in Table 6.2.1-2c in TS 38.321 as follows.

NOTE 3:	For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK.

## Satellite switch with resync

In RAN2#124, we have agreed that UE in idle/inactive mode can by implementation use the information of satellite switch with resynchronization to switch to the new satellite.

In satellite switch with resynchronization, the target satellite is going to provide service for the same cell and same coverage. Based on the broadcast information, idle/inactive UE can stay camping on the same cell with the new satellite, thus possibly skip measurement on neighbor cells before t-Service by UE implementation.

However, skipping measurement before t-Service is not allowed by the current specification. In the current specification, UE shall always perform measurement on neighbor cells before t-Service regardless of other measurement rules, which is designed as time-based measurement initiation.

To allow idle/inactive UE to utilize the information of satellite switch with resynchronization, the time-based measurement initiation should be relaxed for the case of satellite switch, meaning if the t-Service of the serving cell is present and SatSwitchWithReSync is absent in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of other measurement rules for cell reselection.

Proposal 2-1: Allow idle/inactive UE capable of time-based measurement initiation to skip neighbor measurement before t-Service if the current serving cell broadcasts satellite switch with resynchronization.

Proposal 2-2: If Proposal 2-1 is agreed, adopt the TP below.

If the t-Service of the serving cell is present and satSwitchWithReSync is absent in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

## Dedicated NTN assistance info for connected mode RRM

Connected mode neighbour NTN cell measurements can already be configured in Release 17. In release 17 the UE uses the frequency of a configured measurement object and associates it to the list of NTN-Config in SIB19. Connected mode RRM measurement in NTN is a mandatory feature of release 17 NR NTN.

In RAN2#126 the following was agreed:

R2-2405079	Clarification on (cond)event D2 configuration	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: It is proposed to reject second changes ( i.e., optionally present part) proposed in RIL 302.

-	QC agrees with the analysis of how this was implemented so far but sees the benefit of leaving this optional

-	Nokia supports Ericsson and QC views

-	Samsung is ok with E302 but not as an alignement to IoT NTN

E302 is agreed

E302 is as follows:

| E302 | Ericsson (Ignacio) | NTN | 1 | PropAgree | It will be addressed in the Rapp input CR | The present text precludes network to include the field for a different type of cell, e.g., quasi-Earth fixed cell. This is useful when network wants to configure measurements for a neighbor cell whose satellite assistance information is not present in SIB19. For instance, the cell could belong to a different shell in the satellite constellation. | Align the text with IoT NTN and make it optionally present when it is not mandatory. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Furthermore this was introduced 38.331 18.2.0 follows:

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

. . . OMITTED . . .

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18    OPTIONAL -- Cond NeighbourCell

}

. . . OMITTED . . .

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17            OPTIONAL   -- Need R

}

This new feature allows dedicated ephemeris to be introduced to RRM purposes. In order for the network to know whether to configure a dedicated ephemeris to a UE, the network needs to know whether the UE has implemented handling the dedicated ephemeris. Otherwise the UE will not be able to parse the ASN1. In other words, there needs to be a new UE capability to indicate whether the UE supports dedicated ephemeris. This is also evident from the fact that this is not introduced in the first release of NTN.

Proposal 3: Introduce a capability to indicate UE support of dedicated ephemeris and epoch time in measurement object configuration for connected mode RRM measurements.

In 38.300 Stage 2, the following has been captured on how a UE may relax idle, inactive and connected mode measurements:

For a UE in idle/inactive mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB3/SIB4 but not included in SIB19.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19.

For connected mode, this means that a UE may for instance be allowed to not perform NTN neighbour cell measurements on a cell under the condition that the cell is configured in the measurement object, but not broadcasted in SIB19.

This will need to change now that Rel-18 has introduced dedicated ephemeris and epoch time in the measurement object. This is because the point of the dedicated ephemeris and epoch time is to enable measurements of cells that are not indicate in SIB19. The relaxation should thus be allowed if the cell is not broadcasted in SIB19 and that there is no dedicated NTN assistance information.

Proposal 4-1: For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but without corresponding satellite information in the measurement configuration or in SIB19.

Proposal 4-2: For proposal 4-1, adopt TP as follows:

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but without corresponding satellite information in measurement configuration or in SIB19.

## Capability for acquiring SIB19 and measuring NTN in TN cell

In RAN2#124, the following was agreed

Agreements:

- SIB19 can be broadcast in TN cells to provide satellite assistance information for NTN neighbour cells (e.g., ntn-NeighCellConfigList-r17).

Acquiring SIB19 in a terrestrial cell is a new feature for Release 18, which is not available in Release 17. A first important question is whether this implies that a UE is able to measure an NTN cell when the UE is in a terrestrial cell. On one hand, TN to NTN mobility is supported in Release 17, but there is no NTN assistance information provided to perform the idle, inactive or connected mode measurements. Furthermore, there are no capabilities specifically for TN to NTN measurements. Thus a first question is whether the new feature implies that a UE is capable of measuring an NTN cell in a terrestrial cell. We think that the feature to provide NTN assistance information in a terrestrial cell implies that a UE is capable of measuring an NTN cell in a terrestrial cell using the NTN assistance information.

Proposal 5: The UE feature to acquire SIB19 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

This ability while expected to not be very complicated for an NTN-capable UE, it is still a new feature not available for Release 17 UEs. Thus we believe that this feature needs to have a capability. The question is whether the feature is a non-signalled capability or a signalled capability. Idle or inactive mode features are mostly non-signalled capabilities, while connected mode features need to be signalled capabilities. For connected mode, it would be beneficial to know whether the UE is capable of measuring an NTN cell using the NTN assistance information.

Proposal 6: A new non-signalled UE capability to receive SIB19 in TN cell in idle/inactive mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

Proposal 7: A new signalled UE capability to receive SIB19 in TN cell in connected mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

# Conclusion

In this contribution we have the following proposals.

Proposal 1: Modify NOTE 3 in Table 6.2.1-2c in TS 38.321 as follows.

NOTE 3:	For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK.

Proposal 2-1: Allow idle/inactive UE capable of time-based measurement initiation to skip neighbor cell measurement before t-Service if the current serving cell broadcasts satellite switch with resynchronization.

Proposal 2-2: If Proposal 2-1 is agreed, adopt the TP below.

If the t-Service of the serving cell is present and satSwitchWithReSync is absent in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

Proposal 3: Introduce a capability to indicate UE support of dedicated ephemeris and epoch time in measurement object configuration for connected mode RRM measurements.

Proposal 4-1: For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but without corresponding satellite information in the measurement configuration or in SIB19.

Proposal 4-2: For proposal 4-1, adopt TP as follows:

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but without corresponding satellite information in measurement configuration or in SIB19.

Proposal 5: The UE feature to acquire SIB19 in a TN cell indicates that UE is capable of performing measurements of NTN cells using NTN assistance information in a terrestrial cell.

Proposal 6: A new non-signalled UE capability to receive SIB19 in TN cell in idle/inactive mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

Proposal 7: A new signalled UE capability to receive SIB19 in TN cell in connected mode (i.e UE capable of measuring NTN cell using NTN assistance information in TN cell) is introduced.

# Reference

R2-2313912, LS on Rel-18 higher-layers parameter list.

3GPP, TS 38.321 v18.2.0.

3GPP, TS 38.304 v18.2.0.