3GPP TSG-RAN WG2 Meeting #127	R2-2407257

Maastricht, Netherlands, 19th May – 23rd August, 2024

Agenda item:	8.9.2

Source:	Samsung

Title:	On SA2 progress and RAN2 aspects on Store and Forward

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#103 the latest WID for IoT NTN Phase 3 is provided [1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we provide some discussions related to SA2 progress and RAN2 aspects on Store and Forward.

# Discussion

## Basic scenario

- The basic assumption of baseline IoT NTN and NR NTN is that full radio and core access is always provided via the transparent satellite. As an enhancement in IoT NTN Rel-18, methods were also specified whereby access (via NTN) would not always be provided – so-called discontinuous coverage. The methods allow a UE to not waste power to perform idle mode procedures when there is no coverage provided by any satellites. This effectively allows a satellite network with a smaller amount of satellites than what would otherwise be needed to provide continuous coverage. As an extreme example, only a single LEO satellite could potentially provide service, versus the 66 LEO satellites that is generally quoted as the minimum number that would be required in a LEO constellation at 600 km altitude.

- However, even for discontinuous coverage it is assumed that there are enough ground gateways, i.e eNBs on the ground that can provide satellite coverage. In Store and Forward, it is assumed that there may not be enough ground gateways, but that the satellite shall still provide connectivity.

Figure 1. Different types of NTNs.

- From the above it should be clear that a UE, and by extension a service, operating in an S&F network may have to cope with very large delays. In a discontinuous coverage network, a service may also experience large delays. The difference between a discontinuous coverage network and a S&F network is that the delay when the UE has successfully accessed the network will be a lot higher in the S&F network.

Observation 1: The E2E delay in an S&F network may be very large.

## SA2 progress

- SA2 has made conclusions on Store and Forward. Some notable conclusions are:

- SA2 have determined two architecture assumptions. The architecture chosen is up to the deployment.

  - Split MME architecture. In split MME architecture, as the name implies, the eNB and part of the MME will be on the satellite. The MME is split in to MME-onboard and MME-ground. The interface between MME-onboard and MME-ground. MO data is stored in MME-onboard when either service or feeder link is unavailable.

  - Full CN onboard. This includes the full core network and RAN such as eNB, MME, SGW, PGW, HSS, E-SMLC and SMSC.

- eNB broadcasts support of S&F operation as part of system information

- UE indicates support for S&F operation as part of NAS capability

- A time to wait as well as optionally a list of satellite IDs where the UE may re-attempt attach, TAU procedure, exchange signalling or send data to. The satellite IDs are based on SIB information broadcasted by the eNB.

## Barring UEs

In last meeting the following was decided:

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

However, it has not been made clear in the SA2 conclusions whether a legacy UE shall be assumed to not be able to operate in an S&F cell. SA2 for instance has the architecture option to put the full core network in the satellite, where it may be imagined that a legacy UE may be able to operate. This is important for RAN2 as it may decide whether a potential cell barring bit shall always be present in an S&F cell, which is the case for NTN cells. This is because the existence of the bit may itself be used to indicate whether the cell is an S&F cell or not.

Proposal 1: Send LS to SA2 on whether legacy UEs can be assumed able to connect to an S&F cell in both of the S&F architecture options.

## Relation to discontinuous coverage network

In Release 17, discontinuous coverage was introduced. The enhancement to allow for discontinuous coverage are the following:

    - SIB32 indicates long term satellite ephemeris elements.

    - Enhancements to idle mode procedure where the UE is allowed to not perform any idle mode procedures related to NTN if the UE detects that there are no satellites that will provide coverage.

    - UE can indicate over NAS (using TAU procedure) when it is expected that a UE will enter discontinuous coverage [3].

As discussed above, SA2 has decided that satelliteIDs can be sent in a NAS message, which will refer to satellite assistance information in system information. A satellite ID can potentially identify satellite assistance information in SIB31 (serving cell), SIB32 and SIB33. The satellite assistance information in SIB31 and SIB33 are short-term satellite assistance information, so we believe it is unlikely that this shall be used for S&F. So it is more reasonable that the signalled satellite IDs in NAS identify long term ephemeris for discontinuous coverage in SIB32. This implies that according to SA2, discontinuous coverage and S&F are heavily intertwined.

So we believe that a UE capable of S&F can be assumed to be discontinuous coverage-capable. This also means that S&F information should be taken into account for discontinuous coverage operation.

Proposal 2: A UE capable of S&F can be assumed to be discontinuous coverage-capable.

Proposal 3: S&F information provided by NAS can be used for discontinuous coverage operation.

# Conclusion

In this contribution we discussed issues related to S&F network.

Observation 1: The E2E delay in an S&F network may be very large.

Proposal 1: Send LS to SA2 on whether legacy UEs can be assumed able to connect to an S&F cell in both of the S&F architecture options.

Proposal 2: A UE capable of S&F can be assumed to be discontinuous coverage-capable.

Proposal 3: S&F information provided by NAS can be taken into account for discontinuous coverage operation.

# Reference

RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#103, Maastrict, Netherlands, March 2024

RP-240775, Revised WID: Non-Terrestrial Networks () for NR Phase 3, Thales, CATT, RAN#103, Maastricht, Netherlands, March 2024.

3GPP TS 23.401 “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”, Release 18, V18.5.0, March 2024.

# Appendix

---------------------------- 23.700-29 ----------------------------

8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

NOTE 1:	Whether to have a full CN onboard the satellite or Split MME architecture is up to the operation deployment.

NOTE 2:	The security issues (if any) of these conclusions are in the scope of SA WG3.

NOTE 3:	The LI issues (if any) of these conclusions are in the scope SA WG3-LI.

NOTE 4:	The deployments are subject to regulatory requirements.

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles:

1)	In the Split MME architecture, HSS is on the ground.

2)	MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP.

3)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

4)	For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

With the following normative impacts:

1)	When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal.

2)	When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes:

a)	A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network.

b)	Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

c)	Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

3)	How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

4)	MME may indicate to HSS the "Request Time", allowing the HSS to check that no other (e.g. terrestrial) MME has sent an Update Location Request after the "Request Time", and fetches the authentication vector and other details from HSS following current Authentication and security procedures. The MME may triggers Update location with the HSS and Update location ACK is received by MME. i.e. all the subscription details are retrieved by MME-ground. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives the final Update Location Request.

5)	When the wait timer has expired given to the UE in step 2, if the UE has not successfully attached to another PLMN and the UE finds the cell which broadcast the Satellite ID valid to re-attempt the attach procedure, the UE re-sends the Attach or TAU Request message.

6)	During the Attach or TAU procedure with the UE, the MME may also provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges in those satellites.

7)	The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 2:	The estimated delivery time is the estimated time to send data from the UE to Gateway.

8)	The core network can indicate to external SCS/AS whether UE is registered in S&F Mode and the estimated delivery time.

NOTE 3:	Whether any existing monitoring events or procedures can be used or enhanced to achieve above will be determined during normative phase.

NOTE 4:	SA WG2 will align further based on agreements in SA WG3 e.g. on security procedures and Attach/TAU procedures.

The split MME architecture will be described in informative annex.

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles:

-	The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

-	The implementation of the proxies and the interface between them is out of 3GPP scope.

-	The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list.

NOTE 5:	MT traffic is delivered to the UE after it performs an ATTACH.

NOTE 6:	After mobility from S&F operation, MT traffic could be stuck in the ground proxy and will only be retrieved once the UE goes back to S&F operation.

For MT traffic, the UE attaches to a satellite and to allow delivery of MO traffic from the user or applications on the UE. A UE, based on implementation, could first wait for an indication from the user or from an application on the UE of pending MT traffic or could wait based on knowledge of when MT traffic may arrive, before performing the attach.

-	Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite.

-	Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network.

NOTE 7:	The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5].

With the following normative impacts:

-	Store and forward is only supported by EPS.

-	Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification.

NOTE 8:	The S&F monitoring list may assist the UE in retrieving MT data.

-	The UE needs to be aware that a satellite supports S&F mode.

NOTE 9:	How the UE is aware that a satellite supports S&F mode of operation depends on RAN.

-	A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

---------------------------- 23.700-29 ----------------------------