3GPP TSG-RAN WG2 Meeting #127	R2-2407257

Maastricht, Netherlands, 19th August – 23rd August, 2024

Agenda item:	8.9.3

Source:	Samsung

Title:	Procedures for uplink capacity enhancements for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#103 the latest WID for IoT NTN Phase 3 is provided [1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we provide some initial discussions on the objective of Support of Capacity enhancements for the uplink.

# Discussion

- Agreements:

- RAN2#125bis:

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

- RAN2#126:

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

## Background on EDT, PUR and RACH-less

- Early Data Transmission (EDT) was introduced in Release 15 for NB-IoT and LTE-M. EDT is a further enhancement to Release 13 UP/CP CIoT optimizations (RRC Resume procedures). This is because even though Rel-13 enhancements greatly decrease the number of messages needed before any data can be sent compared to before Rel-13, there are still a lot of overhead. This is especially if the data transmitted is small. EDT allows for data to be transmitted in Msg3 when UE moves from RRC idle with suspended RRC connection.

- User plane EDT is implemented by resuming the RRC connection and the security already in Msg3. Msg3 in EDT includes the RRCConnectionResumeRequest and data for DRBs as the full UP stack is resumed.

- Control plane EDT also resumes the RRC connection, but not the AS security as AS security is not supported for CP IoT. Msg3 in EDT includes the RRCEarlyDataRequest, which includes a NAS container that can contain data.

- For both cases, EDT requires the UE to signal in Msg1 that the UE is performing EDT. This is done by either configuring separate RACH opportunity for EDT, or configuring a set of preambles that EDT can use. There may be a TBS threshold configured where the UE only uses the preamble if the TBS is larger than a threshold.

- Preconfigured Uplink Resources is a feature is introduced in Release 16 to further enhance the use case of small data transmission for NB-IoT and LTE-M. In PUR, the UE does not need to perform any random access procedure. Instead the UE is dedicatedly configured with PUSCH resources when the UE is released to RRC IDLE. Thus the PUR resources are configured in RRCConnectionRelease. The UE may request PUR resources via a specific RRC message PURConfigurationRequest.

- Observation 1: EDT being RACH-based is contention-based and uses broadcasted resources and PUR is contention-less and uses dedicated configuration.

- RACH-less was introduced in Release 14 for LTE [2] and further introduced for Release 18 in NR [3]. RACH-less in LTE allows for optimized handovers and SCell establishment whereby the UE skips random access, through the network configuring dedicated PUSCH resources for the UE. The UE then directly sends the RRCConnectionReconfigurationComplete and any MAC CE to the target cell instead of performing CBRA or CFRA handovers.

- Observation 2: RACH-less is a feature to introduce optimized handover skipping the random access procedure.

## RACH-less EDT-like procedures

In RAN2#125bis meeting it was discussed whether the RACH-less EDT-like feature is only introduced for NTN or may also be applicable to terrestrial networks [3]. In our understanding, the RACH-less EDT-like feature is introduced due to the fact that the NTN UE can autonomously compensate the timing advance due to the geometry of a satellite deployment. A terrestrial network does not exhibit the same geometry, which makes it difficult for a terrestrial UE to use the same methods – when the UE moves on the surface of the earth the Euclidean distance from a UE to a point (representing the eNB) changes differently depending on the height of the point. As result the required GNSS position fix accuracy would be much higher in a terrestrial network. Thus we believe that it is not technically useful to discuss such a scenario. Furthermore it is clear that the WID is for non-terrestrial networks.

- Observation 3: The required GNSS position fix accuracy in a TN would be significantly higher compared to NTN owing to the geometry.

Proposal 1: RACH-less EDT-like procedures and any Msg4 enhancement are only introduced for NTN.

One of the key differences between EDT and PUR, is that PUR is entirely pre-configured, whereas EDT is largely not preconfigured. For PUR, this makes it difficult to deploy as in practice it is difficult for both network and UE to predict when and how large future data transmissions will be in the future, outside a few select use cases. EDT on the other hand only requires the UE to be in a suspended RRC states with security configured for UP EDT, while CP EDT does not require any pre-configuration. For RACH-less Early Data transmissions, we think it would be crucial for this feature to require minimal if not any pre-configuration.

Proposal 2: Contention-based RACH-less EDT-like procedure is not dedicatedly pre-configured, but uses broadcasted common resources.

## Other features of EDT without RACH

Contention-based RACH-less EDT-like procedures has the potential to increase the spectral efficiency, but it should be noted that it does not include the RACH component. Not including the random access procedures bring about a lot more risk as it requires that the self-pre-compensation of the TA is a lot more precise compared to when employing RACH.

The two important pieces when it comes to self-pre-compensation is accurate and recent ephemeris and UE GNSS position. Thus if we want to ensure that RACH-less Msg3 transmission can be successful, then we need to look at how to ensure that the ephemeris and the UE GNSS position is sufficiently recent. This could be enabled by ensuring that a UE has recently updated these before performing any RACH-less Msg3 transmission, or by reducing the validity durations of the ephemeris and the UE GNSS position when performing RACH-less Msg3 transmissions.

Proposal 3: RAN2 to discuss how to ensure that ephemeris and UE GNSS position is sufficiently recent to ensure that the synchronization is tighter compared to legacy NTN random access.

One key feature for eMTC and NB-IoT is the extended coverage. In early releases of the two technologies, a lot of time was spent on introducing different amounts of repetitions to all channels. This includes repetitions of all messages of the random access procedure, i.e Msg1, Msg2, Msg3 and Msg4. Similarly, depending on the deployment and capability of the satellite network, it could be valuable to introduce repetitions for RACH-less Msg3 transmissions.

Proposal 4: Introduce possibility to configure repetitions for RACH-less EDT-like transmissions.

In some circumstances, depending on the configuration, it may be expected that RACH-less EDT-like transmission would be less reliable compared to legacy random access. Failure rate can be dependent on the synchronization of the network and the UE, the radio characteristics and the load on the RACH-less Msg3 resources. Given that the legacy random access should be more reliable, if the RACH-less Msg3 fails, then a UE should be able to fall back towards legacy random access.

Proposal 5: If RACH-less Msg3 fails, it should be possible to fallback to random access with RACH.

## New random access procedures

In RAN2#126, the following agreement was taken:

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

CRDSA

Contention-Resolution Slotted Diversity Aloha is a random access technique that is based on Successive Interference Cancellation [4].

The technique is fairly easy to understand intuitively. For each CRDSA attempt, each UE will replicate its packet X time in randomly selected slots in a set of Y slots. If a set of UEs collide in a single slot, then the successive interference cancellation algorithm at the receiver will cancel out the interference contribution of a UE based on a successful decoding in another slot. An example can be seen in Figure 1. In this case the UE transmits two replicas in a set of 6 slots. There are collisions in slots 2, 3, 5, which normally may not be decodable. This is resolved by the UE being able to successively remove interference contributions. As seen in Figure 1, the packet p2 of User 2, can be resolved since there is no collision in Slot 1. For Slot 2, the packet of User 1 can be resolved via p2 = r2 – p2 = r2 – r1. User 4 and 3 cannot be resolved as there is a collision of these packets in Slot 3 and Slot 4.

Figure 1.

To implement some type of CRDSA, there are a number of issues that may need to be resolved.

Protocol design

A lot of standardization challenges of CRDSA is how to design the protocol. Some issues for instance are how the UE selects one or more slots in a CRDSA frame, how the UE indicates which slots that the UE has transmitted in, variable or fixed number of replications, and whether the replications would be exactly the same.

One benefit is that RAN2 is already working sending contention-based RACH-less EDT-like procedures. This means that some of the issues for CRDSA may be solved when working on contention-based Msg3 transmission.

RAN1 and performance-related

Successive Interference Cancellation has been shown to have performance benefits over normal Slotted Aloha, which 4G/5G random access is based on. But the performance in a 3GPP network have to our knowledge not been considered. For instance, it is not clear how well Successive Interference Cancellation would work on NB-IoT and eMTC waveforms. RAN2 is not the correct venue to perform extensive simulations on this. This means that either RAN2 may assume that there are performance benefits, or RAN2 involves RAN1, which may or may not be advisable with the current WID.

Implementation-related considerations

For this case, one issue is that the successive interference cancellation needs to work over multiple slots, and that the end result of the successive interference cancellation decoder cannot fully complete until all of the slots in a CRDSA frame have been completed. This fact may make the feature more difficult to implement.

Successive Interference Cancellation was a popular technique in 3G-times for WCDMA, but there were practical implementation-challenges with this.

Altogether, we believe that RAN2 may continue to study CRDSA as a side-track of contention-based RACH-less EDT-like procedures, but at some point we believe that RAN1 needs to be queried on the feasibility of Successive Interference Cancellation techniques to resolve contentions. If RAN1 indicates that they see no issue, then RAN2 may continue to study it, else RAN2 may have to consider it for future generations, where perhaps the physical layer properties may be designed to work better with successive interference cancellation.

Proposal 6: RAN2 may continue to study basic protocol design for contention-based RACH-less Msg3 transmissions where contention may be resolved via Successive Interference Cancellation.

Proposal 7: Send LS to RAN1 on the feasibility and/or performance challenges of using Successive Interference Cancellation technique to resolve contention in a contention-based RACH-less Msg3 procedure.

# Conclusion

In this contribution we discussion initial issues on uplink capacity enhancements:

- Observation 1: EDT being RACH-based is contention-based and uses broadcasted resources and PUR is contention-less and uses dedicated configuration.

- Observation 2: RACH-less is a feature to introduce optimized handover skipping the random access procedure.

- Observation 3: The required GNSS position fix accuracy in a TN would be significantly higher compared to NTN owing to the geometry.

Proposal 1: RACH-less EDT-like procedures and any Msg4 enhancement are only introduced for NTN.

Proposal 2: Contention-based RACH-less EDT-like procedure is not dedicatedly pre-configured, but uses broadcasted common resources.

Proposal 3: RAN2 to discuss how to ensure that ephemeris and UE GNSS position is sufficiently recent to ensure that the synchronization is tighter compared to legacy NTN random access.

Proposal 4: Introduce possibility to configure repetitions for RACH-less EDT-like transmissions.

Proposal 5: If RACH-less Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 6: RAN2 may continue to study basic protocol design for contention-based RACH-less Msg3 transmissions where contention may be resolved via Successive Interference Cancellation.

Proposal 7: Send LS to RAN1 on the feasibility and/or performance challenges of using Successive Interference Cancellation technique to resolve contention in a contention-based RACH-less Msg3 procedure.

# Reference

RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#103, Maastrict, Netherlands, March 2024

3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)”, V18.1.0, Release 18, March 2024.

R2-2403206, Clarifications on the Scope of EDT enhancement for IoT-NTN, RAN2#125bis, Interdigital, April 2024.

Alessio M. et al, Random Access in DVB-RCS2: Design and Dynamic Control for congestion avoidance, IEEE transactions on Broadcasting, Vol: 60, Issue 1, 2014.