3GPP TSG-RAN WG2 Meeting #127	R2-2407258

Maastricht, Netherlands, 19th August – 23rd August, 2024

Agenda item:	8.8.6

Source:	Samsung

Title:	E-UTRAN TN to NR NTN basic scenario and signalling

WID/SID:	LTE_TN_NR_NTN_mob-Core

Document for:	Discussion and Decision

# Introduction

In RAN#103 the LTE-part of WID for Rel-19 was agreed [1]. The updated objectives of the WID is the following:

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

In this contribution, we provide our views on LTE to NR NTN mobility.

# Discussion

## RRC_INACTIVE

In the current WID, it is stated that only idle mode mobility is supported. However, RRC_INACTIVE was introduced for E-UTRAN connected to 5GC in Release 15. Since the WID only mentions idle mode mobility, this may imply that RRC_INACTIVE is not supported. When an LTE UE in RRC_INACTIVE selects another RAT as part of cell selection or cell reselection, the UE will move to RRC idle and discard the RRC configuration:

--------------- 36.304 ---------------

5.2.4.13	Cell reselection or CN type change in RRC_INACTIVE state

For UE in the RRC_INACTIVE state, upon cell reselection to another RAT or CN type change, UE transitions from RRC_INACTIVE to RRC_IDLE and performs actions as specified in TS 36.331 [3].

--------------- 36.304 ---------------

With this in mind we do not see a reason why mobility from E-UTRAN TN to NR NTN in RRC_INACTIVE should not be supported, as the UE will anyways move to RRC_IDLE upon selecting the NR NTN cell. In fact, if we limit the mobility to only RRC_IDLE, then this may require some specification changes. Such changes may for instance be restrictions on a UE in RRC_INACTIVE to not measure NR NTN cells.

Proposal 1: RAN2 confirms E-UTRAN TN to NR NTN RRC_INACTIVE mobility is supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell. No spec change.

## Signalling

Last meeting the following agreement was taken:

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

For Option 1, we do not think that there would be a need to introduce a new SIB as the current SIB33 already provides the framework required to signal assistance information. E-UTRAN and eMTC does for instance not have the same size restrictions as in NB-IoT which motivated the introduction of SIB33 as opposed to implementing neighbour cell satellite assistance information in SIB31.

Option 2 and Option 3 are similar in that a new IE is introduced. They are somewhat equivalent and both options are likely to work. However Option 3 would be the normal manner in which a parallel list is extended. Option 3 could for instance enable a network to signal neighbour satellite assistance information in a much more efficient manner if the network signals both intra-RAT eMTC NTN and NR NTN. In that case the extended element may signal NR NTN-relevant information.

Proposal 2: Confirm that SIB33 and its ephemeris is used to broadcast NR NTN neighbour cell ephemeris where NeighSatelliteInfo is extended as a parallel list. No new SIB is introduced.

The fields that should be a part of an extended NeighSatelliteInfo should be ntn-PolarizationDL and TA common parameters. One issue is that in Rel-18 SIB33, is that the TA common parameters are slightly different. The main reason for why the TA common parameters are different in IoT NTN is because the needed precision is lower since IoT is normally on lower frequencies compared to future uses of NR NTN, such as FWA. However, since the use case of E-UTRAN to NR NTN is likely for mobile device as opposed to FWA, we believe that it is likely that the current TA common parameters may be sufficient for NR NTN cells. With this, we think that the NR NTN TA common parameters may be optional, and if not included, the UE relies on nta-CommonParameters-r18.

Another reason is that if there is ever a case where the cell signals intra-RAT eMTC NTN and NR NTN, then if there is a satellite that supports both eMTC NTN and NR NTN, then the release 18 TA common needs to be broadcasted for legacy release 18 UEs, and then the release 19 UEs using the NR NTN parameters should be able to use the release 18 TA common. Otherwise both TA common needs to be broadcasted, which in the case for TA common will use a non-negligible amount of bits.

Proposal 3: If NR NTN TA Common parameters in extended NeighSatelliteInfo-v1900 are not present, the UE uses the nta-CommonParameters-r18 in NeighSatelliteInfo-r18.

## Idle mode measurements on NR NTN

In the last meeting the following was noted:

R2-2404211	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT	discussion

... OMITTED ...

Proposal 8: NR NTN neighbor cell measurement initiation is performed as in legacy, i.e. based on RRM measurement as in cell reselection from EUTRA TN to NR TN. No spec impact is needed on cell reselection procedure in TS 36.304.

-	Samsung thinks we should check this more until the next meeting. Ericsson agrees

Come back in the next meeting

One issue with NTN in general is that measuring an NTN cell is likely to be a lot more power consuming compared to a terrestrial cell. This is because a UE will have to autonomously compensate for timing and frequency offset, and then tune to the frequency of the specific NTN cell to measure it. If there are multiple satellites with different timing and frequency offsets, the UE would have to repeat this for every satellite. Thus it is important to ensure that a UE that is capable of measuring NTN, does so in a power efficient manner. This can be done by measuring the NTN cell when it is expected that NTN should be a better option.

Signalling NR NTN cells in idle mode configurations would most likely make most sense in a rural cell. However, even in a large rural cell, it would only be on the cell edge where measuring an NTN cell would be a good option to ensure service continuity.

In Release 17 and Release 18 NR/IoT NTN, location based measurement initiation was introduced. These enhancements introduce the possibility of not performing certain measurements if the UE is within a certain distance from a reference location. These measurements may be intra-frequency measurements, inter-frequency of equal or lower priority, inter-RAT frequencies of lower priority, depending on whether the UE fulfil some signal strength and quality thresholds.

For LTE TN to NR NTN mobility, we believe that similar conditions may be very useful in order to avoid excessive power consumption due to measuring NR NTN cells. Therefore we propose that distance-based initiation to measure NR NTN cells from E-UTRA TN cell is introduced.

Proposal 4: RAN2 to consider the benefits on power saving by introducing a distance-based measurement initiation of NR NTN cells in a terrestrial E-UTRA cell. Distance is between stationary reference location and a UE.

# Conclusion

In this contribution we discussed issues related to E-UTRAN TN to NR NTN mobility.

Proposal 1: RAN2 confirms E-UTRAN TN to NR NTN RRC_INACTIVE mobility is supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell. No spec change.

Proposal 2: Confirm that SIB33 and its ephemeris is used to broadcast NR NTN neighbour cell ephemeris where NeighSatelliteInfo is extended as a parallel list. No new SIB is introduced.

Proposal 3: If NR NTN TA Common parameters in extended NeighSatelliteInfo-v1900 are not present, the UE uses the nta-CommonParameters-r18 in NeighSatelliteInfo-r18.

Proposal 4: RAN2 to consider the benefits on power saving by introducing a distance-based measurement initiation of NR NTN cells in a terrestrial E-UTRA cell. Distance is between stationary reference location and a UE.

# Reference

RP-240846, New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN, Thales, CATT, RAN#103, Maastrict, Netherlands, March 2024

3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”, V18.1.0, Release 18, March 2024.

3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)”, V18.1.0, Release 18, March 2024.

# Examples

---------------------------- 36.331 V18.2.0 Example ----------------------------

–				SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...,

[[ neighSatelliteInfoList-v1900 	NeighSatelliteInfoList-v1900	OPTIONAL 	-- Need OR

]]

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfoList-v1900 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-v1900

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

NeighSatelliteInfo-v1900 ::=	SEQUENCE {

nta-CommonParameters-v1900		SEQUENCE {

nta-Common-v1900					INTEGER (0..66485757)		OPTIONAL,	-- Need OP

nta-CommonDrift-v1900			INTEGER (-257303..257303)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-v1900	INTEGER (0..28949)			OPTIONAL	-- Need OP

}																OPTIONAL,	-- Need OP

ntn-PolarizationDL-r19				ENUMERATED {rhcp,lhcp,linear}	OPTIONAL,  -- Need OP

ntn-PolarizationUL-r19				ENUMERATED {rhcp,lhcp,linear}	OPTIONAL   -- Need OP

}

-- ASN1STOP

SystemInformationBlockType33 field descriptions

epochTime Epoch time of the neighbour satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the neighbour satellite ephemeris and Common TA parameters is the uplink time synchronization reference point when this field is provided in an NTN cell and the eNB when this field is provided in a TN cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If this field is absent, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. The startSFN indicates the SFN nearest to the frame where the message indicating the epochTime is received.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

neighValidityDuration Validity duration of the neighbour satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. If this field is absent, the UE uses validity duration from the serving cell assistance information.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs for nta-Common-r18 and step of 4.072 × 10-3 μs for nta-Common-v1900. Actual value = field value * 32.55208 ×10-3. If the field(s) are absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field(s) are absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field(s) are absent, the UE uses the (default) value of 0.

t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

---------------------------- 36.331 V18.2.0 Example ----------------------------