3GPP TSG-RAN WG2 Meeting #127	R2-2407260

Fukuoka, Japan, 20th May – 24th May, 2024

Agenda item:	8.8.5

Source:	Samsung

Title:	Regenerative payload for NR NTN

WID/SID:	NR_NTN_Ph3

Document for:	Discussion and Decision

# Introduction

In RAN#103 the NR-part of the WID NR NTN Phase in Rel-19 was agreed [1]. The updated objectives of the WID on the topic of Regenerative payload is the following:

4. Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we provide our views on regenerative payload for NR NTN.

# Discussion

## Satellite switch with resync

In the last meeting, the following was discussed:

R2-2405021	Support of regenerative payload	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: PCI unchanged solution is beneficial for signaling overhead reduction in NTN system due to without L3 mobility.

Proposal 2: To support PCI unchanged for regenerative payload with gNB on board, keeping same security key between source satellite gNB and target satellite gNB based on ISL coordination is demanded.

-	LG wonders if different satellites can use the same PCI. CMCC thinks this is possible

-	Nokia thinks satellite switch with resync could be supported in this case, but with impacts on RAN3 and SA3 (and this would probably need a new WI). Ericsson agrees

-	Thales thinks we should support this feature in a clean way, involving RAN3 and SA3. CMCC agrees

-	Ericsson thinks we should not support satellite switch with resync with regenerative payload

-	Ericsson thinks that the possible involvement of other groups needs to be discussed in the plenary. LG agrees

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

Some of the arguments against introducing satellite switch with resync are related to the implementation feasibility, the benefits of Rel-18 satellite switch with resync, as well as impacts on other groups such as SA3.

In the RAN plenary, there were contributions proposing that satellite switch with resync with regenerative payload should be introduced. However, all of the contributions related to this were noted and nothing was agreed, because it would be considered to be up-scoping. So we believe that RAN2 should confirm RAN plenary’s intention and not consider satellite switch with resync for regenerative payload in release 19.

Proposal 1: RAN2 confirms that satellite switch with resync for regenerative payload is not addressed in Rel-19.

## RA SDT

In the last meeting, the following was discussed:

R2-2404680	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1.	The satellite switch with resync feature may be supported in regenerative payload architecture by network implementation of mirror satellite at t-Service.

Proposal 1	From RAN2 perspective, whether to indicate the support of satellite switch with resync for regenerative payload is left to network implementation. The only RAN2 specification impact is to clarify the +ve gap of satellite switch i.e., t-ServiceStart can occur after t-Service.

Proposal 2	For UE power saving when using RA SDT in regenerative payload, the PDCCH monitoring for further RRC message is delayed by UE-gNB RTT after sending HARQ feedback of the contention resolution MAC CE.

-	Apple thinks would change the RA SDT procedure

-	vivo thinks a similar discussion was done for RA-SDT in TN and there was no consensus to change

In contribution R2-2404680 [2] can supposedly be made more power efficient in a regenerative payload situation. The problem in the contribution has to do with how to handle the case where the DL response to an UL packet in Msg3 may take time to arrive to gNB due to the gNB-CN delay in a regenerative payload case. The contribution suggests that UE should not continuously monitor for downlink data after having received a contention resolution message.

Figure 1. Figure from [2].

The contribution seems to suggest that power saving enhancements may be achieved if the UE does not continuously monitor after having received a contention resolution MAC CE. However, in our understanding a UE may have to monitor for DL packets for a long time, since the DL packet in response to the UL packet can arrive at any point. This means that a UE would normally have to monitor for packets for a long time before the SDT procedure is finished when receiving the RRC release. Thus the power saving gains for solving this problem are not very large.

Given that RA-SDT is not a vital feature for NR NTN regenerative payload, and that RA-SDT works in the current specification for regenerative payloads, we do not see a lot of benefit of introducing an enhancement for at most fractional power saving specifically for RA-SDT. Similarly, we do not think that the WID supports power saving enhancements to support an optional and non-vital feature for regenerative payloads.

Proposal 2: Do not optimize RA-SDT procedure for regenerative payload architecture.

## RACH-less optimization

In the last meeting, the following agreement was taken:

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

In contribution R2-2405148 [3], RACH-less handovers are discussed in the context of regenerative payload.

One suggested solution is that target gNB is informed when the UE is present in the target cell. It is stated that for RACH-less handover with a pre-allocated grant, there is a need to for exchange between target and source cell. It is stated that for RACH-less with dynamic UL grant, it seems to imply that no exchange is needed, but it would be desirable for the target cell to know when the UE has handed over to the target gNB.

When performing a RACH-less handover with dynamic UL grant, one can expect that the UE should immediately perform the handover and then start monitoring the PDCCH. In this case we do not see a need to indicate when the UE is present in the target cell. If for instance RACH-less handover is used in a CHO configuration, we believe that the only combination of RACH-less and CHO that makes sense to deploy is with Condevent T1 configured. In that case, RAN3 has introduced support to signal the t1-threshold to the target cell.

Given the above, we do not see that there is a need for any enhancement on RACH-less for regenerative payload.

Proposal 3: No enhancement on RACH-less for regenerative payload is needed.

# Conclusion

In this contribution we discussed issues related to regenerative payload for NR NTN.

Proposal 1: RAN2 confirms that satellite switch with resync for regenerative payload is not addressed in Rel-19.

Proposal 2: Do not optimize RA-SDT procedure for regenerative payload architecture.

Proposal 3: No enhancement on RACH-less for regenerative payload is needed.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#104, Shanghai, China, June 2024.

R2-2404680, Discussion on regenerative payload, Qualcumm Incorporated, RAN2#126, Fukuoka, Japan, May 2024.

R2-2405148, On the feasibility of the existing NTN features over regenerative architecture, Nokia, RAN2#126, Fukuoka, Japan, May 2024.