3GPP TSG RAN WG2 Meeting #127                           R2-2407261

Maastricht, the Netherlands, August 19th – 23rd, 2024

Source:	Sharp

Title:	Discussion on A-IoT paging

Agenda Item:	8.2.3

Document for:	Discussion and Decision

Introduction

A new study item on solutions for Ambient IoT [1] for Rel-19 was approved in RAN#102, with objectives including the following:

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In this document, we share our views on a few aspects of A-IoT paging.

Discussion

Selection criteria

The following was agreed in RAN2#126,

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

In the case of A-IoT paging via a group ID that maps to multiple A-IoT devices, our understanding is that, for the sake of flexibility in inventory, A-IoT devices in the corresponding group should NOT be “hard-coded”. Instead, the group can e.g. be established by means of upper layer commands. Furthermore, it should be able to assign an A-IoT device to multiple groups, each containing a different (but maybe partially overlapping) set of A-IoT devices.

An A-IoT group can be established via upper layer signalling can be established via upper layer signalling.

An A-IoT device can be assigned to multiple A-IoT groups.

Given the above observations, it seems the management of A-IoT groups consumes a lot of time and signaling. For example, for establishment of an A-IoT group for a number of A-IoT devices, the reader has to first page each of those A-IoT devices, one by one, in order to “write” the group ID in those A-IoT devices.

In comparison, in UHF RFID, a “Select” command comes with a few parameters to respectively specify a memory bank, a pointer to a starting memory address, a length, and a mask with a signaled length, such that the group of matching tags are those with the mask matching their memory contents starting at the pointer. With this the reader can easily page devices with e.g. same values in a specific portion of device ID digits, without prior establishment of a “group” for those devices. In our view, this kind of “ad-hoc” grouping mechanism can be a very good trade-off between hard-coded and upper layer established A-IoT device groups. Therefore, in addition to the agreed cases, we propose to study the case where an A-IoT paging message contains a device ID “pattern” in order to address all devices with A-IoT device IDs matching the pattern.

- Study the case where an A-IoT paging message contains a device ID “pattern” in order to address all devices with A-IoT device IDs matching the pattern.

Monitoring of A-IoT paging

The following was agreed in RAN2#126,

Agreements 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

From RAN2 perspective, although in general an A-IoT device “can receive as long as there is enough energy”, if the A-IoT device does successfully receive an A-IoT paging message that is not addressed to it, it is a good indication that a number of subsequent R2D messages (e.g. at least R2D messages for random access) will NOT be addressed to it either. Therefore, the A-IoT device should be able to stop receiving and/or start harvesting energy, without missing any A-IoT paging messages addressed to it. Although this can be a full A-IoT device implementation, the energy harvesting gain (and in general the energy efficiency) can only be promising if a practical minimum period of time for energy harvesting can be guaranteed. For example, if the A-IoT device can know the minimum time to be consumed by the “session” initiated by the received A-IoT paging message, then the A-IoT device can fully make use of such a time window for sleeping or energy harvesting.

- Study the minimum time window for the “session” initiated by a received A-IoT paging message.

Paging message format

An A-IoT paging message can be used to trigger either a “3-step” RA or a “2-step” RA, and the RA can be either contention-based or contention-free. It is highly desirable that a same message format is used for all these cases. The exact RA type can be indicated by control information in the A-IoT paging message.

- A single A-IoT paging message format is used for triggering all types of A-IoT random access.

Conclusion

In this contribution, we discuss a few aspects of A-IoT paging, and make the following observations and proposals:

- An A-IoT group can be established via upper layer signalling can be established via upper layer signalling.

- An A-IoT device can be assigned to multiple A-IoT groups.

- Study the case where an A-IoT paging message contains a device ID “pattern” in order to address all devices with A-IoT device IDs matching the pattern.

- Study the minimum time window for the “session” initiated by a received A-IoT paging message.

- A single A-IoT paging message format is used for triggering all types of A-IoT random access.

References

- RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102.