3GPP TSG RAN WG2 Meeting #127                           R2-2407262

Maastricht, the Netherlands, August 19th – 23rd, 2024

Source:	Sharp

Title:	Discussion on A-IoT random access

Agenda Item:	8.2.4

Document for:	Discussion and Decision

Introduction

A new study item on solutions for Ambient IoT [1] for Rel-19 was approved in RAN#102, with objectives including the following:

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

The following was agreed in RAN2#126,

Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. - RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA 1 A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed Agreements on CFRA procedures - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this document, we share our views on a few aspects of A-IoT random access.

Discussion

Determination of random access type

The random access type can be determined based on e.g. whether A-IoT device(s) is fully identified via the corresponding A-IoT paging message. For example, if the A-IoT paging message contains one or multiple device IDs, then the corresponding, uniquely identified, A-IoT devices can perform contention-free random access; otherwise (e.g. if a group ID is contained in the A-IoT paging message), the addressed A-IoT device(s) performs contention-free random access.

- The random access type is determined based on whether A-IoT device(s) addressed by the corresponding A-IoT paging message is fully identified via the A-IoT paging message.

Random ID generation

Both time-domain multiple-access and frequency-domain multiple access were included in the A-IoT study in RAN1, according to the following RAN1#116-bis agreements,

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

In NR random access procedure, The RA-RNTI (which is used to scramble the CRC of a PDCCH scheduling a PDSCH carrying RAR messages) associated with the PRACH time/frequency occasion in which the PRACH preamble is transmitted is calculated such that it is different for different PRACH time/frequency occasion. This allows “contention” to only occur among UEs transmitting PRACH preambles using a same PRACH time/frequency occasion. In other words, contention never occurs among UEs transmitting PRACH preambles using different PRACH time/frequency occasions. The rationale behind this was that two PRACH transmissions respectively on two different PRACH time/frequency occasions can be easily detected by the gNB without any confusion for both gNB and UEs (e.g. even if a same PRACH preamble was used for these two PRACH transmissions), and there may only be a problem if the two PRACH transmissions are on a same PRACH time/frequency occasion, e.g. in this case if a same PRACH preamble was used for these two PRACH transmissions, it may be impossible to even tell whether the received signal came from one UE or two UEs.

In A-IoT, it is also desirable that contention never occurs among devices transmitting A-IoT Msg1’s in different time/frequency resources. In order to achieve this, and since it has been agreed that the random ID contained in A-IoT Msg1 is the sole criteria for contention resolution, generation of the random ID should take the time/frequency resource for the corresponding A-IoT Msg1 into account.

For example, an A-IoT Msg1 backscattered without any small frequency-shift in baseband and another A-IoT Msg1 backscattered with a non-zero small frequency-shift in baseband are transmitted in two FDM’ed channels and are likely to be both successfully received by the reader, and so it is highly desirable that the random IDs respectively contained in these two A-IoT Msg1’s are always different. This can e.g. be achieved by taking the amount of small frequency-shift into account when generating the random ID.

- When generating the random ID to be contained in an A-IoT Msg1, the selected time/frequency occasion for the corresponding D2R transmission is taken into account.

A-IoT Msg1 with and without data

In our view the difference in design for A-IoT Msg1 with and without data should be minimal. For example, the random ID should be included in A-IoT Msg1 for both cases, and reader should echo the random ID in the corresponding A-IoT Msg2 for both cases. The main difference between these two cases lies in whether device ID is contained in the A-IoT Msg1.

- For both A-IoT Msg1 without data and A-IoT Msg1 with data,

- A random ID is contained in the A-IoT Msg1.

- The random ID is echoed in the corresponding A-IoT Msg2.

- In case of A-IoT Msg1 without data, no device ID is contained in the A-IoT Msg1.

- In case of A-IoT Msg1 with data, a device ID is contained in the A-IoT Msg1.

Conclusion

In this contribution, we discuss a few aspects of A-IoT random access, and make the following proposals:

- The random access type is determined based on whether A-IoT device(s) addressed by the corresponding A-IoT paging message is fully identified via the A-IoT paging message.

- When generating the random ID to be contained in an A-IoT Msg1, the selected time/frequency occasion for the corresponding D2R transmission is taken into account.

- For both A-IoT Msg1 without data and A-IoT Msg1 with data,

- A random ID is contained in the A-IoT Msg1.

- The random ID is echoed in the corresponding A-IoT Msg2.

- In case of A-IoT Msg1 without data, no device ID is contained in the A-IoT Msg1.

- In case of A-IoT Msg1 with data, a device ID is contained in the A-IoT Msg1.

References

- RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102.