3GPP TSG-RAN WG2 Meeting #127											        R2-2407263

Maastricht, 19-23 Aug. 2024

Agenda item:			8.8.4

Title:						Considerations on MBS in NTN

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

The Non-Terrestrial Networks (NTN) for NR Phase 3 work item aims at specifying support for broadcast services via NR NTN in Rel-19 [1]:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

Further, during RAN2#125bis [2] and RAN2#126 [3] the following agreements were made:

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this paper, we share our views on the intended MBS service area indications in NTN.

# 2. Discussion

Due to its comparably large coverage areas, NTNs promise to provide an efficient way for emergency messaging broadcast as well as on-demand media content distribution. However, there are large degrees of freedom for NTN operators to configure antenna beams and resulting beam footprints, such that intended broadcast service areas may be much more confined and smaller than a physical radio cell that can be detected by a UE. Thus, a UE is not aware of the intended broadcast service area and may waste energy when trying to establish an MBS session when outside of the intended service area.

Further, the NTN operator can simultaneously provide multiple MBS services via different frequencies within several portions of one radio cell, which could result in several intended MBS service areas. Thus, the network should provide some guidance to the UE for MBS service selection.

Proposal 1: Information on intended MBS service area is broadcast by gNB, including reference location coordinates as well as distance and/or RSRP thresholds.

In scenarios with earth-moving cells, e.g., where MBS is provided by LEO satellites, the UE that is interested in using a particular MBS service or already established an MBS session may face the issue of service degradation when moving from one intended MBS service area to another, resulting in poor user experience. For example, media access and provisioning to UEs may be interrupted or noticeably affected when moving from one intended MBS service area to a neighbouring service area.

Proposal 2: In scenarios with earth-moving cells, ensuring service continuity between neighbouring service areas is challenging and should be further discussed by RAN2.

Particularly, in scenarios with earth moving cells the network could provide further MBS service-related information. In addition to broadcasting the intended MBS service area, the gNB provides MBS service-specific estimated service times. Based on broadcast service-specific estimated service times, UE decides whether to initiate MBS service or not.

Proposal 3: In addition to broadcasting the intended MBS service area, MBS-specific estimates, e.g., MBS service stop times, are provided by the network.

Based on those estimated MBS service times, UE can decide whether to initiate MBS service or not.

# Conclusions

The following proposals have been made in this contribution:

Proposal 1: Information on intended MBS service area is broadcast by gNB, including reference location coordinates as well as distance and/or RSRP thresholds.

Proposal 2: In scenarios with earth-moving cells, ensuring service continuity between neighbouring service areas is challenging and should be further discussed by RAN2.

Proposal 3: In addition to broadcasting the intended MBS service area, MBS-specific estimates, e.g., MBS service stop times, are provided by the network.

# References

- RP-234076, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#102, December 11-15, 2023, Edinburgh, UK.

- R2-2403733, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#125bis, Changsha, China, April 15th – 19th, 2024.

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#126, Fukuoka, Japan, May 20th-24th, 2024.