# 3GPP TSG-RAN WG2 Meeting #127	                               R2-2407265
Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item	: 8.2.4 (FS_Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Contention based random access procedure

    - Contention-based 4-step random access procedure

In RAN2#126 meeting, RAN2 discussed the 4-step random access procedure in details for A-IOT. However, the size of the random ID in Msg1 is not concluded yet.

Agreements on “4 step” RA 1 A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements 3 A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) 4 The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. 5 “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

During 4-step CBRA, the A-IOT device randomly selects a random ID for A-IoT Msg1 and the A-IOT device transmits the selected random ID in Msg1. After that, if the A-IOT device receives the same random ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed. For this assumption, in RAN2#126 meeting, the collision probability was analysed according to the size of the random ID as shown below. (Please refer the calculation parameters for collision probability in R2-2404243)

- If the size of the random ID is 32bit, the collision probability is approximately 0.000000023%

- If the size of the random ID is 16 bit, the collision probability is approximately 0.0015%.

- If the size of the random ID is 8 bit, the collision probability is approximately 0.38%.

As shown above, if the random ID is increased, the collision probability is decreased. However, since the large size of the random ID causes the signalling overhead, we think that the 16 bit random ID is suitable.

Proposal 1. For 4-step CBRA, the 16 bit random ID is used in Msg1.

    - Contention-based 2-step random access procedure

In RAN2#126 meeting, RAN2 discussed the 2-step random access procedure in details for A-IOT, and the following agreements were made.

Agreements on 2 step CB RA 1. A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information 2. A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed

Similar to 4-step CBRA, in 2 step CBRA, the Msg2 should used to complete the contention resolution. It means that the Msg2 should include the identification information to notify the successful contention resolution.

In order to notify the successful contention resolution, we think that there are two options as follows.

- Option 1. The A-IOT device transmits the A-IOT device ID and random ID in Msg1. After that, if the A-IOT device receives the same random ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed.

- Option 2. The A-IOT device transmits only A-IOT device ID in Msg1. After that, if the A-IOT device receives the same A-IOT device ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed.

In our understanding, the A-IoT device ID can be the EPC number. In addition, according to the TR 22.840, the length of the EPC number ranges from 96 bits to 496 bits. It means that the signalling overhead of the Option 1 would be same as the 4-step CBRA and the signalling overhead of Option 2 would be larger than that of 4-step CBRA.

Since the A-IOT device has the limited energy storage, we think that the signalling overhead should be reduced as much as possible. Thus, we prefer to go Option 1.

Proposal 2. For 2-step CBRA, the A-IOT device transmits the A-IOT device ID and random ID in Msg1. After that, if the A-IOT device receives the same random ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed.

    - Prioritization of the contention-based random access procedure

Based on above 4-step and 2-step CBRA discussions, the procedure of 4-step and 2-step CBRA is as follows.

As shown above figure, the main difference between the 4-step and 2-step CBRA procedure is when the A-IOT device transmits the A-IOT device ID. In 4-step CBRA, the A-IOT device transmits the A-IOT device ID on the Msg3 and in 2-step CBRA, the A-IOT device transmits the A-IOT device ID in Msg1. Considering that, it is questionable whether both CBRA procedure are need to be discussed. Thus, we think that it would be better to prioritize the discussion for one CBRA procedure.

Observation 1. Since there is not much difference between the 4-step and 2-step CBRA procedure, it would be better to prioritize the discussion for one CBRA procedure.

In our view, since the A-IOT device has the limited energy storage, the CBRA procedure should be simplified as much as possible. Thus, we think that the discussion of the 2-step CBRA procedure should be prioritized over the 4-step CBRA procedure.

Proposal 3. The 2-step CBRA procedure should be prioritized over 4-step CBRA procedure.

  - Contention free random access procedure

    - When to use the CFRA procedure

In RAN2#125bis meeting, it was agreed that we will study the support for access triggering for a single device, group of devices, or all devices. For the single device case, the random access from a designated UE would not need contention-based access and the contention free access procedure would be simple and appropriate because the network requests the data transmission only for a designated A-IOT device.

Proposal 4. For random access procedure from a designated A-IOT device, the CFRA is used as a baseline.

In addition, in the RAN2#126 meeting, it was agreed that A-IOT paging message can include multiple A-IOT device IDs. We think that if the A-IOT paging message includes multiple A-IOT device IDs, the CFRA can be used based on the order of the A-IOT device IDs.

For example, if the reader transmits the A-IOT paging message containing the first A-IOT device ID and the second A-IOT device ID, only first A-IOT device and the second A-IOT device perform the random access procedure. In this case, if the first and second A-IOT devices select an access occasion according to the order of the A-IOT device ID, the A-IOT device can perform the CFRA procedure.

Proposal 5. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.

    - How to perform the contention free random access procedure

Since it is assumed that the CFRA may be used for the designated A-IOT device(s) as a baseline, only one A-IOT device would perform the CFRA on the designated access occasion. For this, contention resolution is not needed for CFRA. Thus, we think that the A-IOT device directly transmits the A-IOT device ID without transmitting random ID. Then, the A-IOT device considers that the CFRA is successfully completed. The following figure shows the difference between the 2-step CBRA and CFRA.

Proposal 6. When the A-IOT device performs the 2-step contention-free random access, the A-IOT device transmits a A-IOT device ID without transmitting random ID. Then, the A-IOT device considers that the CFRA is successfully completed.

  - How to allocate the access occasion

In RAN2#125bis meeting, it was agreed that slotted-ALOHA is the baseline for A-IOT random access.

The following is an example for the slotted-ALOHA. If the reader requests the data transmission to the multiple A-IOT devices, the A-IOT device selects a slot and transmits the data. If at least two A-IOT devices perform the contention-based access on a same slot, the collision happens. Then, the collided A-IOT devices re-selects another slot to transmit/receive data.

In the slotted-ALOHA, the important point is how the A-IOT device knows the beginning of the slot, hereinafter calls access occasion, because the A-IOT device cannot support the synchronization system. Thus, the A-IOT device must know the exact timing of access occasion to transmit or receive data.

In order to indicate the beginning of access occasion, the reader should send a start indication for the access occasion to the A-IOT device(s). Then, if the start indication is received from the reader, the A-IOT device(s) starts to perform the contention-based or contention-free access procedure.

Proposal 7. In order to indicate the start of the access occasion, the reader should send a start indication for the access occasion to the A-IOT device(s). Then, the A-IOT device(s) performs the contention-based or contention-free access procedure.

Even if the A-IOT device knows the beginning of access occasion, there may be a collision if multiple A-IOT devices choose the same occasion. Thus, a slot reservation mechanism also needs to be considered.

The simple way is that the A-IOT device considers that the access occasion is reserved when receiving the same random ID transmitted in Msg1. For example, at access occasion#1, the A-IOT device transmits the A-IOT device ID#1 and random ID#1. But, if the A-IOT device receives the random ID#2, the A-IOT device considers that the access occasion is not reserved. Later on, at access occasion#2, the A-IOT device transmits the A-IOT device ID#1 and random ID#1 and the A-IOT device receives the random ID#1. In this case, the A-IOT device considers that the access occasion is reserved and the A-IOT device starts the data transmission and reception procedure.

Proposal 8. After an A-IOT device transmits the A-IOT device ID and random ID in Msg1, the A-IOT device considers that the access occasion is reserved if the A-IOT device receives same random ID in Msg2.

  - Attempt the access procedure

As described above, the A-IOT devices attempt the access procedure on an access occasion when receiving a start indication for the access occasion. However, if all A-IOT devices attempt the access procedure on the same access occasion, the collision probability would be increased. In this sense, how to distribute the access occasion for the multiple A-IOT devices should be discussed.

From the A-IOT device point of view, if the A-IOT device does not know how many access occasion exists, the A-IOT device selects the earliest access occasion. Of course, the A-IOT device can select every access occasion and perform the access procedure. However, it would cause a waste of energy in the A-IOT device and increase the probability of access failure.

In order to avoid the above case, we think that the simple solution is that the number of access occasion is included in the A-IOT paging message. More specifically, if the A-IOT device receives the A-IOT paging message containing the number of the access occasion, the A-IOT device randomly selects one access occasion within the number of the access occasion. Then, the A-IOT device performs the access procedure on the selected access occasion.

Proposal 9. The reader should indicate the number of access occasion in an A-IOT paging message.

Even if the A-IOT device knows the number of access occasion, RAN2 need to discuss how to select the access occasion within the number of access occasion. For this, we think that the energy level of the A-IOT device should be considered.

Since the A-IOT device has small energy storage capability, an A-IOT device may be in a condition that it doesn’t have enough power to complete data transmission. In this case, the A-IOT device will fail to perform the access procedure. However, the reader does not have information on whether an A-IOT device has enough power to complete the access procedure or not. Thus, the A-IOT device should select the access occasion by considering the A-IOT device’s energy level. For instance, an A-IOT device with low energy level may randomly select the first part of access occasions to save its energy (not to wait long time for access attempt). In other way, an A-IOT device with low energy level may randomly select the last part of access occasions to have more time for energy harvesting.

Proposal 10. RAN2 study that the A-IOT device selects the access occasion within the number of access occasion by considering the A-IOT device’s energy level.

  - Re-access procedure

    - Data transmission failure

As discussed above, the A-IoT devices suffers the contention resolution failure due to collision. In addition, even if the contention resolution is successfully completed, the data transmission failure can happen.

When the combination of the inventory and command procedure is performed, it is reasonable to perform the inventory procedure first. Then, the command procedure is performed after successfully performing the inventory procedure. However, from the A-IOT device perspective, the A-IOT device cannot know whether the command procedure will be performed after performing the inventory procedure. In this case, it is hard to identify whether the data transmission procedure is successfully completed or not.

For example, after the A-IOT device performs the inventory procedure, the A-IOT device does not receive a message for the command procedure but the reader transmits the message for the command procedure after finishing the inventory procedure. After that, the reader can transmit the start indication for the access occasion to the A-IOT device. In this case, the reader considers that the data transmission procedure is not successfully completed for A-IOT device but the A-IOT device considers that the data transmission procedure is successfully completed.

Of course, it may be argued that the reader would not transmit the start indication for the access occasion to indicate the start of the new access occasion before the completion of the whole procedure. However, in our view, since the reader does not know whether the A-IOT device does not transmit the response due to the missing message or out-of-energy, the reader would transmit the message only one or two times. Then, the reader may transmit the start indication for the access occasion to indicate the start of the new access occasion.

In order to prevent the above case, we think that the simple solution is that the reader contains the procedure type or number of messages in A-IOT paging message or start indication for the access occasion. Using this information, the A-IOT device can determine whether the data transmission is successfully completed or not.

For example, the reader indicates that this procedure is the combination of the inventory and command procedure in A-IOT paging message or start indication for the access occasion. However, the A-IOT device does not receive the command message before receiving the start indication for the access occasion. In this case, the A-IOT device considers that the data transmission is not successfully completed and the A-IOT device performs the re-access procedure.

Proposal 11. In A-IOT paging message or start indication for the access occasion, the procedure type or number of messages is contained to determine whether to perform re-access procedure or not.

    - How to perform re-access procedure

In RAN2#125bis meeting, it was agreed that how to handle the contention resolution failure and access failure will be studied. During the discussion, the following options were proposed.

- Option 1. If contention resolution or access procedure is failed, the A-IOT device performs the re-access procedure within the same round.

- Option 2. If contention resolution or access procedure is failed, the A-IOT device performs the re-access procedure within the next round.

For Option 1, the A-IOT device continuously performs the access procedure if the access failure happens. In this option, since A-IOT devices that have failed the access procedure and the other A-IOT devices perform the access procedure, the collision probability would be increased.

For Option 2, similar to the RFID, the A-IOT device waits for the next round indication from the reader when the access failure happens. If the next round indication is received, the A-IOT device randomly selects the access occasion. Then, the A-IOT device performs the access procedure on the selected access occasion. This option causes that the completion of the access procedure would be delayed.

In our view, if the A-IOT devices that have failed the access procedure attempt the access procedure on the same access occasion, it would cause a waste of energy in the A-IOT device and increase the probability of access failure. Thus, we prefer to go Option 2.

Proposal 12. If contention resolution or access procedure is failed, the A-IOT device perform the re-access procedure within the next round.

In addition, RAN2 need to discuss how to indicate the next round. For this, we think that there are two options as follows.

- Option 1. The start indication for the access occasion is used to indicate a new next round.

- Option 2. The A-IOT paging message is used to indicate a new next round.

If the next round indication is included in the start indication for the access occasion, the A-IOT devices missing the paging message cannot perform the access procedure until they receive the A-IOT paging message. It means that the data transmission of the A-IOT devices that missed the paging message would be delayed. In addition, from the scheduling point of view, it would be better to perform the access procedure for the A-IOT devices that have failed the access procedure and the A-IOT devices that missed the A-IOT paging message in the next round. For this, we think that it is required that the A-IOT paging message contains an indication for the A-IOT device that missed the A-IOT paging message to perform the access procedure. If an A-IOT device successfully performs the whole procedure, the A-IOT device does not need to perform the access procedure even if the A-IOT paging message is received again.

Proposal 13. The A-IOT paging message contains an indication to indicate a new next round.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. For 4-step CBRA, the 16 bit random ID is used in Msg1.

Proposal 2. For 2-step CBRA, the A-IOT device transmits the A-IOT device ID and random ID in Msg1. After that, if the A-IOT device receives the same random ID in Msg2, the A-IOT device considers that the contention resolution is successfully completed.

Observation 1. Since there is not much difference between the 4-step and 2-step CBRA procedure, it would be better to prioritize the discussion for one CBRA procedure.

Proposal 3. The 2-step CBRA procedure should be prioritized over 4-step CBRA procedure.

Proposal 4. For random access procedure from a designated A-IOT device, the CFRA is used as a baseline.

Proposal 5. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.

Proposal 6. When the A-IOT device performs the 2-step contention-free random access, the A-IOT device transmits a A-IOT device ID without transmitting random ID. Then, the A-IOT device considers that the CFRA is successfully completed.

Proposal 7. In order to indicate the start of the access occasion, the reader should send a start indication for the access occasion to the A-IOT device(s). Then, the A-IOT device(s) performs the contention-based or contention-free access procedure.

Proposal 8. After an A-IOT device transmits the A-IOT device ID and random ID in Msg1, the A-IOT device considers that the access occasion is reserved if the A-IOT device receives same random ID in Msg2.

Proposal 9. The reader should indicate the number of access occasion in an A-IOT paging message.

Proposal 10. RAN2 study that the A-IOT device selects the access occasion within the number of access occasion by considering the A-IOT device’s energy level.

Proposal 11. In A-IOT paging message or start indication for the access occasion, the procedure type or number of messages is contained to determine whether to perform re-access procedure or not.

Proposal 12. If contention resolution or access procedure is failed, the A-IOT device perform the re-access procedure within the next round.

Proposal 13. The A-IOT paging message contains an indication to indicate a new next round.