# 3GPP TSG-RAN WG2 Meeting #127                                    R2-2407266

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item	: 7.11.2 (NR_MBS_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on multicast DRX to support NTN in INACTIVE

Document for	: Discussion and Decision

Introduction

In RAN2#126, the following agreements were made.

Agreements: 1 RAN2 confirms configuration of MBS Multicast in NR NTN can be supported for RRC connected mode in Rel-17/18 Spec. FFS for inactive in R18 2 RAN2 agrees to introduce an NTN specific HARQ RTT timer for PTM in 5.7b, e.g. HARQ-RTT-TimerDL-PTM-NTN. No new configuration in RRC will be added. We will introduce a capability. 3 drx-HARQ-RTT-TimerDL-PTM is extended by “UE-gNB” RTT and set the extended values to HARQ-RTT-TimerDL-PTM-NTN. 4 MBS Multicast HARQ configuration takes precedence.

In this contribution, we show our views on how to handle the drx-HARQ-RTT timerDL-PTM in INACTIVE.

Discussion

In RAN2#126 meeting, the extension of the drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerDL-PTM for MBS multicast DRX was agreed for UE in CONNECTED in order to support the MBS multicast DRX for the NTN. More specifically, if the UE receives the MBS downlink assignments from the NTN, the UE extends the drx-HARQ-RTT-TimerDL-PTM and drx-HARQ-RTT-TimerDL by considering the UE-gNB RTT.

Observation 1. In order to receive the MSB downlink assignments for retransmission in NTN, the UE in CONNECTED extends the drx-HARQ-RTT-TimerDL-PTM and drx-HARQ-RTT-TimerDL by considering the UE-gNB RTT

In Rel-18, even though the UE in RRC_INACTIVE cannot send HARQ feedback, it was agreed that the MBS multicast DRX in INACTIVE should follow MBS multicast DRX configuration. Since the HARQ retransmission can be requested by the UEs in CONNECTED, the UE in INACTIVE can receive the potential HARQ retransmission. For this reason, the UE in INACTIVE starts the drx-HARQ-RTT-TimerDL-PTM.

Observation 2. The UE in INACTIVE starts the drx-HARQ-RTT-TimerDL-PTM to receive the HARQ retransmission requested by the UEs in CONNECTED.

In our view, if the drx-HARQ-RTT-TimerDL-PTM is not extended for NTN, the UE in INACITVE may not receive the MBS downlink assignments for retransmission due to large propagation delay in NTN. More specifically, if the MBS downlink assignments are transmitted via NTN, the UE starts the drx-RetransmissionTimerDL and this timer expires during the RTT because the drx-HARQ-RTT-TimerDL-PTM is shorter than the RTT. In this case, the UE in INACTIVE cannot receive the retransmission of the MBS downlink assignments requested by the UEs in CONNECTED.

Thus, we think that the UE in INACTIVE should extend the drx-HARQ-RTT-TimerDL-PTM by considering the UE-gNB RTT if the MBS downlink assignments are received from the NTN.

Proposal 1. The UE in INACTIVE should extend the drx-HARQ-RTT-Timer-PTM by considering the UE-gNB RTT if the MBS downlink assignments are received from the NTN.

If the proposal 1 is agreeable, we need to discuss whether to introduce a new capability for extension of the drx-HARQ-RTT-TimerDL-PTM for the UE in INACTIVE. In our view, the new capability is not needed. Even though the network knows that the UE in INACTIVE supports the extension of and drx-HARQ-RTT-TimerDL-PTM, the network does not retransmit the MBS downlink assignments for the UE in INACTIVE.

Proposal 2. For extension of the drx-HARQ-RTT-Timer-PTM, the new capability for the UE in INACTIVE should not be introduced.

For proposal 1, we provide the text proposal for MAC in section 4.

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation 1. In order to receive the MSB downlink assignments for retransmission in NTN, the UE in CONNECTED extends the drx-HARQ-RTT-TimerDL-PTM and drx-HARQ-RTT-TimerDL by considering the UE-gNB RTT

Observation 2. The UE in INACTIVE starts the drx-HARQ-RTT-TimerDL-PTM to receive the HARQ retransmission requested by the UEs in CONNECTED.

Proposal 1. The UE in INACTIVE should extend the drx-HARQ-RTT-Timer-PTM by considering the UE-gNB RTT if the MBS downlink assignments are received from the NTN.

Proposal 2. For extension of the drx-HARQ-RTT-Timer-PTM, the new capability for the UE in INACTIVE should not be introduced.

Text proposal

5.7b	Discontinuous Reception (DRX) for MBS Multicast

For MBS multicast, the MAC entity may be configured by RRC with a DRX functionality per G-RNTI or per G-CS-RNTI that controls the UE's PDCCH monitoring activity for the MAC entity's G-RNTI(s) and G-CS-RNTI(s) as specified in TS 38.331 [5]. When in RRC_CONNECTED, if multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity is allowed to monitor the PDCCH for this G-RNTI or G-CS-RNTI discontinuously using the multicast DRX operation specified in this clause; otherwise the MAC entity monitors the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6]. When in RRC_INACTIVE, if the UE is configured to receive multicast in RRC_INACTIVE and multicast DRX is configured for a G-RNTI, the MAC entity is allowed to monitor the PDCCH for this G-RNTI discontinuously using the multicast DRX operation specified in this clause; otherwise the MAC entity monitors the PDCCH for this G-RNTI as specified in TS 38.213 [6]. The multicast DRX operation specified in this clause is performed independently for each G-RNTI or G-CS-RNTI and independently from the DRX operation specified in clauses 5.7 and 5.7a.

RRC controls multicast DRX operation per G-RNTI or per G-CS-RNTI by configuring the following parameters:

-	drx-onDurationTimerPTM: the duration at the beginning of a DRX cycle;

-	drx-SlotOffsetPTM: the delay before starting the drx-onDurationTimerPTM;

-	drx-InactivityTimerPTM: the duration after the PDCCH occasion in which a PDCCH indicates a new DL multicast transmission for the MAC entity;

-	drx-LongCycleStartOffsetPTM: the long DRX cycle drx-LongCycle-PTM and drx-StartOffset-PTM which defines the subframe where the long DRX cycle starts;

-	drx-RetransmissionTimerDL-PTM (per DL HARQ process for MBS multicast): the maximum duration until a DL multicast retransmission is received;

-	drx-HARQ-RTT-TimerDL-PTM (per DL HARQ process for MBS multicast): the minimum duration before a DL multicast assignment for HARQ retransmission is expected by the MAC entity.

The following MAC timer is used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-PTM-NTN (per DL HARQ process for MBS multicast): the minimum duration before a DL multicast assignment for HARQ retransmission is expected by the MAC entity.

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the Active Time includes the time while:

-	drx-onDurationTimerPTM or drx-InactivityTimerPTM or drx-RetransmissionTimerDL-PTM for this G-RNTI or G-CS-RNTI is running.

When multicast DRX is not configured for a G-RNTI or G-CS-RNTI, and the cfr-ConfigMulticast is configured for at least one of the active BWP(s) of the Serving Cell(s), and unicast DRX is configured, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if the PDCCH indicates a DL multicast transmission; or

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if the PDCCH addressed to G-RNTI indicates a DL multicast transmission; or

1>	if the PDCCH addressed to G-CS-RNTI indicates a DL multicast transmission and CS-RNTI is configured; or

1>	if a MAC PDU is received in a configured downlink multicast assignment and CS-RNTI is configured:

2>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]; and

2>	if HARQ feedback for MBS multicast is enabled:

3>	if the MAC PDU is received on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, and the cfr-ConfigMulticast is configured for at least one of the active BWP(s) of the Serving Cell(s), or when multicast DRX is configured for a G-RNTI for multicast in RRC_INACTVE, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	if HARQ feedback for MBS multicast is enabled:

3>	if the MAC PDU is received on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

4>	set HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL-PTM plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]; and

3>	if CS-RNTI is configured:

4>	if the MAC PDU is received on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

4>	else:

5>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else if drx-HARQ-RTT-TimerDL-PTM is configured:

3>	if the MAC PDU is received on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

4>	set HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL-PTM plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback that would be performed if HARQ feedback were enabled.

3>	else:

4>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback that would be performed if HARQ feedback were enabled.

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL-PTM expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL-PTM.

1>	if a HARQ-RTT-TimerDL-PTM-NTN expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-PTM-NTN.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received:

2>	stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI, or the corresponding G-CS-RNTI;

2>	stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI, or the corresponding G-CS-RNTI.

1>	if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM:

2>	start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe.

1>	if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI:

2>	monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL multicast transmission:

3>	if HARQ feedback for MBS multicast is enabled:

4>	if the PDCCH is indicated on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

5>	set HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL-PTM plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

4>	else:

5>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

4>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]:

5>	if the PDCCH addressed to G-RNTI indicates a DL multicast transmission; or

5>	if the PDCCH addressed to G-CS-RNTI indicates a DL multicast transmission and CS-RNTI is configured:

6>	if the PDCCH is indicated on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

7>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

7>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

6>	else:

7>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else if drx-HARQ-RTT-TimerDL-PTM is configured for multicast in RRC_INACTIVE, and the UE supports ptm-RetransmissionInactive:

4> if the PDCCH is indicated on a non-terrestrial network:

5>	set HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process equal to HARQ-RTT-TimerDL-PTM plus the latest available UE-gNB RTT value;

5> start the HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding multicast transmission.

4> else:

5>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding multicast transmission.

3>	else if drx-HARQ-RTT-TimerDL-PTM is configured:

4>	if the PDCCH is indicated on a non-terrestrial network and the UE supports harq-RTT-TimerDL-ForNTN-MulticastMBS-r17:

5>	set HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL-PTM plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-PTM-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback that would be performed if HARQ feedback were enabled.

4>	else:

5>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback that would be performed if HARQ feedback were enabled.

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a new multicast transmission for this G-RNTI or G-CS-RNTI:

3>	start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception.

NOTE 1:	A PDCCH indicating activation of multicast SPS is considered to indicate a new transmission.

NOTE 2:	The UE may start the drx-HARQ-RTT-TimerDL or HARQ-RTT-TimerDL-NTN after receiving a PTM transmission only if ptp-Retx-Multicast or ptp-Retx-SPS-Multicast was included in the UECapabilityInformation message to network.

The MAC entity needs not to monitor the PDCCH for a G-RNTI or a G-CS-RNTI if it is not a complete PDCCH occasion (e.g. the Active Time for a G-RNTI or a G-CS-RNTI starts or ends in the middle of a PDCCH occasion).