3GPP TSG-RAN WG2 Meeting #127	R2-2407269

Maastricht, Netherlands, 19 - 23 Aug, 2024

| Agenda Item: | 8.6.2 |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on inter-CU LTM |
| Document for: | Discussion and Decision |

1.	Introduction

In this paper, we discuss reference configuration for inter-CU LTM, RRC signaling structure, and inter-CU LTM with dual connectivity.

2.	Discussion

2.1.	Configuration for inter-CU LTM

Reference configuration

In Rel-18, a single reference configuration can be configured for intra-CU LTM. The reference configuration for intra-CU LTM is generated by the CU based on the information provided by the DUs associated with LTM candidate cells. Since the complete configuration of a candidate cell is generated by applying the candidate cell configuration on top of the reference configuration, each candidate cell configuration does not need to include the information included in the reference configuration. This implies the generation of reference configuration may require a complex implementation, e.g., multiple handshaking between the CU and the DUs to optimize the reference configuration and candidate cell configurations.

If a single reference configuration is forced to inter-CU LTM, the generation of reference configuration may become more complicated than in the intra-CU case because whole CUs and DUs are involved in generating the reference configuration. Further, it is likely that the commonality of candidate cell configurations between different CUs is very small because of, e.g., different capabilities of CUs involved in inter-CU LTM configuration. Consequently, the gain in signaling efficiency achieved by introducing reference configuration may be reduced.

Observation 1. A single reference configuration for inter-CU LTM would require joint signaling for the entire CU and DU involved in the LTM preparation, which may be too complicated to implement.

Observation 2. The gain in signaling efficiency achieved by using reference configuration may be marginal because the commonality of candidate cell configurations between different CUs may be very small.

Based on above observations, multiple reference configurations can be introduced to enjoy the benefit in signaling efficiency from using reference configuration while avoiding complicated network signaling. In Rel-18 LTM design, it seems that there are no serious problems in generating a single reference configuration for candidate cells within a CU. In other words, Rel-18 LTM principle says that a reference configuration is generated per CU. Maintaning the legacy principle and maximizing the gain from reference configuration, we propose that multiple reference configurations are provided for inter-CU LTM, where each reference configuration is generated per CU.

Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific.

If multiple configuration for inter-CU LTM is adopted, RAN2 would need to discuss whether or not the reference configurations for all involved CUs should be provided. Similar to Rel-18 LTM, it makes sense that the reference configuration associated with an involved CU is optionally provided. For example, in inter-CU LTM preparation, each involved CU determines whether or not the reference configuration is provided and, if so (i.e. determined to provide), generates the reference configuration by incorporating candidate cell configurations associated with the CU. In this example, inter-CU signaling is not necessary to generate a reference configuration.

Proposal 2. It is optional whether to provide a reference configuration for each CU involved in inter-CU LTM configuration.

Generation of complete configuration

In Rel-18 LTM, a candidate cell configuration can be provided as either complete or delta configuration. If the candidate cell configuration is provided as complete, the UE considers the candidate cell configuration as the complete configuration and directly applies the candidate cell configuration without pre-processing during LTM cell switch to the candidate cell. Otherwise (i.e., candidate cell configuration is delta), the UE generates the complete configuration by applying the candidate cell on top of the corresponding reference configuration and then applies the complete configuration during LTM cell switch to the candidate cell. We think Rel-18 LTM work for generating the complete configuration can be reused for Rel-19 inter-CU LTM without any additional work.

Proposal 3. Rel-18 LTM work for the generation of complete configuration is re-used for Rel-19 inter-CU LTM.

RRC signaling structure for inter-CU LTM configuration

If multiple reference configurations for inter-CU LTM is adopted, it may be necessary to introduce a new signaling for associating a candidate cell configuration with which reference configuration. For example, multiple reference configurations are provided as the list of reference configurations and a candidate cell includes the identifier of the corresponding reference configuration. In this case, during LTM cell switch to the candidate cell, the UE generate the complete configuration by applying the candidate cell configuration on top of the reference configuration indicated by the reference configuration identifier if the candidate cell configuration is delta. Of course, signaling methods other than that described in the above example may also be considered. For example, in [1], a new LTM configuration is proposed, where the LTM configuration includes the list of Rel-18 LTM configurations (i.e. intra-CU LTM configuration). To minimize the impact on RRC signaling, the Rel-18 LTM signaling structure should be re-used as much as possible.

Proposal 4. We propose RAN2 to discuss signaling method for association between reference and candidate cell configurations to minimize the impact on RRC signaling.

In terms of RRC signaling for LTM L1 measurement, one or more CSI resources for one or more LTM candidate cells are integratively provided as the form of list under the LTM configuration in Rel-18 LTM. Based on the integrated CSI resource pool for LTM L1 measurement (i.e., list of LTM CSI resources), the LTM CSI report configuration is configured in the CSI measurement configurations of LTM candidate cells and the current serving cell. To be specific, each LTM CSI report configuration refers to one or more CSI resources in the list of LTM CSI resources. In Rel-19 inter-CU LTM, in order for UE to measure and report of CSI resources of inter-CU LTM candidate cell, it is straightforward to provide one or more CSI resources for all LTM candidate cells in the form of a single list (i.e., same as Rel-18 LTM). If the CSI resources are provided separately per CU, then LTM CSI report configuration for inter-CU LTM candidate cell may become complex. For example, each LTM CSI report configuration should include e.g. CU identifier to refer where the configuration of CSI resources to be measured is provided.

Proposal 5. As in Rel-18 LTM configuration, CSI resources for LTM L1 measurement for all LTM candidate cells are provided in the form of a single list.

2.2.	Inter-CU LTM with dual connectivity

In [2], the following DC scenarios are considered for Rel-19 inter-CU LTM:

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In summary, the first scenario is inter-SN LTM without MN change and the second one is inter-MN LTM without SN change or with SN release.

Inter-SN LTM without MN change

In legacy, the SN change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another. Similar to the existing SN change procedure, it is straightforward that inter-SN LTM procedure is initiated either by MN or SN. We don’t see any blocking issue to follow the legacy principle.

Proposal 6. Inter-SN LTM procedure is initiated either by MN or SN.

The legacy SN change procedure always involves signalling over MCG SRB towards the UE. In other words, the target SN configuration of inter-SN change is provided to UE in MN format, i.e., the MN indicates the new configuration to the UE in the MN RRC reconfiguration message including the target SN RRC reconfiguration message. This is because MN configuration is usually changes with the SN change. In order for UE to apply both MN configuration and SN configuration at the same time, MN configuration and SN configuration are provided within the same container (i.e., MN RRCReconfiguration message). We believe that this principle equally applies to inter-SN LTM. That is, reference and candidate cell configurations for inter-SN LTM are provided to UE in MN format.

Proposal 7. Reference and candidate cell configurations for inter-SN LTM are provided to UE in MN format.

In terms of inter-node signaling, the preparation of inter-SN LTM configuration, the inter-SN LTM cell switch execution, and the inter-SN LTM cell switch completion can re-use Rel-18 S-CPAC work. This is because, in our view, the main difference between S-CPAC and inter-SN LTM comes from interaction between UE and the network rather than the inter-node signaling.

Proposal 8. Rel-18 S-CPAC works for inter-node signaling of preparation, handover execution, and handover completion are re-used for inter-SN LTM.

In Rel-18 S-CPAC, upon inter-SN S-CPAC execution, the UE uses the first unused sk-Counter value for S-KgNB generation, based on the per-SN pre-configured sk-Counter value list. It is FFS whether or not inter-SN LTM follows the security update of Rel-18 S-CPAC. Since S-CPAC is UE based mobility, SN security key update based on per-SN pre-configured sk-Counter value list is introduced for S-CPAC. On the other hand, it seems that inter-SN LTM does not need to follow the SN security update for S-CPAC because inter-SN LTM is network based mobility. Above all, SA3 agreed to start work on security procedure to support inter-CU LTM [3]. To be specific, the objective of SA3 task is as follows [4]:

The objective of this feature is to support the inter-CU LTM in 5G systems. This work item covers: WT1: Security procedure to support inter-CU LTM: - Case 1: When CU is acting as MN when DC is not configured - Case 2: NR-DC is configured and CU is acting as SN and MCG is unchanged - Case 3: NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Subsequent LTM mobility procedures between cell switches NOTE 1: SA3 work depends on the work progress in RAN WGs. NOTE 2: SA3 Work Tasks of this WID may be added/deleted/updated based on the revision to the RAN WID (NR mobility enhancements Phase 4).

Based on [3] and [4], RAN2 should wait for SA3 progress on security handling for inter-CU LTM with dual connectivity.

Proposal 9. RAN2 waits for SA3 progress on security handling for inter-CU LTM with dual connectivity.

Inter-MN LTM without SN change or with SN release

Based on [5], the SN security key (i.e., KSN) is derived based on the MN security key (i.e., KgNB). This implies that the SN security key should be updated in case of MN change without SN change because MN change leads to update of MN security key. Since inter-MN LTM cell switch is triggered by the network (i.e., source cell), the network can initiate the SN security key update by triggering e.g. SN modification procedure at an appropriate time. Therefore, there is no need of a new signaling for SN security key update at inter-MN LTM execution.

Proposal 10. Network ensures SN security key update at inter-MN LTM execution.

In Rel-18 LTM, a candidate cell configuration for intra-MN LTM does not include SCG configuration. This is because intra-MN LTM configuration including SCG configuration may make the RRC signaling for LTM configuration more complicated since subsequent mobility is baseline for LTM and the order of subsequent LTM based on pre-configured candidate cells is unpredictable. We believe it makes sense for Rel-19 inter-CU LTM to inherit the legacy principle, i.e., the configuration for inter-MN LTM does not include SCG configuration.

Proposal 11. Configuration for inter-MN LTM does not include SCG configuration.

To reduce data interruption due to SCG release, DRB(s) associated with SCG is(are) necessary to be relocated to MCG before SCG release. For example, if the SCG release command and the new DRB configuration are provided to UE together, the DRBs associated with SCG are relocated to MCG with minimal interruption. However, if UE autonomously releases SCG at inter-MN LTM execution, data throughput may be degraded due to late DRB relocation. To be specific, the UE autonomously releases SCG at LTM execution and then the DRBs associated with SCG are relocated to MCG after LTM completion, which means that the UE may experience data interruption between LTM execution and LTM completion. In addition to the DRB configuration, there may be other configurations necessary to be updated upon SCG release. In this reason, it is desirable that the UE only releases SCG configuration at inter-MN LTM execution if SCG release is configured by the network.

Proposal 12. UE only releases SCG configuration at inter-MN LTM execution if SCG release is configured by the network.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Observation 1. A single reference configuration for inter-CU LTM would require joint signaling for the entire CU and DU involved in the LTM preparation, which may be too complicated to implement.

Observation 2. The gain in signaling efficiency achieved by using reference configuration may be marginal because the commonality of candidate cell configurations between different CUs may be very small.

Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific.

Proposal 2. It is optional whether to provide a reference configuration for each CU involved in inter-CU LTM configuration.

Proposal 3. Rel-18 LTM work for the generation of complete configuration is re-used for Rel-19 inter-CU LTM.

Proposal 4. We propose RAN2 to discuss signaling method for association between reference and candidate cell configurations to minimize the impact on RRC signaling.

Proposal 5. As in Rel-18 LTM configuration, CSI resources for LTM L1 measurement for all LTM candidate cells are provided in the form of a single list.

Proposal 6. Inter-SN LTM procedure is initiated either by MN or SN.

Proposal 7. Reference and candidate cell configurations for inter-SN LTM are provided to UE in MN format.

Proposal 8. Rel-18 S-CPAC works for inter-node signaling of preparation, handover execution, and handover completion are re-used for inter-SN LTM.

Proposal 9. RAN2 waits for SA3 progress on security handling for inter-CU LTM with dual connectivity.

Proposal 10. Network ensures SN security key update at inter-MN LTM execution.

Proposal 11. Configuration for inter-MN LTM does not include SCG configuration.

Proposal 12. UE only releases SCG configuration at inter-MN LTM execution if SCG release is configured by the network.

4.	References

[1] R2-2405666, On inter-CU aspects for LTM, Nokia, RAN2#126

[2] RP-241515, Revised Work Item: NR mobility enhancements Phase 4

[3] R2-2406244, Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases (S3-242400; contact: Apple), Apple

[4] S3-242401, New WID on security aspects of NR mobility enhancement

[5] TS 33.501 v18.6.0 Security architecture and procedures for 5G system (Release 18)

5.	Annex

5.1.	RAN2 agreements

RAN2#125bis:

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

- FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

RAN2#126:

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

- For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

- Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

- FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

- Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

- RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

- For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

5.2.	RAN3 agreements

RAN3#123bis:

- Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

- Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

- Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

- Early data forwarding can be supported for inter-CU LTM.

RAN3#124:

- Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

- Introduce a new procedure on Xn to transfer the TA information.

- A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

- Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

- The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

- Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

- Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

- A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own.