3GPP TSG-RAN WG2 #127	R2-2407274

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on scheduling enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4		Scheduling enhancements

- Introduction

In RAN #103, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions.

And the following agreements related to LCP enhancements were reached in RAN2#126 meeting.

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In this contribution, we provide the discussion on LCP enhancements for UL scheduling.

- Discussion

In Rel-18, Delay Status Report (DSR) was introduced to support UL scheduling for XR services. DSR can provide a serving gNB with delay status including the remaining time and the total amount of delay-critical UL data. Based on the DSR, the gNB can schedule UL resources for UEs which have data with short remaining time to improve capacity. However, according to the current LCP procedure, the UE allocates the resources to each logical channel based on its priority without considering the delay status. Therefore, the UE may delay the transmission of the delay-critical data if there is another data from logical channels with a higher priority. As a result, the delay-critical data may be discarded due to the timer expiry, even though the DSR has been reported.

To address the issue, delay-aware LCP enhancements can be supported in Rel-19. One potential solution is to introduce a new LCP mapping restriction. For example, the UE can select only logical channels with delay-critical data during the LCP procedure so that UL resources are not allocated for other data. In this case, the legacy LCH selection mechanism can be reused to have minor spec impacts. However, this solution may have a negative impact on NW capacity. If more UL resources are allocated than are needed for the delay-critical data, the remaining resources may be wasted. It was proposed to allow the remaining resources to be used for other data, but this would complicate the UE behavior in the LCP procedure. Furthermore, SRB traffic can also be affected. Since the discard timer is configured only for DRBs, the traffic from SRBs would always be considered as non-delay critical. As a result, the LCP restriction based on the delay status may prioritize the DRB traffic and delay the SRB traffic.

Observation 1:	LCP mapping restriction based on the delay status may have a negative impact on NW capacity and SRB traffic.

Another direction for LCP enhancements is to adjust the LCH priority based on the delay status. For example, an additional LCH priority with a higher level can be configured. If delay-critical data is buffered for a logical channel, the additional priority can be used for the logical channel instead of the legacy priority. The adjustment to the higher priority can provide more chances to transmit the delay-critical data. This solution can be implemented by adding a step to determine which LCH priority to use before resource allocation procedure, which would not introduce much complexity. Only simple specification changes would be needed without affecting the legacy LCP procedure (i.e., selection of LCHs and resource allocation). As for the impact on SRB traffic, it can be avoided by configuring additional LCH priorities so that DRBs have a lower priority level than SRBs. From the above, the LCH priority adjustment based solution would be preferred over the LCP restrictions based solution.

Observation 2:	LCH priority adjustment based on the delay status can provide more chances to transmit delay-critical data without introducing much complexity and without affecting SRB traffic.

Proposal 1:	RAN2 to support LCH priority adjustment based solution rather than LCP restrictions based solution.

In our understanding, there is no need to introduce a separate remaining time threshold for delay-aware LCP. The separate threshold may have a specification impact not only in MAC but also in PDCP/RLC. Therefore, it would be preferrable to reuse the threshold configured for Rel-18 DSR. In other words, LCH priority adjustments can be triggered if the LCH has data whose remaining time is below the R18 threshold (i.e., the LCH has delay-critical data).

Observation 3:	Introducing a separate remaining time threshold for delay-aware LCP may have an extra specification impact not only in MAC but also in PDCP/RLC.

Proposal 2:	The remaining time threshold configured for DSR can be reused for triggering LCH priority adjustments.

LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization). If the LCH priority adjustment is supported, it needs to be discussed whether the additional LCH priority can be used for those purposes as well. For BSR trigger, there seems to be no advantage to using the additional priority. BSRs should always be triggered based on the legacy priority to avoid complexity.

Observation 4:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Proposal 3:	LCH priority adjustment should not be applied to BSR trigger, even if supported for delay-aware LCP.

On the other hand, for intra-UE prioritization, it may be useful to adjust the LCH priority based on the delay status. In intra-UE prioritization, if there are overlapping UL grants, only the UL grant with the highest priority is considered as a prioritized UL grant and can be used for transmission. The priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed or have data available that can be multiplexed in the MAC PDU. This can lead to the same problem as in the LCP procedures. In other words, an UL grant for which data from LCHs with a higher priority are multiplexed may de-prioritize another UL grant for which delay-critical data from LCHs with a lower priority are multiplexed. Therefore, it could be considered to apply LCH priority adjustment to intra-UE prioritization as well.

Observation 5:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant with a higher priority.

Proposal 4:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- Summary and proposal

In this contribution, we provided the following observations and proposals.

Observation 1:	LCP mapping restriction based on the delay status may have a negative impact on NW capacity and SRB traffic.

Observation 2:	LCH priority adjustment based on the delay status can provide more chances to transmit delay-critical data without introducing much complexity and without affecting SRB traffic.

Proposal 1:	RAN2 to support LCH priority adjustment based solution rather than LCP restrictions based solution.

Observation 3:	Introducing a separate remaining time threshold for delay-aware LCP may have an extra specification impact not only in MAC but also in PDCP/RLC.

Proposal 2:	The remaining time threshold configured for DSR can be reused for triggering LCH priority adjustments.

Observation 4:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Proposal 3:	LCH priority adjustment should not be applied to BSR trigger, even if supported for delay-aware LCP.

Observation 5:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant with a higher priority.

Proposal 4:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- References

[1]	RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3