3GPP TSG-RAN WG2 Meeting #127               	R2-2407279
Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.4

Source: 	Meta

Title:  	Discussion on Scheduling Enhancement for XR

Document for:	Discussion and Decision

# Introduction

In RAN #102 plenary meeting, it was agreed that one of the WI objectives in the Phase 3 XR WID is [1]:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

In RAN2 #126, the following agreements were made:

Agreement - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In this paper, we discuss scheduling enhancements for XR.

# Discussion

Release 18 introduced the Delay Status Report (DSR) to provide feedback on buffered data remaining time when a UE requests uplink resources, allowing gNB to perform delay-aware uplink scheduling. However, a limitation is that when multiple PDU sets in a LCG have different remaining times, only the shortest remaining time below the threshold is reported. To address this, consider enhancing DSR to report remaining time with finer granularity and potentially additional information such as multiple pairs of remaining time and buffer details. Meanwhile, it is important for DSR to avoid introducing excessive signal overhead.

Observation 1: For enhancing DSR to report with multiple pairs of remaining time and buffer size for the LCG, signal overhead needs to be considered.

In RAN2 #125-bis and #126 meetings, incorporating the importance information in DSR was discussed. In Rel-18, PDU Set Importance (PSI) is used during congestion when performing discard operations. Given the unclear benefits of PSI in DSR and the desire of keeping delay-critical and importance requirements separate, it is not advisable to integrate PSI information within DSR.

Proposal 1: No need to enhance DSR to incorporate importance information.

# Conclusion

In this paper, we discuss scheduling enhancement for XR.

Observation 1: For enhancing DSR to report with multiple pairs of remaining time and buffer size for the LCG, signal overhead needs to be considered.

Proposal 1: No need to enhance DSR to incorporate importance information.

# References

[1] RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”