3GPP TSG-RAN WG2 Meeting #127	R2-2407283

Maastricht, Netherlands, 19th Aug – 23rd Aug, 2024

Agenda Item		:	8.2.3 (FS_Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on A-IoT paging

Document for	:	Discussion and Decision

1.	Introduction

In RAN2#126 meeting [1], the following agreements were reached regarding the Ambient IoT (A-IoT or AIoT) paging.

Agreements – RAN2#126 - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this contribution, we further discuss the issues related to A-IoT paging.

2.	Discussion

2.1.	Content required for determination of transmission resource

In RAN2#126 meeting [1], the following agreements on the different identified A-IoT procedures (i.e., inventory, inventory and command, command only) were made.

- As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. - From RAN2 point of view we will study “Command only” use case. - FFS the options on how to support it: - Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. - Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command)

In the “inventory only” and “inventory and command” cases, procedures commonly begin with Step A, A-IoT paging. After this, subsequent steps proceed, possibly including random access (RA), device ID transmission, and data transmission between reader and device. In the “command only” case, although the specific steps are FFS, the procedure begins with A-IoT paging (i.e., the transmission of the initial trigger message).

In Step B, where device ID transmission for inventory is performed, contention between devices could occur if multiple devices are triggered in Step A (e.g., when A-IoT paging message contains a group ID that maps to multiple A-IoT devices). RA procedure could be used for contention resolution. RAN2 considers slotted ALOHA as the baseline for such RA procedure, which provides multiple access occasions (i.e., slots), allowing devices to randomly choose one for access to prevent collisions. Our detailed view on the RA procedure for A-IoT is provided in our companion paper [2].

We note that, for A-IoT devices to choose a random access occasion properly, the number of access occasions should be known. Otherwise, the A-IoT devices would not know within which range to choose the access occasion. In the cases mentioned in the above agreements (e.g., “inventory only” and “inventory and command”), A-IoT paging in Step A comes before and triggers RA procedure in Step B. Thus, the appropriate method to indicate the number of access occasion before RA procedure begins is through A-IoT paging. Therefore, we derive Proposal 1.

Proposal 1: A-IoT paging message should include the number of access occasions for the subsequent RA procedure, so that A-IoT devices are aware of the number of access occasions.

Meanwhile, as mentioned in Section 1, in the RAN2#126 meeting, RAN2 agreed to study a case where A-IoT paging message contains multiple IDs of A-IoT devices. The need for this use case remains unconfirmed, pending the SA2 discussion. At the same time, RAN2 agreed that A-IoT paging message should indicate information that allows the device to determine resources to be used for response.

We think that the case of A-IoT paging message including multiple IDs of devices could indicate information for devices’ resource determination for RA procedure. For example, A-IoT devices can choose their access occasion based on the order of the IDs listed in the A-IoT paging message. For instance, the device matched with the first ID selects the first access occasion, and the device matched with the third ID selects the third access occasion. Given this, we think that RAN2 could confirm the need for this A-IoT paging case without waiting for the SA2 discussion.

Proposal 2: Given that A-IoT paging message including multiple IDs of A-IoT devices could indicate information for resource determination in the RA procedure, RAN2 should confirm the need for A-IoT paging message to include multiple IDs of A-IoT devices without waiting for the SA2 discussion.

In RAN1#116bis meeting [3], RAN1 reached an agreement on the frequency-domain multiple access for A-IoT as follows.

Agreement – RAN1#116bis - Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS

According to this RAN1 agreement, frequency-domain multiple access can be introduced to device-to-reader (D2R) transmissions. This implies that A-IoT procedures can be performed across multiple different frequencies. Therefore, it is necessary for RAN2 to explore ways to incorporate frequency-domain multiple access into the A-IoT procedure. Considering that the A-IoT procedure, especially A-IoT RA procedure, is initiated by the A-IoT paging, A-IoT paging message should include frequency information for performing the RA procedure

Proposal 3: A-IoT paging message should include frequency information for performing the A-IoT RA procedure.

2.2.	Content required for A-IoT across different procedures

As mentioned in Section 2.1, RAN2 agreed on the different identified A-IoT procedures, such as “inventory only”, “inventory and command”, and “command only”. We note that, failure could occur during A-IoT procedure.

In the “inventory and command” case, the A-IoT device may not know whether the command procedure will follow the inventory procedure unless explicitly indicated. During the A-IoT procedure, transmission failures can occur due to reasons like contention resolution failure, channel unreliability, or CRC errors. If a transmission failure occurs during the command procedure, the reader may detect it, but the A-IoT device may not, potentially leading the A-IoT device to incorrectly assume that the overall procedure completed successfully. A more detailed discussion of this transmission failure issue is provided in our companion paper [2].

Observation 1. Without knowing the type of A-IoT procedure, the A-IoT device may not detect a failure in the command procedure following the inventory procedure, and therefore cannot handle the failure effectively.

To help A-IoT devices effectively manage such failure issues, we propose a simple solution: include the A-IoT procedure type (or the number of messages for the procedure) in the A-IoT paging message. This information could enable the A-IoT device to determine whether the command procedure has been completed successfully and thus handle failures appropriately.

Proposal 4: A-IoT paging message should include information indicating the type of ongoing procedure, or at least the number of messages to be involved in that procedure.

For A-IoT devices that detect a failure in the command procedure using the indication proposed in Proposal 4, it is necessary for them to re-access to complete the unfinished procedure. We believe that such re-access should occur in the next distinct random access round, rather than in the same round where the failure occurred, to prevent further contention and conserve device energy (see our companion paper [2]). At the same time, devices that have successfully completed the procedure should not access the next round reserved for devices that experienced failure, for the same goal of preventing further contention and energy wastage.

To ensure only devices that experienced a procedure failure access the next access round, it should be distinguishable to the A-IoT devices whether the round is reserved for re-access by failed devices. Therefore, we derive Proposal 5.

Proposal 5: A-IoT paging message should include an indication of whether the access round triggered by the A-IoT paging message is reserved for re-access by devices with procedure failure.

We note that, Proposal 5 could be utilized to resolve A-IoT paging message missing problem as well. A-IoT devices may miss A-IoT paging messages due to issues such as insufficient energy, poor channel reliability, or CRC errors, etc. Therefore, it is necessary to consider methods for triggering all A-IoT devices targeted by A-IoT paging messages. To this end, A-IoT paging messages could be re-transmitted multiple times for the same A-IoT service, aiming to trigger re-access from devices that failed to receive the A-IoT paging message. Through this, devices that missed the A-IoT paging message can have another chance to receive it. For example, devices that failed to receive the A-IoT paging message due to low energy levels can catch up after recharging.

For the efficient operation of such paging re-transmissions and subsequent procedures, A-IoT devices that have already responded to the A-IoT paging message should refrain from responding again. Based on the re-access indication of Proposal 5, A-IoT devices could distinguish between re-transmitted A-IoT paging messages that may contain re-access indications (i.e., those they have already responded to) and new ones.

3.	Conclusion

This contribution makes the following observations and proposals:

Proposal 1: A-IoT paging message should include the number of access occasions for the subsequent RA procedure, so that A-IoT devices are aware of the number of access occasions.

Proposal 2: Given that A-IoT paging message including multiple IDs of A-IoT devices could indicate information for resource determination in the RA procedure, RAN2 should confirm the need for A-IoT paging message to include multiple IDs of A-IoT devices without waiting for the SA2 discussion.

Proposal 3: A-IoT paging message should include frequency information for performing the A-IoT RA procedure.

Observation 1. Without knowing the type of A-IoT procedure, the A-IoT device may not detect a failure in the command procedure following the inventory procedure, and therefore cannot handle the failure effectively.

Proposal 4: A-IoT paging message should include information indicating the type of ongoing procedure, or at least the number of messages to be involved in that procedure.

Proposal 5: A-IoT paging message should include an indication of whether the access round triggered by the A-IoT paging message is reserved for re-access by devices with procedure failure.

4.	References

- Draft Report of 3GPP TSG RAN WG2 meeting #126

- R2-2407265, “Discussion on random access aspects for Ambient IoT”, LGE

- Final Report of 3GPP TSG RAN WG1 #116b v1.0.0

- (draft) RAN3 #124 Meeting Report