3GPP TSG-RAN WG2 Meeting #127                                   R2-2407286

Maastricht, Netherlands, 19th Aug. – 23th Aug, 2024

Agenda Item	: 8.4.4

Source	: LG Electronics Inc.

Title	: Discussion on Procedures for UE MR PDCCH monitoring triggered by LP-WUS in RRC_CONNECTED Mode

Document for	: Discussion and Decision

Introduction

In the RAN#104 meeting, the following is left as the remaining open issue for the RAN2-related aspects of LP-WUS in RRC_CONNECTED mode:

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

In this contribution, we would like to clarify and show our views on the RAN2-related aspects of LP-WUS in RRC_CONNECTED mode.

Discussion

From the last RAN2#126 meeting, RAN2 has started to discuss the details of the LP-WUS operation methods identified in RAN1#116bis meeting. The agreement made for Option 1-1 is as follows:

Agreement

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

As a continuation of the last meeting, it would be appropriate to confirm the companies' understanding of the remaining options, Option 1-2-1, Option 1-2-2, and Option 1-3, and then discuss which option is the optimal.

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS

Option 1-2 allows UE to monitor LP-WUS outside at least legacy C-DRX active time. When LP-WUS is detected outside the legacy C-DRX active time, UE moves to either the legacy DRX active time or the new DRX active time, depending on the situation. Option 1-2 includes two variants, Option 1-2-1 and Option 1-2-2, which depend on whether the legacy DRX On Duration is configured for PDCCH monitoring or not.

In Option 1-2-1, the legacy DRX On Duration is configured for both PDCCH monitoring and CSI/SRS reporting. PDCCH monitoring will be performed during both the legacy DRX active time and the new DRX active time by receiving LP-WUS. To achieve power-saving gain compared to legacy C-DRX, Option 1-2-1 should be configured together with Option 1-1 as shown in the RAN1 agreement. Each LP-WUS monitoring occasion should encompass the period just before every legacy DRX On Duration to facilitate potential PDCCH monitoring during the legacy DRX On Duration. Unlike Option 1-1, where the PDCCH monitoring interval fully overlaps the duration of the existing DRX, the PDCCH monitoring interval in Option 1-2-1 can fully overlap, partially overlap, or not overlap the duration interval of the existing DRX, depending on the configuration.

< Option 1-2-1 configured together with Option 1-1 >

In Option 1-2-2, unlike other options, the legacy DRX On Duration is not configured for PDCCH monitoring but for DRX related operation other than PDCCH monitoring (e.g., CSI/SRS reporting). This means that the PDCCH monitoring operation is performed according to the additional configuration (e.g., introduction of new DRX active time and parameters for the new DRX active time) only when LP-WUS is received, regardless of the DRX On Duration. Moreover, further discussion on how to perform DRX related operation other than PDCCH monitoring (e.g., CSI/SRS reporting) in legacy DRX On Duration will be needed. Unlike Option 1-1, where the PDCCH monitoring interval fully overlaps the duration of the existing DRX, the PDCCH monitoring interval in Option 1-2-2 can fully overlap, partially overlap, or not overlap the duration interval of the existing DRX, depending on the configuration. Additionally, for Option 1-2-2, we think that the combination with Option 1-3 should be considered, given that the decision to monitor PDCCH should be independent of the legacy DRX On Duration.

< Option 1-2-2 >

In Option 1-3, the UE monitors LP-WUS at least during the DRX On Duration. The main intention of Option 1-3 is to utilize LP-WUS as a way for a UE that received a PDCCH skipping indication to wake up earlier during micro-sleep state. If the UE that started PDCCH monitoring according to legacy DRX behavior receives a PDCCH skipping indication via DCI, the UE may monitor for LP-WUS and resume PDCCH monitoring when LP-WUS is detected.

< Option 1-3 >

Comparison among Options

Among the four options, we first want to eliminate Option 1-3 from consideration. This is because Option 1-3 offers marginal power-saving gain only in the specific scenario. The power-saving gain of Option 1-3 is negligible compared to the legacy DRX operation, as it can only be achieved during legacy DRX active time only when the PDCCH skipping indication exists.

Of the remaining three options, we believe that Option 1-1 can achieve similar performance to Option 1-2-1 and Option 1-2-2 in terms of power consumption and data latency. This can be done by shortening the length of the DRX cycle by the LP-WUS monitoring cycle in a DRX configuration without specification impact as in the figure below.

Both Option 1-2-1 and Option 1-2-2 require additional configuration for PDCCH monitoring behavior to receive data outside the legacy DRX active time. This includes the introduction of new DRX active time and parameters for the new DRX active time, which could lead to specification changes. In addition, in Option 1-2-2, the distinction between ‘legacy DRX behavior besides PDCCH monitoring’ and ‘PDCCH monitoring behavior’ would lead a huge specification impact. For example, if there is no condition for CSI/SRS reporting, Option 1-2-2 is not a good choice in terms of power saving because the UE's MR is always on during all legacy DRX active time to perform CSI/SRS reporting. To address this issue, it would be necessary to separately define whether/how CSI/SRS reporting should be performed at each DRX active time interval.

In conclusion, since the similar performance to Option 1-2-1 and Option 1-2-2 can be achieved through DRX configuration in Option 1-1, there seems to be no reason to choose Option 1-2-1 nor Option 1-2-2 while enduring the spec impact. We believe that a well-configured DRX configuration in Option 1-1(with a very short DRX cycle) is sufficient to provide reliable, low-latency data reception without any specification changes.

Proposal 1. The option that LP-WUS replaces DCP is the optimal option for the procedure to allow UE MR PDCCH monitoring triggered by LP-WUS.

Conclusion

Based on the above discussion, we made the following proposal.

Proposal 1. The option that LP-WUS replaces DCP is the optimal option for the procedure to allow UE MR PDCCH monitoring triggered by LP-WUS.

References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power wake up signal and receiver for NR