3GPP TSG RAN WG2 Meeting #127.                                                                        R2-2407287

Maastricht, NL, August 19th – 23rd, 2024

Source:	Meta

Title:	Discussion on RRM Measurement Prediction Framework

Agenda item:	8.3.2.2

Document for:	Discussion and Decision

# Introduction

In Rel-19, RAN2 has been tasked with studying the potential benefits of AI/ML aided Mobility [1] with the following objectives:

Objective: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2]

In this paper, we provide views on the general AI/ML framework for RRM measurement prediction.

# Discussion

The following cases for RRM prediction based on AI/ML were agreed in previous RAN2 meetings

- For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal prediction

The agreed cell-level measurement cases 1-3 have different requirements for model input and outputs related to beam and cell-level measurements.

## Measurement Quantities for Model Input/Output

For RRM prediction model case 1 and 3 the model input is based on beam level measurements. The beam level measurements are typically L1 measurements and in this context L1-RSRP measurements should be used. Since the context of the measurements is applicable for mobility however, we expect raw L1 measurements to have too much variability leading to difficulty in model training and inference. Therefore, for case 1 and 3, filtered L1-RSRP should be considered as model input. The following agreement was made with respect to simulation assumptions for RRM prediction evaluation

- For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation.

While simulations can assume company specific L1 filtering, eventual specification should assume some standardized filtering of L1 measurements to ensure consistency for different models and overall RRM prediction framework. Additionally, L1 filtered RSRP should be available at a much smaller timescale than L3 filtered beam level RSRP which is inherently a slow process.

- For RRM prediction case 1 and 3, model input should consider filtered L1-RSRP with standardized and configurable filter coefficients.

For case 1, the model output is beam-level and we think the same L1 filter should be applied for model output for data collection as the model inputs i.e., the model output for case 1 should be filtered L1-RSRP which can be consolidated with additional filtering to derive cell-level results. It should be further discussed if the coefficients for L3 filtering and beam consolidation in case 1 needs to be aligned for consistency. For case 2/3 the model output is cell level and should be considered as filtered L3 RSRP.

- For RRM prediction case 1, the model output should be filtered L1 RSRP and the data for model training (set A of beams) should apply the same L1 filter as the model input (set B of beams)

    - FFS: If L3 filtering of model output needs to be aligned for consistency.

- For RRM prediction case 2/3, the model output may be filtered L3 RSRP.

## UE and Network Side Models

For UE sided model, the RRM prediction for case 1-3 is done at the UE based on network configuration. The configuration may be based on model selection/activation using model ID or associated ID or may be based on providing parameters to the UE e.g., prediction type (spatial/temporal/frequency domain), prediction window etc. which can be used by the UE to perform inferencing using an appropriate model. RAN2 should further discuss whether model selection/activation at UE side is controlled by gNB or whether such functionality is UE autonomous.

- For UE side models, network may either provide configuration for RRM predictions using which UE can make appropriate inferencing, or network may be able to perform model selection/activation based on associated ID where the selected model will provide the appropriate prediction.

Based on the type of prediction i.e., case 1/2/3, the UE may need to report either beam-level or cell-level measurement predictions back to the gNB. For case 1, it should be further discussed whether the UE is responsible for filtering/consolidating the beam level model outputs and then reporting the cell-level results to the network. If beam level results are reported, the current L1 beam reporting framework can be utilized

- For Case 1, further discuss if model output or beam level output is reported by the UE to the network or if cell level metrics are reported based on model output.

- L1 beam reporting framework for LTM may be used for RRM prediction inference reporting to the gNB for UE side models.

For network side models, if beam level metrics are used as input as in case 1/3, the network can configure the UE to apply L1-filtering to the reported measurement results which are used for inference on the network side. For this L1 filtering approach may need to be specified. Such filtering configuration can also ensure consistency of data for training and inference since different filtering assumptions during training data collection and inference data collection may lead to sub-optimal AI/ML model performance.

- For network side models, filtering of measurements before reporting can be configured to the UE by the gNB. The filtering may be L1-filtering of beam level measurements.

If network expects UE to report any assistance information, it should be ascertained that such assistance does not disclose any proprietary information.

## Cluster based RRM Prediction

Clustering based approaches i.e., predicting RRM measurements for a cell based on measurement results from other can be useful since it may help simplify deployment by leading naturally to model generalization across multiple cells. However, this approach should be carefully evaluated for single cell RRM measurement framework against cell specific models. For the case when measurement event prediction is the goal and not only RRM prediction, the cluster based approach should lead to a robust handover framework.

- Consider cluster-based measurement as input for AI/ML models for measurement event prediction. Further evaluate usefulness for RRM prediction within a cell.

## System KPIs

To ensure efficient handovers are predicted using AI/ML model outputs for AI/ML aided mobility, RAN2 should consider HO Ping pong rate as an important KPI.

HO ping pong rate is important especially if raw L1 measurements are used as input. To this end, alignment on MTS (minimum-time-to-stay) should be reached across companies for evaluation. Recommended value of 1s from TR36.839 can be used as a starting point.

- HO ping pong rate should be used as KPI for AI/ML based RRM prediction framework with minimum-time-to-stay (MTS) alignment across companies. A default MTS of 1 sec can be used as a starting point.

# Conclusion

The main proposals from this paper are outlined here:

- For RRM prediction case 1 and 3, model input should consider filtered L1-RSRP with standardized and configurable filter coefficients.

- For RRM prediction case 1, the model output should be filtered L1 RSRP and the data for model training (set A of beams) should apply the same L1 filter as the model input (set B of beams)

    - FFS: If L3 filtering of model output needs to be aligned for consistency.

- For RRM prediction case 2/3, the model output may be filtered L3 RSRP.

- For UE side models, network may either provide configuration for RRM predictions using which UE can make appropriate inferencing, or network may be able to perform model selection/activation based on associated ID where the selected model will provide the appropriate prediction

- For Case 1, further discuss if model output or beam level output is reported by the UE to the network or if cell level metrics are reported based on model output.

- L1 beam reporting framework for LTM may be used for RRM prediction inference reporting to the gNB for UE side models.

- For network side models, filtering of measurements before reporting can be configured to the UE by the gNB. The filtering may be L1-filtering of beam level measurements.

- Consider cluster-based measurement as input for AI/ML models for measurement event prediction. Further evaluate usefulness for RRM prediction within a cell.

- HO ping pong rate should be used as KPI for AI/ML based RRM prediction framework with minimum-time-to-stay (MTS) alignment across companies. A default MTS of 1 sec can be used as a starting point.

# References

- RP-240082, Revised SID on AI/ML for Mobility in NR, OPPO (Rapporteur), 3GPP TSG RAN Meeting#103, Maastricht, Netherlands, March, 2024.

- Chair Notes, 3GPP RAN2 Meeting#126, Fukuoka City, Japan, May 2024.

- Chair Notes, 3GPP RAN2 Meeting#125bis, Changsha, China, April 2024.