3GPP TSG RAN WG2 Meeting #127.                                                                        R2-2407289

Maastricht, NL, August 19th – 23rd, 2024

Source:	Meta

Title:	Evaluation Assumptions for RLF/HO Failure Prediction

Agenda item:	8.3.4.1

Document for:	Discussion and Decision

# Introduction

In Rel-19, RAN2 has been tasked with studying the potential benefits of AI/ML aided Mobility [1] with the following objectives:

Objective: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2]

In this paper, we provide views on the evaluation assumptions for RLF/HO failure prediction.

# Discussion

RAN2 has made the following agreements with respect to the scenarios that will be prioritized for RLF/HO failure prediction:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is down prioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

Based on the agreements, HO failure prediction is deprioritized and only RLF prediction in FR2 is prioritized. To this end, two methods for RLF prediction are identified e.g., indirect method where RLF is predicted based on temporal domain serving cell measurement predictions and direct method which directly provides RLF prediction. We provide our view on these problems and evaluation assumptions in the following.

## AI/ML based RLF Prediction

The following figure illustrates the RLF procedure based on the RLF Timer T310 which is triggered when the link quality deteriorates below Qout(<=-8dB). RLF is declared with T310 expires and this case is prioritized in RAN2.

Figure 1: RLF procedure (from TR 36.839)

In 36.839, two handover states are defined under which RLF is logged

- State 1: Before the event A3 entering condition is satisfied.

- State 2: After the event A3 entering condition is satisfied but before the handover command is successfully received by the UE.

For RAN2 study, we should concentrate on the RLF events in State 1 when A3 entering condition is not triggered. The RLF events in this state may be true RLF events due link conditions at the UE or can be considered as handover failure when another cell with SINR>-8dB is available. Both these conditions should be studied since the goal of RLF prediction in this study is to facilitate mobility.

- Study RLF failure prediction in State 1 (before event A3 entering condition is satisfied) and consider both cell-specific and multi-cell measurements to determine true RLF vs Handover failure.

For UE-side models performing direct RLF event prediction, several variables (e.g., timers, prediction window etc.) need to be configured. Data collection for such model training must be under the assumption that true RLF occurrences can be logged and collecting such data synthetically may need accurate modeling of the different RLF aspects in Figure 1.

- Synthetic data collection for direct RLF prediction may require accurate modeling of in-sync/out-of-sync and RLF timers in simulators as well as the necessity of true RLF events for supervised training of AI/ML models.

On the other hand, for indirect RLF prediction, the RRM measurement prediction framework can be potentially reused, and generation and collection of synthetic data for true RLF conditions and handover failure conditions under state 1 may be relatively easier using SLS. The evaluation assumptions from RRM measurement prediction may be reused as a starting point.

- Data collection for indirect RLF prediction may be similar to RRM measurement case.

- For Indirect RLF predictions re-use evaluation assumptions from RRM measurement prediction as a starting point.

## RLF Modeling in Simulations

### RLF Parameters

In TR 36.839, the following table is provided for RFL modelling.

The main difference in the scenario for TR 36.839 and the current study is the table considers LTE in FR1 whereas the current study will prioritize FR2 intra-frequency cases. The first parameter to consider in this case is the Qout. During the IMT-2020 evaluation calibration campaign, 3GPP evaluated DL geometry for FR2 scenarios and the calibration results from [4] is shown in Figure 2.

Figure 2: DL Geometry for Dense Urban – eMBB with red curve showing the SINR for FR2 30GHz CF

(TR 37.910 Figure A.2)

From the figure, we can see that the tail of the distribution is significantly less than the -8dB threshold defined in 36.839. Therefore, we can reuse this assumption as a starting point for further evaluation.

- Based on calibrations from IMT-2020, Qout threshold of -8dB can be used as a starting point for FR2 evaluations

Next, we should reconsider the values of T310 since the time of 1s in the table would be quite large for FR2. Given we are evaluating L1 measurement periodicity of 20/40ms, the T310 should be smaller multiple of this value i.e., T310 in the range of 200ms is more suitable for evaluations.

- Consider T310 values which are smaller multiples of L1 measurement periodicity e.g., 200ms as a starting point.

### Data Collection for RLF Model Training

For training of AI/ML models for RLF prediction (indirect), we need to collect data which can be used for supervised learning. This implies the UE trajectory should face situations where the SINR dips below the Qout threshold of -8dB. Also, since the UE will ideally move across cells, it is important to consider what happens when the UE encounters a true RLF condition during data collection. There are two options of how to model this in simulations

- Option-1: When a UE faces RLF conditions along a trajectory, the UE trajectory is terminated for data collections purposes

- Option-2: When a UE faces RLF conditions along a trajectory, a handover is modeled (potentially without interruption time) to continue data collection till the end of the trajectory

Option-1 is simpler but may suffer from insufficient data to train the model since there would never be the realistic scenario of a successful handover after RLF. Option-2 therefore would offer a more realistic data collection setting for RLF prediction

- For data collection for RLF prediction, consider modeling a pseudo-handover along UE trajectory based on A3 event criterion such that data collection can continue in a realistic setting along the trajectory.

For interference modelling, wrap-around should be used but the UE trajectory can still be terminated or bounded based on the bounding circle technique shown in the figure below. Note that pico-cell modelling is not a requirement in this study.

Figure 3: UE trajectory bounding (TR 36.839 Figure 5.4.5.1.2)

- Interference calculation can be based on wrap-around model, but UE trajectory can be bounded using a bounding circle as shown in TR 36.839.

# Conclusion

The main proposals from this paper are outlined here:

- Study RLF failure prediction in State 1 (before event A3 entering condition is satisfied) and consider both cell-specific and multi-cell measurements to determine true RLF vs Handover failure.

- For Indirect RLF predictions re-use evaluation assumptions from RRM measurement prediction as a starting point.

- Based on calibrations from IMT-2020, Qout threshold of -8dB can be used as a starting point for FR2 evaluations

- Consider T310 values which are smaller multiples of L1 measurement periodicity e.g., 200ms as a starting point.

- For data collection for RLF prediction, consider modeling a pseudo-handover along UE trajectory based on A3 event criterion such that data collection can continue in a realistic setting along the trajectory.

- Interference calculation should be based on wrap-around model, but UE trajectory can be bounded using a bounding circle as shown in TR 36.839.

# References

- RP-240082, Revised SID on AI/ML for Mobility in NR, OPPO (Rapporteur), 3GPP TSG RAN Meeting#103, Maastricht, Netherlands, March 2024.

- Chair Notes, 3GPP RAN2 Meeting#126, Fukuoka City, Japan, May 2024.

- Chair Notes, 3GPP RAN2 Meeting#125bis, Changsha, China, April 2024.

- TR 37.910, Study on self-evaluation towards IMT-2020 submission (Release 16).