3GPP TSG-RAN WG2 Meeting #127	R2-2407290

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.2.2 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on functionality aspects of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses functionality of ambient IoT (A-IOT). In this contribution, we discuss mainly security related aspects, need of resource allocation/scheduling, and message size/status indication. The related agreements in R2-126 meeting are as follows.

Agreements ... Agreements from stage 2 general aspects in R2-126 meeting 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command) … Agreements from A-IoT paging in R2-126 meeting 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. … Agreements from functionality aspects in R2-126 meeting 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed 4 AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) …

2.	Discussion

Security related aspects

Regarding security related aspects and SA2/SA3 dependent discussion, the related discussion in SA2/SA3 is still in-progress. It seems early to proceed with the related RAN2 discussion. Particularly, RAN2 agreed to study “command only” used in the previous meeting (R2-126). Options on how to support “command only” case is FFS. One of the options is that initial trigger message from the reader contains the command, and its feasibility depends on SA2/SA3 work and conclusions. The other option is to use baseline procedure for “inventory and command. Considering SA2/SA3 progress, the option of initial trigger message containing the command can be discussed after the option seems feasible by SA2/SA3 progress. The option to exploit “inventory and command” use case can be focused for “command only” use case.

Proposal 1. Postpone the discussion about initial trigger message containing the command until it seems feasible from SA2/SA3 progress.

Need of resource allocation/scheduling

Regarding resource allocation for A-IoT random access, since RAN2 confirms slotted-ALOHA is the baseline for A-IoT random access in a previous meeting (R2-125bis), radio resources for a set of slots (or access occasions) needs to provided to the devices which are triggered for random access. Then, each device selects a slot randomly in the set of slots for its access. The details are closely tied with A-IoT paging procedure and A-IoT random access procedure. So, it is further discussed in our companion papers submitted to agenda item 8.2.3 (A-IoT paging) and agenda item 8.2.4 (A-IoT random access).

Another aspect of resource allocation is to determine TB size at an access occasion. When a device gets an access occasion, how many bits are allocated for the D2R transmission needs to be provided to the device. For comparison, in legacy NR, when an uplink grant is scheduled for a UE using a downlink control information (DCI), the uplink grant size is also informed by the DCI. In A-IoT, it’s still under discussion in RAN1 whether R2D control information is included in PHY frame structure for R2D transmission or not and what information is conveyed in R2D control information. Particularly, it is also under discussion in RAN1 whether TB size of D2R transmission can be indicated to the device using R2D control information or not. Even if R2D control information is designed to contain TB size of the corresponding D2R transmission, the reader is not always able to use it to inform TB size to the device because the reader is not always expected to be aware of the required TB size for D2R transmission. With the case, a device may be provided with its access occasion but TB size for the access occasion is not provided. To guarantee A-IoT communication even for the case, the device should be allowed to send its data using D2R transmission within the maximum TB size at least when the TB size cannot be determined by reader and it is not provided to the device at the access occasion. The device is able to determine TB size of D2R transmission according to its data within the maximum TB size. The maximum TB size can be pre-configured or provided using R2D control information. If D2R control information containing TB size of D2R transmission is introduced or PHY frame structure for D2R transmission includes a postamble for indicating end of the frame, the device can determine TB size according to its data. Otherwise, the device can determine TB size as same as the maximum TB size and padding can be placed in the unused part.

Based on the above-mentioned analysis, it should be considered that a device is provided with an access occasion without TB size information in A-IoT communication because a reader is not always expected to be aware of the required TB size of D2R transmission. In the case, for D2R transmission, a device needs to decide TB size and performs D2R transmission within the maximum TB size at its access occasion.

Proposal 2. A device decides TB size and performs D2R transmission within the maximum TB size at its access occasion when a device is provided with the access occasion without TB size information.

A-IoT resource allocation/scheduling may consider A-IoT device’s remaining energy level because energy can be considered as a limited resource for an A-IoT device. When a device is provided with an access occasion, the device cannot use the access occasion if it doesn’t have enough energy.

If remaining energy of A-IoT device is not considered in resource allocation/scheduling, it is not guaranteed that the allocated resource is successful used for D2R transmission. To address this issue, one possible way is to provide enough time for A-IoT device’s energy harvesting. For example, before triggering devices, reader energizes devices during sufficient time. However, it is difficult to determine proper duration of energizing A-IoT devices because reader needs to take into account the A-IoT devices in the various conditions such as a device in the poor condition for energy harvesting, other device requiring large amount of energy (e.g. largest data), and etc. Furthermore, it does not seem easy to obtain such information about A-IoT devices. Therefore, reader may apply enough margin for safe operation. It will decrease resource efficiency of A-IoT system. Otherwise, possibility of communication failures will increase. To address this issue, remaining energy level of a device needs to be considered for resource allocation/scheduling.

For an example, a A-IoT device may respond with remaining energy indication for the device specific A-IoT paging as shown in Figure 1. Based on the remaining energy indication, the reader can determine duration of energizing A-IoT devices and take into account the duration for resource allocation/scheduling. Then, the device performs energy harvesting during the duration. After that, the reader and the device perform data communication.

Figure 1 Example for using remaining energy level indication

Proposal 3. Study how to take into account A-IoT device’s remaining energy in access mechanism and resource allocation/scheduling.

Message size/status indication

Regarding a message size/status indication, it is agreed in the previous meeting (R2-126) that legacy NR BSR/SR is not needed for A-IoT communication. Still, it is not decided whether further indication of device message size/status is needed or not. We would like to discuss need of a D2R message size/status indication.

A D2R message is generated in process of inventory or command procedure, and it is generally triggered by R2D message. In other words, the corresponding D2R message is generated as the response of an R2D message. Whether there is a D2R message or not is already known to the reader when the reader sent the R2D message to the device.

For the size of the message, it is expected that a D2R message is a fixed size at each step in a given procedure. For a simple example, Msg2 of 4-step CBRA is expected to be 16bits payload plus fixed size header if RN16 is used. For other example, when a command is sent, the corresponding response may be sent using a fixed format and size. Then, the size of D2R message is known to the reader without explicit size information from the device. Generally, variable size messages need to be handled. For variable size messages, the upper bound of the message sizes can be used. If information on D2R message size is not available at all, the maximum TB size can be used. The maximum TB size is still under discussion in RAN. Considering SA discussion about maximum A-IoT data size (say, 1000bits) and PHY transmission size limitation expected by relatively loose sampling frequency offset (SFO) requirement (say, 10^5 PPM), it seems to make sense to assume the maximum TB size for D2R message transmissions. So, a message size/status indication does not seem to be essentially needed.

To enhance efficiency of resource allocation for a variable size message, message size indication can be considered. However, the indication should be sent to the reader before D2R transmission. It requires additional steps between R2D message and the corresponding D2R message, and makes the baseline procedures for inventory and command further complicated.

Based on the above-mentioned analysis, we conclude that D2R message size/status indication is not needed in A-IoT communication.

Proposal 4. D2R message size/status indication is not needed.

3.	Conclusion

This document discusses functionality of ambient IoT (A-IOT).

Proposal 1. Postpone the discussion about initial trigger message containing the command until it seems feasible from SA2/SA3 progress.

Proposal 2. A device decides TB size and performs D2R transmission within the maximum TB size at its access occasion when a device is provided with the access occasion without TB size information.

Proposal 3. Study how to take into account A-IoT device’s remaining energy in access mechanism and resource allocation/scheduling.

Proposal 4. D2R message size/status indication is not needed.