3GPP TSG-RAN WG2 Meeting #127							R2-2407294

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Relay discovery and (re)selection for multi-hop Relay

Source: 	Huawei, HiSilicon

Agenda item:	8.13.2

Document for:	Discussion/Decision

# Introduction

At RAN #104 meeting, the new work item on NR sidelink multi-hop relay [1] was approved and its objectives are listed as below. In this paper, we will discuss the relay discovery and relay (re)selection aspects.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] 1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3] 2. Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: A. Intra-gNB multi-hop indirect to direct path switching using existing framework B. Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: C. Intra-gNB direct to multi-hop indirect path switching D. Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

# Discussion

Rel-19 Multi-hop relay can further extend the coverage by supporting the multiple hop sidelink relay on top of Rel-17 U2N relay. Figure 1 shows the scenario where the remote UE connects with the network through two relay UEs, i.e., remote UE -> first relay UE -> last relay UE -> gNB.

Figure 1. Multi-hop relay with two relaying hops

We will reuse the terminology “U2N Relay UE” to name the relay UE connected with the gNB directly, and use the terminology “Intermediate Relay UE” to name the relay UEs between the U2N Relay UE and the U2N Remote UE in the discussion for forward compatibility.

## General design of relay discovery

For Rel-17 U2N relay and Rel-18 U2U relay, the relay discovery procedures have been thoroughly discussed. In our understanding most, features can be inherited and applied in the multi-hop relay scenario, e.g., the protocol stack of discovery message, the resource pool configuration, SL-SRB4 for discover message, RRC states condition aspects. Hence, we have  the following proposal.

Proposal 1:  The following design principles for relay discovery can be inherited and applied in the multi-hop relay scenario.

- Model A and Model B discovery models are supported for the Relay discovery in multi-hop relay.

- The protocol stack of discovery message for R17/R18 can be reused. SL-SRB4 is used for discovery message and no ciphering or integrity protection is needed in PDCP layer.

- U2N Remote UE/U2N Relay UE/Intermediate Relay UE can perform Relay discovery message transmission and monitoring while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED.

- Dedicated resource pool and shared resource pool can be configured for Relay discovery, and the configuration can be carried  in SIB12 or by  dedicated RRC signaling or pre-configuration

- Both Resource allocation mode 1 and mode 2 can be support for Relay discovery.

As we discussed in another paper [2], the multi-hop Layer-2 UE-to-Network relay can be supported by the following two architectures, i.e., “U2N relay + remote UE” architecture and “U2U relay + U2N relay” architecture, as shown in Figure 2.

(a) Architecture 1: “U2N relay + remote UE” architecture

(b) Architecture 2: “U2U relay + U2N relay” architecture

Figure 2. General architectures for Multi-hop relay

In above two architectures, the relay discovery procedures for the U2N Remote UE can be different. In architecture 1, the U2N Remote UE performs the direct discovery and finds a proper Intermediate Relay UE to establish the PC5 connection. By Direct discovery we mean that discovery procedure is performed between Remote UE and the intermediate Relay UE and Both Model A and Model B discovery procedure can be used. In architecture 2, the U2N Remote UE performs the U2U relay discovery to find a proper U2N Relay UE and needs to establish the E2E PC5 connection with the U2N Relay UE, which can be more complex. Besides, as we illustrated in [2], architecture 2 also can need more complicated mechanisms on the RRC connection management and QoS handling. Therefore, we propose in [2] to use the  “U2N relay + remote UE” architecture to support the multi-hop Layer-2 U2N relay in Release19.

Proposal 2: Use the “U2N relay + remote UE” architecture to support the multi-hop Layer-2 U2N relay in Release 19.

- The U2N Remote UE finds a proper Intermediate Relay UE based on the direct discovery procedure (i.e. discovery procedure is performed between Remote UE and the intermediate Relay UE and Both Model A and Model B discovery procedure can be used).

## Conditions for performing Relay discovery

Based on proposal 2, we will discuss the conditions for the Remote UE and Intermediate Relay UE to perform the Relay discovery. For the Remote UE, it should be able to perform the relay discovery if the signaling quality of its serving cell is not good, or it cannot find a proper serving cell on Uu, Hence similar to legacy discovery procedure in Rel-17, we have the following proposal.

Proposal 3: The condition for the Remote UE to perform the discovery for multi-hop Relay can be  one of the following, i.e., if the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could  not find  a serving cell, the Remote UE can perform discovery procedure.

For the Intermediate Relay UE, we can consider the following aspects as  the condition of performing the discovery procedure.

- The PC5 link quality of the Intermediate Relay UE with the U2N Relay UE or the other Intermediate Relay UE

- The hop number between the Intermediate Relay UE and the gNB

- The number of the UE(s) served by the U2N Relay UE of the Intermediate Relay UE, the U2N Relay UE of the Intermediate Relay UE means the U2N Relay UE providing the relaying service for this Intermediate Relay UE.

Technically, we should guarantee that the indirect link between the Intermediate Relay UE and the network should be good if providing the relay service for the Remote UE. The Intermediate Relay UE only needs make sure that the PC5 link between the Intermediate Relay UE and the U2N Relay UE (for 2 hop relaying scenario) or between the Intermediate Relay UE and another Intermediate Relay UE (for >2 hop relaying scenario) is good, since the U2N Relay UE and the other Intermediate Relay UE should  guarantee the link quality of the Uu link and the other PC5 link between them. Therefore, the first condition for the Intermediate Relay UE is the PC5 link quality of the Intermediate Relay UE with the U2N Relay UE or the other Intermediate Relay UE is good, e.g., above a threshold.

The second and third aspects are necessary to ensure the quality of relaying service. If the hop number between the Remote UE and the network is too large, the quality of service, e.g., the latency performance, cannot be guaranteed. Even though the gNB can provide proper relaying configuration like the Uu Relay RLC channel configuration to guarantee the performance on Uu link, it cannot control the overall performance well since the resource allocation mode 2 used on PC5 link is out of gNB’s control. In addition, the number of the UE(s) served by the U2N Relay UE would also impact the QoS of the UE(s).  Since all the relaying traffic for the connected Remote UE(s) and Intermediate Relay UE(s) is relayed by the U2N Relay UE, congestion can happen if the number of the UE(s) served by the U2N Relay UE is too large. Therefore, RAN2 needs to discussion the solutions to limit the hop number and/or the number of UE(s) connected with the U2N Relay UE. We think the Intermediate Relay UE should stop performing discovery, i.e., stop providing relaying service for the Remote UE(s), if the hop number between the Intermediate Relay UE and the gNB exceeds a threshold or the number of UEs served by the U2N Relay UE of the Intermediate Relay UE exceeds a threshold.

Observation 1: If the hop number between the intermediate  Relay UE and the network is too large or the number of the UE(s) served by the U2N Relay UE is too large, the latency and QoS requirement of the Remote UE(s) cannot  be guaranteed. Thus, the hop number between the Remote UE and the network the number of UE(s) connected with the U2N Relay UE should be limited.

Proposal 4: Consider the following conditions for the Intermediate Relay UE to perform the discovery.

- The PC5 link quality of the Intermediate Relay UE with U2N Relay or another Intermediate Relay UE (for > 2 hops) is above a threshold

- The hop number between the Intermediate Relay UE and the gNB is below a threshold

- The number of the UE served by the U2N Relay UE of the Intermediate Relay UE is below a threshold.

## Relay selection and reselection

For Relay selection and reselection procedures, the legacy PC5 radio measurements mechanism can be reused, including:

- The U2N Remote UE uses the SD-RSRP measurements to evaluate the link quality if there is no PC5 connection between the U2N Remote UE and the candidate Intermediate Relay UE.

- The U2N Remote UE uses the SL-RSRP measurements to evaluate the link quality between the Remote UE and the serving Intermediate Relay UE. Serving Intermediate Relay UE is the Intermediate Relay UE that is providing the relaying service for the U2N Remote UE.

- It is left to Remote UE implementation whether to use SL-RSRP or SD-RSRP or both for measurement  if there is no data transmission from serving Intermediate Relay UE to U2N Remote UE.

Proposal 5: Release-17 PC5 link measurement and evaluation principles for relay selection and  reselection can be reused for multi-hop relay selection and reselection.

Similar to Release 17, the U2N Remote UE can trigger U2N relay selection in following cases .

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE

Hence, we propose the following

Proposal 6: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE  and the remote UE  is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE. The serving Intermediate Relay UE sends the notification message when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens.

- Upon receiving the notification message from the serving Intermediate Relay UE, which happens when the serving Intermediate Relay UE receives the notification message indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 7: The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 8: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens, or upon receiving the notification message from the U2N Relay that indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

There can be several candidate  U2N Relay UE(s) or candidate Intermediate Relay UE(s) around the U2N Remote UE when performing relay selection and reselection. In the current spec, the only information the Remote UE has about  the candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, we think this information may  not be enough to select a good  relay UE. Considering the case that the Remote UE can discover the following candidate relay UE(s) in the following.

- relay UE 1: the SD-RSRP is value 1, and the relay UE 1 is connected to the gNB with a U2N Relay UE. Value 1 satisfies the relay selection criterion, i.e., above the configured threshold.

- relay UE 2:  the SD-RSRP is value 2, and the relay UE 2 is connected to the gNB with a U2N Relay UE and an Intermediate Relay UE. Value 2 satisfies the relay selection criterion, and Value 2 is larger than value 1.

Based on the PC5 link quality, the Remote UE will select the relay UE 2 with a high probability, since the PC5 link quality between the Remote UE and the relay UE is better. However, from the QoS perspective, the relay UE 1 can be better since the hop number between the relay UE 1 and the network is smaller. Therefore, we think it is better to indicate the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure, which can be used to select a better relay UE.

Proposal 9: RAN2 can discuss how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

# Conclusion

In this paper, we provide the following observations and proposals.

General design of relay discovery

Proposal 1:  The following design principles for relay discovery can be inherited and applied in the multi-hop relay scenario.

- Model A and Model B discovery models are supported for the Relay discovery in multi-hop relay.

- The protocol stack of discovery message for R17/R18 can be reused. SL-SRB4 is used for discovery message and no ciphering or integrity protection is needed in PDCP layer.

- U2N Remote UE/U2N Relay UE/Intermediate Relay UE can perform Relay discovery message transmission and monitoring while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED.

- Dedicated resource pool and shared resource pool can be configured for Relay discovery, and the configuration can be carried  in SIB12 or by  dedicated RRC signaling or pre-configuration

- Both Resource allocation mode 1 and mode 2 can be support for Relay discovery.

Proposal 2: Use the “U2N relay + remote UE” architecture to support the multi-hop Layer-2 U2N relay in Release 19.

- The U2N Remote UE finds a proper Intermediate Relay UE based on the direct discovery procedure (i.e. discovery procedure is performed between Remote UE and the intermediate Relay UE and Both Model A and Model B discovery procedure can be used).

Conditions for performing Relay discovery

Observation 1: If the hop number between the intermediate  Relay UE and the network is too large or the number of the UE(s) served by the U2N Relay UE is too large, the latency and QoS requirement of the Remote UE(s) cannot  be guaranteed. Thus, the hop number between the Remote UE and the network the number of UE(s) connected with the U2N Relay UE should be limited.

Proposal 3: The condition for the Remote UE to perform the discovery for multi-hop Relay can be  one of the following, i.e., if the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could  not find  a serving cell,  the Remote UE can perform discovery procedure.

Proposal 4: Consider the following conditions for the Intermediate Relay UE to perform the discovery.

- The PC5 link quality of the Intermediate Relay UE with U2N Relay or another Intermediate Relay UE (for > 2 hops) is above a threshold

- The hop number between the Intermediate Relay UE and the gNB is below a threshold

- The number of the UE served by the U2N Relay UE of the Intermediate Relay UE is below a threshold.

Relay selection and reselection

Proposal 5: Release-17 PC5 link measurement and evaluation principles for relay selection and  reselection can be reused for multi-hop relay selection and reselection

Proposal 6: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE camps is below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 8: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens, or upon receiving the notification message from the U2N Relay that indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

Proposal 9: RAN2 can discuss how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

# References

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

R2-2407295, Control plane procedures for multi-hop relay, Huawei, HiSilicon.