3GPP TSG-RAN WG2 Meeting #127							R2-2407295

Maastricht, Netherlands, 19 - 23 August, 2024

Title: 	Control plane procedures for multi-hop relay

Source: 	Huawei, HiSilicon

Agenda item:	8.13.3

Document for:	Discussion/Decision

# Introduction

At RAN #104 meeting, the new work item on NR sidelink multi-hop relay [1] was approved and its objectives are listed as below. In this paper we will discuss the control plane procedures to support the multi-hop relay.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] 1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3] 2. Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: A. Intra-gNB multi-hop indirect to direct path switching using existing framework B. Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: C. Intra-gNB direct to multi-hop indirect path switching D. Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

# Discussion

In Rel-17 SL relay and Rel-18 SL relay enhancements, a remote UE can be connected to the network through a relay UE, thus supporting the coverage extension of the network. Rel-19 Multi-hop relay can further extend the coverage by supporting the multiple hop sidelink relay on top of Rel-17 U2N relay. Figure 1 shows the scenario where the remote UE connects with the network through two relay UEs, i.e., remote UE -> first relay UE -> last relay UE -> gNB.

Figure 1. Multi-hop relay with two relaying hops

In the following sections, we will discuss how to support the baseline control plan procedures for the multi-hop relay, including the general architecture, protocol stack, RRC connection management, and so on. As indicated by the WID scope, only the multi-hop Layer-2 UE-to-Network relay will be considered.

## Terminology

Our discussion will be mainly based on the scenario shown in Figure 1, but also considers the forward compatibility to support more than two hop relays. For the sake of forward compatibility, in the following discussion, we reuse the terminology “U2N Relay UE” to name the relay UE connected with the gNB directly, and use the terminology “Intermediate Relay UE” to name the relay UEs between the U2N Relay UE and the U2N Remote UE.

Proposal 1: To be forward compatible, RAN2 can use the terminology “U2N Relay UE” to name the relay UE connected with the gNB directly, and use the terminology “Intermediate Relay UE” to name the relay UEs between the U2N Relay UE and the U2N Remote UE.

## General architecture

Based on Rel-17 and Rel-18 protocols, the multi-hop Layer-2 UE-to-Network relay can be support by the following two architectures, as shown in Figure 2.

(a) Architecture 1 1: “U2N relay + remote UE” architecture

(b) Architecture 2: “U2U relay + U2N relay” architecture

Figure 2. General architectures for Multi-hop relay

Architecture 1 is the “U2N relay + remote UE” architecture, where the Intermediate Relay UE connected to the gNB via the U2N relay UE based on the Rel-17 U2N relay mechanisms/procedures, and the Intermediate Relay UE can further provide the relaying service for the U2N Remote UE. In this architecture, the U2N Remote UE connects with the Intermediate Relay UE via the PC5 link but there is no E2E PC5 connection with the U2N Relay UE.

Architecture 2 is the “U2U relay + U2N relay” architecture, in which the Remote UE performs the discovery procedure and establishes the E2E PC5 connection with the U2N Relay UE and the Intermediate Relay UE acts as an U2U relay UE. In this architecture, the Intermediate Relay UE does not need to be connected to the gNB via the U2N Relay UE. The Intermediate Relay UE can be out of coverage or in any RRC state regardless of the RRC state of the U2N remote UE when providing the relaying service.

The above two architectures can both support the multi-hop relay and they are both forward compatible. However, Architecture 2 can be more complicated, since the Intermediate Relay UE and the U2N relay can be under the control of different gNBs or the Intermediate Relay UE can be out of the network control, which will introduce challenges on the RRC connection management and the QoS handling procedures. Besides, RAN2 need to further discuss the forward compatibility of the U2U relay for Architecture 2, which is going to be time consuming according to the Rel-18 discussion. Therefore, we propose to use the “U2N relay + remote UE” architecture as shown in Figure 2 to support the multi-hop Layer-2 U2N relay.

Proposal 2: Use the “U2N relay + remote UE” architecture to support the multi-hop Layer-2 U2N relay in Rel 19. There is PC5 connection between the U2N Relay UE and the Intermediate Relay UE, and between the Intermediate Relay UE and the Remote UE, but there is no E2E PC5 connection between the U2N Relay UE and the Remote UE.

Based on the “U2N relay + remote UE” architecture, we can have the following principle similar to the Rel-17 U2N relay.

- U2N Relay UE, Intermediate Relay UE, and U2N Remote UE should all be in RRC_CONNECTED State to perform the transmission/reception of relaying data.

- The Intermediate Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all the U2N Remote UE(s) that connected to this Intermediate Relay UE are in RRC_INACTIVE or in RRC_IDLE.

- The U2N Relay UE can be RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all the connected Intermediate Relay UE(s) and the connected U2N Remote UE(s) are in RRC_INACTIVE or in RRC_IDLE.

Proposal 3: The following RRC state combinations should be supported for multi-hop relay:

- U2N Relay UE, Intermediate Relay UE, and U2N Remote UE can  all be in RRC_CONNECTED to perform the transmission/reception of relaying data.

- The Intermediate Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all connected  U2N Remote UE(s) are in RRC_INACTIVE or in RRC_IDLE.

- The U2N Relay UE can be RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all connected Intermediate Relay UE(s) and connected U2N Remote UE(s) are in RRC_INACTIVE or in RRC_IDLE.

Since there is no direct Uu link for the Intermediate Relay UE and the Remote UE in Rel-19, only resource allocation mode 2 can be supported.

Proposal 4:  Only resource allocation mode 2 is supported for Intermediate Relay UE and the Remote UE in Rel-19 .

## Protocol stack for Layer-2 multi-hop relay

Following the legacy design for Layer-2 relay, the Control Plane and the User Plane protocol stacks of the “U2N relay + remote UE” architecture for multi-hop relay with one intermediate Relay UE can be extended as shown in Figure 3. SRAP layer is reused for bearer mapping and remote UE identification purpose. These protocol stack can be taken as the basis for the R19 Mult hop relay including two or more hops.

(a) Control plan

(b) User plan

Figure 3. Protocol stack for multi-hop L2 U2N relay

Proposal 5: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3.  SRAP layer should be  reused for bearer mapping and remote UE identification purpose.

## RRC Connection establishment

Figure 4 shows the RRC Connection establishment procedure to support the multi-hop L2 U2N relay.

Figure 4.	RRC Connection establishment for multi-hop L2 U2N relay

- The U2N Remote UE and Intermediate Relay UE perform discovery procedure, the remote UE selects the proper relay UE and triggers the RRC Connection establishment procedure. The details for relay discovery can be found in another discussion paper [2].

- The U2N Remote UE selects the Intermediate Relay UE and establish the PC5 connection. Whether the Remote UE knows the relay UE is an Intermediate Relay UE or an U2N Relay UE is  discussed in another  discussion paper [2].

- The Remote UE sends the RRCSetupRequest message to the Intermediate Relay UE. Similar to the Rel-17, the message can be delivered via the specified PC5 relay RLC channel between the U2N Remote UE and the Intermediate Relay UE. The parameters specified for Rel-17 NR sidelink L2 U2N Relay operations can be reused.

- The Intermediate Relay UE sends the SUI message to the gNB, which contains the L2 ID of the Remote UE.

- gNB configures the PC5 relay RLC channel and Uu relay RLC channel for the relaying of the SRB0 message to the Intermediate Relay UE and the U2N Relay UE, including:

- the PC5 relay RLC channel of the Intermediate Relay UE between the Intermediate Relay UE and the U2N Relay UE

- the PC5 relay RLC channel of the U2N Relay UE between the Intermediate Relay UE and the U2N Relay UE

- the Uu relay RLC channel of the U2N Relay UE between the U2N Relay UE and the gNB

Besides, gNB configures the mapping between SRB0 and the above Uu/PC5 relay RLC channels.

- Based on the relaying configuration, the Intermediate Relay UE and the U2N relay UE delivers the RRCSetupRequest message of Remote UE to the gNB.

- gNB sends the RRCSetup message to the U2N Remote UE with the relaying of the Intermediate Relay UE and the U2N Relay UE, which contains the SRB1 configuration (e.g., PC5 Relay RLC channel configuration for SRB1 message) to the Remote UE.

- The gNB configures the PC5 relay RLC channel and Uu relay RLC channel for the relaying of the SRB1 message to the Intermediate Relay UE and the U2N Relay UE, including:

- the PC5 relay RLC channel of the Intermediate Relay UE between the U2N Remote UE and the U2N Relay UE

- the PC5 relay RLC channel of the Intermediate Relay UE between the Intermediate Relay UE and the U2N Relay UE

- the PC5 relay RLC channel of the U2N Relay UE between the Intermediate Relay UE and the U2N Relay UE

- the Uu relay RLC channel of the U2N Relay UE between the U2N Relay UE and the gNB

Besides, gNB configures the mapping between SRB1 and the above Uu/PC5 relay RLC channels.

- The U2N Remote UE sends the RRCSetupComplete message to the gNB via  the  Intermediate Relay UE and the U2N Relay UE.

- gNB provides the relaying configuration to the Intermediate Relay UE and the U2N Relay UE to support the relaying of the SRB2 message and DRB data of the Remote UE.

- gNB sends the RRCReconfiguration message to the Remote UE to provide the SRB2 and DRB configuration.

Proposal 6: In multi-hop relay, remote UE sends the SRB0 message via the specified PC5 relay RLC channel. The parameters specified for Rel-17 SL U2N relay can be reused.

Proposal 7: The gNB provides the relaying configuration to the Intermediate Relay UE and the U2N Relay UE for relaying Remote UE’s SRB1/SRB2/DRB data, which includes the mapping configuration and the configuration for

- the PC5 relay RLC channel of the Intermediate Relay UE between the U2N Remote UE and the U2N Relay UE,

- the PC5 relay RLC channel of the Intermediate Relay UE between the Intermediate Relay UE and the U2N Relay UE,

- the PC5 relay RLC channel of the U2N Relay UE between the Intermediate Relay UE and the U2N Relay UE, and

- the Uu relay RLC channel of the U2N Relay UE between the U2N Relay UE and the gNB.

Proposal 8: support  the procedure in Figure 4 as baseline to support RRC Connection establishment for multi-hop L2 U2N relay.

In the above procedure, the Intermediate Relay UE and the U2N Relay UE are assumed to be in RRC_CONNECTED state. We can further discuss whether and how to support the RRC_IDLE/INACITVE relay UEs, particularly when the Intermediate Relay UE is in RRC_IDLE/INACITVE state.

Proposal 9: Based on the baseline procedure, RAN2 can further discuss whether and how to support RRC_IDLE/INACITVE Intermediate Relay UE.

## Radio link failure handling

Following the Rel-17 principle, the U2N Remote UE will suspend Uu radio link management operation, since the remote UE and the Intermediate relay UE connects to network via indirect link and there is no Uu link anymore.  The PC5/Uu RLF may happen for the multi hop scenario in the following cases as depicted in Figure 5.

(a) PC5 RLF of the U2N Remote UE

(b) PC5 RLF of the Intermediate Relay UE

(c) Uu RLF of the U2N Relay UE

Figure 5.	RLF declaration cases in multi-hop L2 U2N relay

In case (a) of Figure 5, when the U2N Remote UE detects the PC5 RLF over the PC5 link between the Remote UE and the Intermediate Relay UE, it should trigger the RRC connection re-establishment, which is similar to the Rel-17 handling.

Proposal 10: Similar to Rel-17 SL relay, when the U2N Remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

In case (b) of Figure 5, when the Intermediate Relay UE detects the PC5 RLF over the PC5 link between the Intermediate Relay UE and the U2N Relay UE, it can send a notification message with cause value “PC5 RLF” to the U2N Remote UE and trigger the RRC connection re-establishment. Upon receiving the notification message, the Remote UE can determine whether to release the PC5 link with the Intermediate Relay UE and triggers the RRC connection re-establishment. If the Remote UE releases the PC5 link, the Remote UE can perform the relay reselection and the cell selection, and trigger the RRC connection re-establishment via the new relay or via the Uu link.

Proposal 11: When the Intermediate Relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the U2N Remote UE and trigger the RRC connection re-establishment.

Proposal 12: Upon receiving the notification message with cause value “PC5 RLF”, the Remote UE can determine whether to release the PC5 link with the Intermediate Relay UE and triggers the RRC connection re-establishment.

In case (c) of Figure 5, when the U2N Relay UE detects the Uu RLF, it can send a notification message with cause value “Uu RLF” to the Intermediate Relay UE and trigger the RRC connection re-establishment. Upon receiving the notification message with cause value “Uu RLF”, the Intermediate Relay UE triggers RRC connection re-establishment as legacy in Rel-17. Furthermore, the Intermediate Relay UE needs to notify the U2N Remote UE, but what cause value should be contained in the notification message should be discussed. The following options can be considered.

- Option 1: introduce a new cause value which indicates Uu RLF of U2N Relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link of Intermediate Relay UE

- Option 3: reuse the “PC5 RLF” cause value

The Remote UE behavior when receiving the notification message from the Intermediate Relay UE can be similar as before.

Proposal 13: Upon receiving the notification message with cause value “Uu RLF” from the U2N Relay UE, the Intermediate Relay UE needs to send a notification message to the Remote UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu RLF of U2N Relay UE

- Option 2: introduce a new cause value which indicates RLF happens on the indirect link of Intermediate Relay UE

- Option 3: reuse the “PC5 RLF” cause value

## System information

In ReL-17, the RRC_CONNECTED Remote UE can use the on-demand SIB framework to request the SIB(s). The RRC_IDLE or RRC_INACTIVE Remote UE can indicate the requested SIB type(s) to the Relay UE, then Relay UE can trigger the on-demand SIB acquisition procedure on the Uu and forward the acquired SIB(s) to the Remote UE via PC5.

In Rel-19 multi-hop relay, the Rel-17 on-demand SIB framework can be reused but with some enhancements for the RRC_IDLE or RRC_INACTIVE Remote UE cases. First, the RRC_IDLE or RRC_INACTIVE Remote UE indicates the required SIB(s) to the Intermediate Relay UE. If the Intermediate Relay UE is also in RRC_IDLE or RRC_INACTIVE and has no required SIB(s), the Relay UE needs to send the request to the U2N Relay UE, e.g., via the PC5 message. It means that the PC5 message form the Intermediate Relay UE to the U2N Relay UE should indicate all the required SIB(s) requested from all the connected Remote UE(s) and the Intermediate Relay UE itself.

Proposal 14: The PC5 message form the Intermediate Relay UE to the U2N Relay UE should indicate all the required SIB(s) requested from all the connected Remote UE(s) and the Intermediate Relay UE itself.

## Paging delivery

In ReL-17, the RRC_IDLE or RRC_INACTIVE Remote UE provides the Remote UE ID (5G-S-TMSI or I-RNTI) to the relay UE, then the U2N relay UE will forwards the paging to the Remote UE if the paging message contains the Remote UE ID.

In Rel-19 multi-hop relay, the U2N Relay UE need to help forward the paging for the Remote UE and the Intermediate Relay UE if they are both in  RRC_IDLE or RRC_INACTIVE state. Therefore, in addition to the 5G-S-TMSI or I-RNTI of the Intermediate Relay UE, the Intermediate Relay UE needs to forward the Remote UE IDs to the U2N relay UE.

Proposal 15: The Intermediate Relay UE needs to forward the Remote UE IDs to the U2N relay UE for paging delivery to the Remote UE.

# Conclusion

In this paper, we have the following proposals

Terminology

Proposal 1: To be forward compatible, RAN2 can use the terminology “U2N Relay UE” to name the relay UE connected with the gNB directly, and use the terminology “Intermediate Relay UE” to name the relay UEs between the U2N Relay UE and the U2N Remote UE.

General architecture

Proposal 2: Use the “U2N relay + remote UE” architecture to support the multi-hop Layer-2 U2N relay in Rel 19. There is PC5 connection between the U2N Relay UE and the Intermediate Relay UE, and between the Intermediate Relay UE and the Remote UE, but there is no E2E PC5 connection between the U2N Relay UE and the Remote UE.

Proposal 3: The following RRC state combinations should be supported for multi-hop relay:

- U2N Relay UE, Intermediate Relay UE, and U2N Remote UE can  all be in RRC_CONNECTED to perform the transmission/reception of relaying data.

- The Intermediate Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all connected  U2N Remote UE(s) are in RRC_INACTIVE or in RRC_IDLE.

- The U2N Relay UE can be RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED if all connected Intermediate Relay UE(s) and connected U2N Remote UE(s) are in RRC_INACTIVE or in RRC_IDLE.

Proposal 4:  Only resource allocation mode 2 is supported for Intermediate Relay UE and the Remote UE in Rel-19

Protocol stack for Layer-2 multi-hop relay

Proposal 5: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3.  SRAP layer should be  reused for bearer mapping and remote UE identification purpose.

RRC Connection establishment

Proposal 6: In multi-hop relay, remote UE sends the SRB0 message via the specified PC5 relay RLC channel. The parameters specified for Rel-17 SL U2N relay can be reused.

Proposal 7: The gNB provides the relaying configuration to the Intermediate Relay UE and the U2N Relay UE for relaying Remote UE’s SRB1/SRB2/DRB data, which includes the mapping configuration and the configuration for

- the PC5 relay RLC channel of the Intermediate Relay UE between the U2N Remote UE and the U2N Relay UE,

- the PC5 relay RLC channel of the Intermediate Relay UE between the Intermediate Relay UE and the U2N Relay UE,

- the PC5 relay RLC channel of the U2N Relay UE between the Intermediate Relay UE and the U2N Relay UE, and

- the Uu relay RLC channel of the U2N Relay UE between the U2N Relay UE and the gNB.

Proposal 8: support  the procedure in Figure 4 as baseline to support RRC Connection establishment for multi-hop L2 U2N relay.

Proposal 9: Based on the baseline procedure, RAN2 can further discuss whether and how to support RRC_IDLE/INACITVE Intermediate Relay UE.

Radio link failure handling

Proposal 10: Similar to Rel-17 SL relay, when the U2N Remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

Proposal 11: When the Intermediate Relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the U2N Remote UE and trigger the RRC connection re-establishment.

Proposal 12: Upon receiving the notification message with cause value “PC5 RLF”, the Remote UE can determine whether to release the PC5 link with the Intermediate Relay UE and triggers the RRC connection re-establishment.

Proposal 13: Upon receiving the notification message with cause value “Uu RLF” from the U2N Relay UE, the Intermediate Relay UE needs to send a notification message to the Remote UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu RLF of U2N Relay UE

- Option 2: introduce a new cause value which indicates RLF happens on the indirect link of Intermediate Relay UE

- Option 3: reuse the “PC5 RLF” cause value

System information

Proposal 14: The PC5 message form the Intermediate Relay UE to the U2N Relay UE should indicate all the required SIB(s) requested from all the connected Remote UE(s) and the Intermediate Relay UE itself.

Paging delivery

Proposal 15: The Intermediate Relay UE needs to forward the Remote UE IDs to the U2N relay UE for paging delivery to the Remote UE.

# References

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

R2-2407294, Relay discovery and (re)selection for multi-hop Relay, Huawei, HiSilicon.