3GPP TSG RAN WG2 #127				R2-2407296

Maastricht, NL, Aug 19 – 23, 2024

Agenda Item:	7.2.6	  MAC corrections (NR_pos_enh2)

Source:		ASUSTeK

Title:				Corrections on SL-PRS

Document for: Discussion and Decision

- Introduction

In this document, we propose some corrections on SL-PRS in MAC spec.

- Discussion

1st Change

According to R2#125 agreement that “SL-PRS resource request MAC CE’s priority in LCP is lower than SL-BSR MAC CE but higher than MAC CE for IAB-MT Recommended Beam Indication”, it is proposed to adopt the following text proposal.

##### 5.4.3.1.3 Allocation of resources … Logical channels shall be prioritised in accordance with the following order (highest priority listed first): … - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-PRS Resource Request; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;- MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; …

2nd Change

For the prioritization between SR and SL-PRS, SL-PRS multiplexed with SL MAC PDU in Shared SL-PRS resource pool as specified in clause 5.22.1.3.1a (Sidelink process not associated with Dedicated SL-PRS resource pool) has been considered. However, standalone SL-PRS in Dedicated SL-PRS resource pool in clause 5.22.1.3.5 (Sidelink process not associated with Dedicated SL-PRS resource pool) is missing. It is proposed to adopt the following text proposal.

### 5.4.4 Scheduling Request The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission. … As long as at least one SR is pending, the MAC entity shall for each pending SR: … 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap: … 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or in clause 5.22.1.3.5, or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a or the priority of the SL-PRS in clause 5.22.1.3.5 for the SL-PRS resource: 4> consider the SR transmission as a prioritized SR transmission. …

3rd Change

The description for Sidelink transmission information of SL-PRS is missing. It is proposed to adopt the following text proposal.

## 3.1 Definitions … Sidelink transmission information: Sidelink transmission information included in an SCI for an SL-SCH transmission or SL-PRS transmission with or without SL-SCH transmission on Shared SL-PRS resource pool as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, SL-PRS request, SL-PRS resource ID, a priority, a communication range requirement and Zone ID and COT sharing information. … #### 5.22.1.3 Sidelink HARQ operation and SL-PRS transmission ##### 5.22.1.3.1 Sidelink HARQ Entity … For each sidelink grant, the Sidelink HARQ Entity shall: 1> if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or … 4> deliver the MAC PDU, the SL-PRS, if available, the sidelink grant and the Sidelink transmission information of the TB and/or the SL-PRS to the associated Sidelink process; 4> instruct the associated Sidelink process to trigger a new transmission. 3> else: 4> flush the HARQ buffer of the associated Sidelink process.

4th Change

There may be different priorities indicated in SL-PRS Resource Request MAC CE for different pending SL-PRS transmissions. It is proposed to adopt the following text proposal.

#### 5.22.1.5 Scheduling Request In addition to clause 5.4.4, the Scheduling Request (SR) is also used for requesting SL-SCH resources for new transmission when triggered by the Sidelink BSR (clause 5.22.1.6) or the SL-CSI reporting (clause 5.22.1.7) or SL-DRX Command indication. The Scheduling Request (SR) is also used for requesting SL-PRS resources for new transmission when triggered by SL-PRS resource request (clause 6.1.3.74). If configured, the MAC entity performs the SR procedure as specified in this clause unless otherwise specified in clause 5.4.4. For a sidelink logical channel or for SL-CSI reporting or for SL-DRX Command indication or for Sidelink consistent LBT failure recovery or for SL-PRS Resource Request, at most one PUCCH resource for SR is configured per UL BWP. The SR configuration of the logical channel that triggered the Sidelink BSR (clause 5.22.1.6) is also considered as corresponding SR configuration for the triggered SR (clause 5.4.4). The value of the priority of the triggered SR corresponds to the value of priority of the logical channel that triggered the SR. Each sidelink logical channel and Sidelink consistent LBT failure recovery may be mapped to zero or one SR configuration, which is configured by RRC. If the SL-CSI reporting procedure is enabled by RRC, the SL-CSI reporting is mapped to one SR configuration for all PC5-RRC connections. The SR configuration of the SL-CSI reporting triggered according to 5.22.1.7 is considered as corresponding SR configuration for the triggered SR (clause 5.4.4). The value of the priority of the triggered SR triggered by SL-CSI reporting corresponds to the value of the priority of the Sidelink CSI Reporting MAC CE. The SR configuration of the SL-CSI reporting is considered as corresponding SR configuration for the triggered SR of SL-DRX Command indication triggered according to 5.28.3. The value of the priority of the triggered SR triggered by SL-DRX Command indication corresponds to the value of the priority of the Sidelink DRX Command MAC CE. The SR configuration of the Sidelink consistent LBT failure recovery triggered according to 5.31.2 is considered as corresponding SR configuration for the triggered SR (clause 5.4.4). The value of the priority of the triggered SR triggered by Sidelink consistent LBT failure recovery corresponds to the value of the priority of the SL LBT failure MAC CE. SL-PRS resource request may be mapped to zero or one SR configuration, which is configured by RRC. The value of the priority of the triggered SR triggered by SL-PRS resource request corresponds to the lowest value of the priority of all SL-PRS(s) indicated in the SL-PRS Resource Request MAC CE. … #### 6.1.3.74 SL-PRS Resource Request MAC CE The SL-PRS Resource Request MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has the following fields: - Destination index: The Destination Index field identifies the destination. The length of this field is 5 bits. The value is set to one index corresponding to SL destination identity associated to same destination reported in sl-PosTxResourceReqList if present. The value is indexed sequentially from 0 in the same ascending order of SL destination identity in sl-PosTxResourceReqList as specified in TS 38.331 [5]. When multiple lists are reported, the value is indexed sequentially across all the lists in the same order as presented in SidelinkUEInformaitonNR message; - SL-PRS priority: Priority of pending SL-PRS transmission. The length of this field is 3 bits; - SL-PRS Bandwidth: Requested minimum bandwidth of pending SL-PRS transmission. The length of this field is 5 bits. Encoding of this field is the same as sl-PRS-Bandwidth in IE SL-PRS-QoS-Info as specified in TS 38.331 [5] that codepoint value 0 corresponds to the value "mhz5" of the field sl-PRS-Bandwidth, codepoint value 1 corresponds to the value "mhz10" of the field sl-PRS-Bandwidth, and so on; - R: Reserved bit, set to 0. Figure 6.1.3.74-1: SL-PRS Resource Request MAC control element

- Conclusion

In this document, we have some text proposals for SL-PRS in MAC spec.

Reference

Annex