3GPP TSG-RAN2 Meeting #127	R2-2407301

# Maastricht, Netherlands, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4944 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on smtc4list-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R17 NTN, smtc4list-r17 was added to MeasObjectNR and SIB2/SIB4, which contains a list of SSB-MTC4-r17, and each SSB-MTC4-r17 includes a PCI list (optional, need M) and an offset. Combined with the legacy SSB-MTC (smtc1 in MeasObjectNR, smtc in SIB2/SIB4), the network is able to configure at most 4 SMTCs for the UE to perform measurements. Considering the optionality of the PCI list in SSB-MTC4-r17, there could be some misunderstanding, for instance: At T1, the SMTCs are configured as follows: SMTC1: PCI {1,2,3} offset 10 SMTC2: PCI {4,5,6} offset 11 SMTC3: PCI {7,8,9} offset 12 At T2, the SMTCs are reconfigured as follows: SMTC1: PCI {7} offset 10 SMTC2 PCI {4,5,6} offset 11 SMTC3: PCI absent offset 12 SMTC4: PCI absent offset 13 Issue1: In SMTC3, PCI list is absent, the previous value {7,8,9} should be maintained, however, PCI 7 is inlcuded in SMTC1 as well, it is unclear whether UE should use offset 10 or offset 12 for PCI 7. (Whether the same PCI can be applied in different SMTCs?) Issue2: At T2, SMTC4 is newly configured while the PCI list is absent, it is unclear which PCIs correspond to offset 13. (Whether the PCI list should be mandatory when the SMTC is configured for the first time?) To solve the ambiguity, we think the simplest way is to clarify in the filed description that PCI list should always be included. Impact analysis Impacted functionality: NTN Inter-operability: If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. If the UE is implemented according to the CR and the network is not, the Consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the filed description that PCI list should always be included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity on the mapping between PCI and offset. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

AdditionalPCIIndex-r17  ::=  INTEGER(1..maxNrofAdditionalPCI-r17)

-- TAG-SSB-MTC-STOP

-- ASN1STOP

SSB-MTC field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

SSB-MTC2 field descriptions

pci-List PCIs that follow this SMTC.

SSB-MTC3 field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

pci-List PCIs that follow this SMTC, used for IAB-node discovery.

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS block index 0, the second bit corresponds to SS block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS block is not to be measured while value 1 indicates that the corresponding SS block is to be measured (see TS 38.215 [9]). When the field is not configured the IAB-MT measures on all SS blocks. Regardless of the value of this field, SS blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

SSB-MTC4 field descriptions

pci-List PCIs that follow this SMTC. This field is mandatory present.

offset Offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Offset is given in number of subframes.

SSB-MTC-AdditionalPCI field descriptions

additionalPCI PCI of the additional SSB different from serving cell PCI.

periodicity Periodicity of the SS/PBCH blocks, see 5.5.2.10. Periodicity is given in number of subframes.

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

<End of Change>